# 3GPP TSG-RAN WG4 Meeting #114	R4-2502136

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		Handling of band 108 sub-bands introduction

Agenda Item:	6.21.2

Document for:	Discussion

# Introduction

According to the WID in [1], the following objectives were defined for LTE_terr_bcast_bands_sub_108 WID:

In this RAN4 WI two new bands for LTE-based 5G Broadcast will be specified as a subset of band 108, with the following objective: - Specify two new SDO bands from 470 to 608 MHz and 606 to 698 MHz, as subsets of the existing band 108 in a Release independent manner from Rel-17. - Specify related UE RF core requirements for 6/7/8 MHz PMCH [RAN4]1 - Specify related BS RF core requirements for 6/7/8 MHz PMCH [RAN4]

In this contribution, we provide analysis of the possible solution to introduce sub-bands of the SDO band 108.

# Discussion

Standalone Downlink Only (SDO) band 108 (i.e., 470 MHz – 698 MHz) was introduced in Rel-18 [2]. With a follow-up Rel-19 WID in [1], two new sub-bands of band 108 (i.e., 470 – 608 MHz and 606 – 698 MHz) are to be introduced, to help with market introduction in larger regions such as Europe, U.S. and India.

As with every new operating band introduction, the straightforward way of adding a new band is to define related system parameters and assign a new band number (e.g., band 112 and band 113 in this case). However, in case of SDO 108 sub-bands, there seems to be an opportunity to optimize this process, to exploit the already existing band number, and following as approach taken for band 46 (see TS 36.104 extract below, for reference).

Table 5.5-1A Sub-bands for Band 46

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| 46a | 5150 MHz | – | 5250 MHz | 5150 MHz | – | 5250 MHz |
| 46b | 5250 MHz | – | 5350 MHz | 5250 MHz | – | 5350 MHz |
| 46c | 5470 MHz | – | 5725 MHz | 5470 MHz | – | 5725 MHz |
| 46d | 5725 MHz | – | 5925 MHz | 5725 MHz | – | 5925 MHz |

One advantage of this approach is reduced ambiguity which may arise with increasing number of SDO bands (band 107 and 108 already defined), and their inter-relations. WID claims that band 107 is not wide enough, while band 108 cannot be fully exploited right now. Two new sub-bands are considered to be just an intermediate step towards wider (or full) spectrum allocations within 108 boundaries. Therefore, in the long term, it is expected that the market will adopt band 108 to a wider extent (while 108 sub-bands will extinct, even if used for the purpose of early deployments). The above understanding may be further refined based on feedback from interested market players. Still, based on the above description, we would like to propose the following:

Proposal 1: Consider defining 108 sub-bands as the “108a” and “108b” sub-bands, instead of introducing brand new E-UTRA band numbers.

Irrespective of the above, channel raster for new sub-bands would have to be defined.

Example implementation of sub-bands 108a and 108b was presented in the attached draft CR content.

## Release independence

Standalone Downlink Only (SDO) band 108 (i.e., 470 MHz – 698 MHz) was introduced in Rel-18 [2]. According to the WID [1], two 108 sub-bands are to be introduced in release independent manner from Rel-17, impacting TS 36.307. According to our understanding, it is unclear how 108 sub-bands could be introduced into Rel-17 release independence specification, as there would be no UE nor BS with related capabilities available anyway, i.e. band 108 was introduced in Rel-18. In our view, 108 sub-bands may be introduced in a release independent way only from Rel-18 onwards (to benefit from hardware supporting band 108). Anyhow, this topic requires clarification:

Proposal 2: Clarify release version for the release-independent introduction of 108 sub-bands (as it is not possible to introduce 108 sub-bands from Rel-17 due to missing baseline implementation of Rel-18 band 108).

# Conclusions

Based on the discussion above, the following is proposed:

Proposal 1: Consider defining 108 sub-bands as the 108a and 108b sub-bands, instead of introducing new band numbers.

Table: Sub-bands for SDO operation within Band 108

| E-UTRA operating band | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- |
|  | FDL_low – FDL_high |  |  |
| 108a | 470 MHz | – | 608 MHz |
| 108b | 606 MHz | – | 698 MHz |

Proposal 2: Clarify release version for the release-independent introduction of 108 sub-bands (as it is not possible to introduce 108 sub-bands from Rel-17 due to missing baseline implementation of Rel-18 band 108).

# References

[1] 	 RP-243268, New bands for LTE based 5G terrestrial broadcast for early deployments, LTE_terr_bcast_bands_sub_108, WID, RAN#106

[2]		RP-232590, New bands and bandwidth allocation for LTE based 5G terrestrial broadcast -part 2, LTE_terr_bcast_bands_part2, WID, RAN#101

# Annex A: Example implementation of band 108 sub-bands in TS 36.104: Operating bands (clause 5.5)

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1. Unless stated otherwise, requirements specified for the TDD duplex mode apply for downlink and uplink operations in Frame Structure Type 2 [4].

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 28, 31, 41 (in certain regions), 42, 43, 48, 54, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88, 103, 106 which are defined in Table 5.5-1.

LTE based 5G terrestrial broadcast is designed to operate in the E-UTRA operating bands 107, 108.

Table 5.5-1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| … |  |  |  |  |  |  |  |
| 108 | N/A |  |  | 470 MHz | – | 698 MHz | SDO (NOTE 11, NOTE 12) |
| NOTE 1: Band 6, 23 are not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 4: Band 46 is divided into four sub-bands as in Table 5.5-1A. NOTE 5: The range 2180 – 2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 6: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 7: Void NOTE 8: This band is restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 9: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. NOTE 10: This band is restricted to NB-IoT standalone operation. NOTE 11: This band is restricted to LTE based 5G terrestrial broadcast operation. NOTE 12: Band 108 is divided into two sub-bands as in Table 5.5-7. |  |  |  |  |  |  |  |

Table 5.5-1A Sub-bands for Band 46

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |
| 46a | 5150 MHz | – | 5250 MHz | 5150 MHz | – | 5250 MHz |
| 46b | 5250 MHz | – | 5350 MHz | 5250 MHz | – | 5350 MHz |
| 46c | 5470 MHz | – | 5725 MHz | 5470 MHz | – | 5725 MHz |
| 46d | 5725 MHz | – | 5925 MHz | 5725 MHz | – | 5925 MHz |

Table 5.5-2: Void

Table 5.5-3: Void

Table 5.5-3A: Void

Table 5.5-3B: Void

Table 5.5-3C: Void

Table 5.5-4: Void

Table 5.5-5: Void

Table 5.5-6: Void

Table 5.5-7: Sub-bands for SDO operation within Band 108

| 108 sub-band | Downlink (DL) operating band BS transmit UE receive |  |  |
| --- | --- | --- | --- |
|  | FDL_low – FDL_high |  |  |
| 108a | 470 MHz | – | 608 MHz |
| 108b | 606 MHz | – | 698 MHz |
| NOTE: Whenever band 108 is referred, the same requirements also apply to its sub-band(s), unless otherwise stated. |  |  |  |