3GPP TSG-RAN WG4 Meeting #114	R4-2502147

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	Other aspects for NR MIMO Phase 5 RRM requirements

Agenda item:	7.22.3.2

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID RP-242394, in particular about asymmetric DL sTRP/UL mTRP deployment scenarios (Objective 5) and the aspects related to CJT calibration reporting (Objective 3).

Discussion on two TAs

Regarding asymmetric DL sTRP/UL mTRP with two TAs enhancement, the following was agreed in RAN4#112bis meeting [1]:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

It was then agreed in RAN4#113 meeting to further discuss the following  [2]:

Issue 4-1: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Alt 1: Define requirement for 2TA following the legacy principle. - Alt 2: Wait for RAN1.

In response to the LS from RAN4, RAN1 has confirmed that if the UE is configured with a PL-Offset then the UE is not expected to receive a SSB from the UL TRP(s). On the other hand, if the UE is not configured with a PL offset, then the UE may expect to receive SSB from the UL-only TRP(s).

Agreement (RAN1 #119 Orlando) The answer to the Question 1 in LS R1-2409353 is: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

When the UE is not configured with a PL offset and the UL-only TRP is transmitting reference signals such as SSB(s), the UE can independently monitor and measure the DL reference signals associated with the respective TRPs. The Rel-18 two TAs enhancement for multi-DCI multi-TRP scenario allow UL transmissions from a UE toward two TRPs whose links may be characterized by a large difference in the propagation delays with respect to the CP length. The most relevant Rel-18 enablers are the following ones:

- Two TAGs can be configured for a serving cell configuration, each corresponding to a TRP.

  - To allow the UE to know which beam/TCI state is associated to which TAG, the TAG ID has been introduced as a field in the TCI state IE.

  - That allows a UE to receive a TA command TA1 for first TRP and a TA command TA2 for second TRP, for both intra-cell and inter-cell multi-TRP.

- The UE is required to monitor two DL reference timings, i.e., one DL reference per TAG.

  - As with legacy single TA requirements, it is up to the UE to decide which RS, among the RSs transmitted by each TRP, to use as DL reference timing for each TAG.

Similar to the Rel-18 two TA enhancements for m-DCI m-TRP, when the UE is not configured with a PL offset, the UE shall monitor two DL reference timings, one per TAG or TRP, and apply each TA command to the respective DL reference timing to determine the UL transmission timing toward that TAG. In that case, the system performance would be improved by using the independent reference timing from both TRPs, since the UL timing would be more robust against small timing offset variations caused by UE movement.

RAN1 LS has clarified that when UE is not configured with PL offset, the UE may expect to receive SSB from the UL-only TRP.

Rel-18 2 TA enhancement for m-DCI m-TRP, requires a UE to monitor two DL reference timings, i.e., one DL reference per TAG.

Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

Figure 1. One or two DL reference timings in asymmetric DL sTRP/UL mTRP scenario.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset the UE shall monitor two DL reference timings, one per TAG or TRP, whereas in case the UE is configured with PL offset, the UE shall monitor the DL reference timing of the DL/UL TRP.

RAN1 has also discussed UE autonomous timing adjustment, as per FFS in the agreement below.

Agreement (RAN1 #118bis Hefei) Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

However, it is still unclear how autonomous timing adjustments are performed in this situation. If a UE monitors just one DL reference timing from the DL/UL capable TRP (TRP1) and uses that for adjusting the UL transmissions toward both TRP1 and TRP2 (the UL-only TRP), the compensation/adjustment the UE does may suffice to compensate/adjust the UL timing error for TRP1, but may not suffice, and may potentially even amplify/increase the timing error for TRP2: as due to UE movements, different compensation for the two TRPs may be required. As one example, in case a UE is moving closer to a TRP but further away from the other TRP, the UE would perform an authonomous adjustment on the opposite direction than what is needed for the UL only TRP. The current proposal in RAN1 to just have UE autonomous timing adjustment for TRP1 is in our view limiting the deployment of this feature, as there will be no requirements on the error for the UL transmission toward TRP2. If the autonomous adjustments for TRP2 are completely turned off, this will force the network to send TAC much more often than that is needed.

Additionally, when the UE has to perform autonomous timing adjustments, if the UE independently monitors and measures the DL RS from each TRP, in case the UL-only TRP transmits RSs, then the likelihood of having incorrect timing adjustments is low.

While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP as compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs.

The current proposal in RAN1 to just have UE autonomous timing adjustment for the DL/UL capable TRP is in our view limiting the deployment of this feature, as there will be no requirements on the error for the UL transmission toward the UL-only TRP.

In general, it would be beneficial to define conditions to determine if UE autonomous timing adjustment applies to just TRP1 or to both TRP1 and TRP2. For example, that condition can be based on if the UL-only TRP transmits RS or on some UE measurements, although other options are not precluded. At least we can consider the case that the UL-only TRP transmit DL RSs such as SSBs, or the case where low mobility is detected at the UE.

With asymmetric DL sTRP/UL mTRP, specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

Discussion on TCI switching requirements

Regarding asymmetric DL sTRP/UL mTRP TCI switching, the following was agreed and discussed in the last RAN4#113 meeting [1]:

Issue 4-2: Whether to define UL TCI state activation requirement based on SRS? Way forward: - Option 1: Not define requirement (Samsung, HW, vivo) - Option 2: RAN4 to define UL TCI state activation requirement based on SRS. (Xiaomi, Ericsson, Nokia) - Option 3: Further discuss based on the feasibility of defining requirement and testing (QC, Apple, MTK)

The uplink TCI state switching delay requirements specified up to Rel-18 have been defined for an uplink TCI state which is associated with a downlink reference signal. The TCI states in an UL-only TRP would be associated with the SRS which is an uplink reference signal.

The existing Rel-18 UL TCI state switching requirements have been defined considering a TRP which transmits downlink reference signal. In the case of asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements. Hence, the applicability of the existing uplink TCI state switch delay requirements should be extended to include uplink TCI states which are associated to an uplink reference signal (SRS).

Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

In the case of asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

Issue 4-3: RRM core impacts of active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

An asymmetric DL sTRP/UL mTRP deployment scenario is a single-DCI scenario where the symmetric DL/UL TRP will transmit and receive the PDCCH and the PUCCH respectively. In Rel-18 MIMO evo, RAN4 has defined downlink as well as uplink TCI state switching delay for unified TCI state switch for single-DCI mTRP.

In case of downlink TCI state switching delay requirements, we see no difference in the downlink TCI state switching behavior and hence see no need to define new set of downlink TCI state switch delay requirements.

Similar to the downlink TCI state switching delay requirements, there is no difference in the uplink TCI state switching behavior either when the UE is indicated to switch to a target uplink TCI state associated to the uplink only TRP. Hence, the legacy RAN4 unified TCI state switching delay requirements for UL TCI state can be reused.

Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Although the delay for UL TCI state switch does not need to be changed from the legacy, the requirements still need some updates for the UL-only TRP TCI state switch case.

The legacy TCI state switch delay requirements have been defined based on a TCI state being known or unknown. For a TCI state to be considered as known there are set of conditions defined which are primarily centered around a downlink reference signal being transmitted by the TRP. The known/unknown conditions are used to determine if the UE has reported a valid recent measurement from the target TCI state before making the switch. In case the target TCI state is unknown the UE is expected to perform measurements on the target TCI state, in order to determine transmission parameters such as spatial filter of the target TCI state.

However, in the case of an asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any reference signal when PL offset is configured for the UE, this implies that the conditions defined up to Rel-18 for an uplink TCI state to be known cannot be used in the context of an asymmetric DL sTRP/UL mTRP.

Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

For an uplink-only TRP, since there is no downlink reference signal being transmitted when PL offset is configured for the UE, the legacy known/unknown conditions cannot be reused.

RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

Reporting for CJT calibration

General simulation setup discussion

The simulation setup for CJT calibration was agreed on [2], however some points need to be clarified for the simulation alignment. Firstly, the model for the signal generator for the TRP1 and TRP2 need to be clarified. Regarding that, 3 scenarios can be considered:

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the subcarriers carrying TRS symbols.

- Scenario 3: Data on all the OFDM symbols; except for the TRS symbols.

There can be performance differences between the 3 scenarios. For example, on scenario 3 the ICI can affect frequency offset estimation. On Scenario 2 and 3, the timing offset estimation can be affected by ISI when the timing offset is close to CP length. On the other hand, Scenario 1 might not be realistic enough, since the symbols not carrying TRS are typically also used for PDSCH/PDCCH.

RAN4 to consider CJT simulation setup with TRP1/TRP2 carrying random data on OFDM symbols not carrying TRS.

Another point is how to consider the TRS configuration for TRP1 and TRP 2. Below we show one example of TRS configuration using in legacy test setup in 3.8.133. We need to clarify what is the configuration for TRP1 and TRP2. Is reusing the configuration of this table, the TRS Res for the two TRPs would be overlapping. But there is also an option of using different k0 for TRP1 and TRP2.

For CJT calibration simulations adopt the following TRS configuration:

| Parameter | Unit | Configuration 1 | Configuration 2 |
| --- | --- | --- | --- |
| SCS | kHz | 15 | 30 |
| TRP1 configures as NZP |  | TRS resources 1,2,3,4 |  |
| TRP1 configures as ZP |  | TRS resources 5,6,7,8 |  |
| TRP2 configures as NZP |  | TRS resources 5,6,7,8 |  |
| TRP2 configures as ZP |  | TRS resources 1,2,3,4 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for TRS resources 1,2,3,4 k0=1 for TRS resources 5,6,7,8 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = FFS for TRS resources 1,3,5,7 l0 = FFS for TRS resources 2,4,6,8 |  |
| rowNote 1 |  | 1 |  |
| CSI-RS periodicity | slots | 20 | 40 |
| CSI-RS offset | slots | 10 for TRS resources 1,2,5,6 11 for TRS resources 3,4,7,8 | 20 for TRS resources 1,2,5,6 21 for TRS resources 3,4,7,8 |
| Note 1: row value from TS38.211 Table 7.4.1.5.3-1 defines density, CDM Type and number of ports, etc. |  |  |  |

From the simulation setup discussed in the previous RAN4 meeting, it is not completely clear whether the offset error to be measured is constant or a random variable from 0 to the maximum offset. For the CFO, TO, and PO tests to be representative across different values of offset measurements and reported calibration values, these should be randomly distributed across the range of simulated values:

Clarify CFO, TO, and PO in the simulation assumption, and define it as:

  - CFO error per TRP (CFO1 and CFO2) should be randomly chosen from the range [-0.5A ppm; 0.5A ppm] such that |CFO1 - CFO2| is uniformly distributed in [0; A ppm]

  - TO error per TRP (TO1 and TO2) should be randomly chosen from the range [-0,5 B CP; 0,5 B CP] such that |TO1 - TO2| is uniformly distributed in [0; B CP]

  - PO error per TRP (PO1 and PO2) should be randomly chosen from the range [-π;  π] such that |PO1 - PO2| is uniformly distributed in [-π;  π]

Phase requirements

For phase offset reporting, time-averaging phase offset measurements over multiple transmission occasions may improve robustness against measurement noise and can be used to test performance requirements in RAN4. In a network implementation where a one-port CSI-RS resource for CSI is not MRT-precoded, the gNB may need to align its phase offset measurement on SRS with the phase offset reported by a UE. In case of time-averaging, the CSI-RS transmission occasions need to be paired with corresponding SRS transmissions and the number of measurements included in the time-averaging performed by a UE needs to correspond to the number of measurements averaged by the gNB.

Different mechanisms of time-averaging at the UE and NW side can take place. For instance, one question would be: may we have several PO measurements in time being UE reported and the time-averaging takes place only in the NW side? In that case, the number of estimations to be sent must be determined and configured in the UE in order to send these PO estimates toward the NW. However, one disadvantage of this option is the fact that quantization could introduce some error and it requires more bit overhead than a single time CJT calibration report. Alternatively, these measurements could be instead buffered in the UE and the NW for respective DL and UL estimates in order to exchange a single CJT PO estimate of an averaged value in the UE. As depicted in Figure 2, the buffering could happen after the CJT calibration triggering and thus the UE and NW sends SRS and CSI-RS respectively by means of periodic/semi-persistent schemes to enable a number of measurements for UL and DL estimations. The number of measurements to take at each side and other averaging related parameters might be needed to be signaled to the UE and sent from the NW. Otherwise the buffering could start before and the CSI-RS and SRS might have been preconfigured before the triggering. Also in this case the NW must indicate the number of measurements to include in the time-averaging and any other related parameters. Notice that in any of the previously mentioned possible implementations, a pairing/linkage in time domain is required between pairs of CSI-RS/SRS transmission occasions in order to have a consistent final PO estimate from the averaged UL and DL measurements. This also means that each pair of CSI-RS/SRS transmission occasions must be located the closest possible to each other.

Figure 2. Illustration of CSI-RS and SRS transmissions for (a) when the measurements are collected and averaged after the triggering and (b) when the measurements are collected and averaged before the triggering

Some initial simulations were carried out with 200m ISD to understand the consequences of averaging for the PO estimates calculated at the gNB and UE sides with different number of samples. As illustrated in Figure 3  it is possible to obtain already an accurate PO estimation with a single measurement, i.e., no time averaging neither at the UE or the gNB side, for the scenario analysed. A similar result can be obtained by averaging the same number of measurements for UL and DL, 10 buffered measurements each, while ensuring that the UL/DL measurements are close to each other when transmitted. However, we may also observe that by asymmetrically setting the time-averaging at gNB and no time averaging at the UE, it might give as a result some error in the PO estimate and as a result performance losses. This lead us to think that pairing the time occasions of the time-averaging measurements happening at the UE with those measurements happening at the gNB is significantly important to obtain a good PO estimate.

Taking into account the simulation results obtained we consider that a deeper study might be required on how the time averaging might benefit the calculation of the PO estimates. For that, other scenarios in which the SRS or CSI-RS may be further stressed, need to be considered. For example, increasing the ISD might be a way, however other potential scenario conditions may be also studied. Regarding the impact on the specification, the need for alignment between gNB and UE in time-averaging PO estimates may require additional signalling to ensure this as already mentioned before. This is currently being proposed in the RAN 1 discussion.

Figure 3. Normalized throughput results for different CJT setups with/without UL/DL PO Calibration.

Therefore, in the case that averaging is carried out at gNB for UL measurements, a symmetrical averaged estimation is needed at the UE side with same number of buffered measurements, or viceversa. For the scope of RAN 4 RRM studies, if we perform a test with not MRT-precoded CSI-RS is maybe simpler because we may not need to care about the things happening at gNB side neither the accuracy of the UL PO estimate. Our test may only focus on the accuracy of the UE reported phase offset. On the other hand, making the test with a MRT-precoded single port CSI-RS may bring some additional questions on how to obtain such a precoder without depending on a SRS measurement during the test. However, in a not MRT-precoded case, we may need to assume that UE needs to set time averaging with a given number of measurements to align with the gNodeB. That would be a justification to define requirements for PO CJT estimations  also including the effect of time-averaging with different number of buffered measurements. In the rest of this section, the LLS simulations carried out only consider the UE time-averaging of the PO estimates and the accuracy is analysed only for the UE reported estimates, i.e. assuming not MRT-precoded PO estimation for CJT calibration.

Assuming not MRT-precoded single port CSI-RS phase offset estimation may enable the accuracy test by only estimating the accuracy of the UE reported phase offset quantity.

For not MRT-precoded single port CSI-RS phase offset estimation requires symmetrical averaging at both UE and network sides, otherwise there is a loss in performance due to bad phase offset compensation.

Assume not MRT-precoded single port CSI-RS for the accuracy test for the phase offset estimation CJT reporting.

RAN4 to define the number of samples that shall be used by the UE for PO CJT for phase offset report.

Conclusion

In this paper, the following Observations and Proposals were made:

Observation 1: RAN1 LS has clarified that when UE is not configured with PL offset, the UE may expect to receive SSB from the UL-only TRP.

Observation 2: Rel-18 2 TA enhancement for m-DCI m-TRP, requires a UE to monitor two DL reference timings, i.e., one DL reference per TAG.

Observation 3: Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

Proposal 1: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset the UE shall monitor two DL reference timings, one per TAG or TRP, whereas in case the UE is configured with PL offset, the UE shall monitor the DL reference timing of the DL/UL TRP.

Observation 4: While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP as compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs.

Observation 5: The current proposal in RAN1 to just have UE autonomous timing adjustment for the DL/UL capable TRP is in our view limiting the deployment of this feature, as there will be no requirements on the error for the UL transmission toward the UL-only TRP.

Proposal 2: With asymmetric DL sTRP/UL mTRP, specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

Observation 6: Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

Observation 7: In the case of asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Proposal 3: Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

Observation 8: Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

Proposal 4: For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Observation 9: Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Observation 10: Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

Observation 11: For an uplink-only TRP, since there is no downlink reference signal being transmitted when PL offset is configured for the UE, the legacy known/unknown conditions cannot be reused.

Proposal 5: RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

Proposal 6: RAN4 to consider CJT simulation setup with TRP1/TRP2 carrying random data on OFDM symbols not carrying TRS.

Proposal 7: For CJT calibration simulations adopt the following TRS configuration:

| Parameter | Unit | Configuration 1 | Configuration 2 |
| --- | --- | --- | --- |
| SCS | kHz | 15 | 30 |
| TRP1 configures as NZP |  | TRS resources 1,2,3,4 |  |
| TRP1 configures as ZP |  | TRS resources 5,6,7,8 |  |
| TRP2 configures as NZP |  | TRS resources 5,6,7,8 |  |
| TRP2 configures as ZP |  | TRS resources 1,2,3,4 |  |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for TRS resources 1,2,3,4 k0=1 for TRS resources 5,6,7,8 |  |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = FFS for TRS resources 1,3,5,7 l0 = FFS for TRS resources 2,4,6,8 |  |
| rowNote 1 |  | 1 |  |
| CSI-RS periodicity | slots | 20 | 40 |
| CSI-RS offset | slots | 10 for TRS resources 1,2,5,6 11 for TRS resources 3,4,7,8 | 20 for TRS resources 1,2,5,6 21 for TRS resources 3,4,7,8 |
| Note 1: row value from TS38.211 Table 7.4.1.5.3-1 defines density, CDM Type and number of ports, etc. |  |  |  |

Proposal 8: Clarify CFO, TO, and PO in the simulation assumption, and define it as:

  - CFO error per TRP (CFO1 and CFO2) should be randomly chosen from the range [-0.5A ppm; 0.5A ppm] such that |CFO1 - CFO2| is uniformly distributed in [0; A ppm]

  - TO error per TRP (TO1 and TO2) should be randomly chosen from the range [-0,5 B CP; 0,5 B CP] such that |TO1 - TO2| is uniformly distributed in [0; B CP]

  - PO error per TRP (PO1 and PO2) should be randomly chosen from the range [-π;  π] such that |PO1 - PO2| is uniformly distributed in [-π;  π]

Observation 12: Assuming not MRT-precoded single port CSI-RS phase offset estimation may enable the accuracy test by only estimating the accuracy of the UE reported phase offset quantity.

Observation 13: For not MRT-precoded single port CSI-RS phase offset estimation requires symmetrical averaging at both UE and network sides, otherwise there is a loss in performance due to bad phase offset compensation.

Proposal 9: Assume not MRT-precoded single port CSI-RS for the accuracy test for the phase offset estimation CJT reporting.

Proposal 10: RAN4 to define the number of samples that shall be used by the UE for PO CJT for phase offset report.

- References

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#112bis, Hefei, Anhui, China, 14th ‒ 18th, October, 2024.

- R4-2420108, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4#113, Orlando, US, 18 ‒ 22 November, 2024

- Annex A: CJT simulation results

CFO requirements

Carrier frequency offset estimation error values in Hz are shown in Table 1.

- Scenario 3: Data on all the OFDM symbols; except for the TRS symbols.

A comparison for the carrier frequency offset computation without averaging and with averaging of 3 and 5 samples measurements are shown in parts per billion (ppb) with respect to the carrier frequency of 3.5 GHz are shown in Table 1.

Table 1: Carrier frequency offset estimation error (in ppb) under assumptions of Scenario 3.

| Quant Levels | FO (ppb) | SCS | SNR | No averaging |  | 3 samples |  | 5 samples |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | P50 | P90 | P50 | P90 | P50 | P90 |
| 32 | 100 | 15 | -3 | 24.76 | 64.50 | 16.10 | 43.96 | 13.64 | 38.83 |
| 32 | 100 | 15 | 0 | 22.24 | 55.93 | 12.86 | 35.03 | 10.26 | 28.39 |
| 32 | 100 | 15 | 3 | 16.00 | 44.56 | 10.75 | 26.68 | 7.88 | 20.81 |
| 32 | 200 | 15 | -3 | 32.04 | 83.66 | 20.58 | 52.84 | 15.87 | 40.90 |
| 32 | 200 | 15 | 0 | 24.97 | 68.70 | 15.54 | 38.79 | 12.08 | 31.02 |
| 32 | 200 | 15 | 3 | 20.69 | 51.16 | 11.65 | 27.83 | 9.23 | 23.36 |
| 256 | 100 | 15 | -3 | 25.12 | 64.33 | 16.80 | 44.26 | 13.63 | 38.12 |
| 256 | 100 | 15 | 0 | 22.74 | 55.91 | 12.91 | 34.68 | 10.47 | 28.24 |
| 256 | 100 | 15 | 3 | 16.29 | 44.53 | 11.00 | 26.58 | 7.75 | 20.26 |
| 256 | 200 | 15 | -3 | 33.29 | 85.22 | 19.98 | 52.46 | 15.59 | 40.37 |
| 256 | 200 | 15 | 0 | 25.21 | 69.22 | 15.21 | 38.16 | 11.95 | 29.63 |
| 256 | 200 | 15 | 3 | 20.77 | 50.21 | 11.01 | 26.98 | 8.59 | 22.24 |

TO requirements

Time offset estimation error values as a percentage of the cyclic prefix (% of CP) are shown in Table 2.

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the subcarriers carrying TRS symbols.

- Scenario 3: Data on all the OFDM symbols; except for the TRS symbols.

Scenario 1: TRP1 and TRP2 transmitting only TRS

Table 2: Time offset estimation error (% of CP).

| Quantization Levels | TO (% CP) | SCS | SNR | Scenario 1 |  | Scenario 2 |  | Scenario 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | P50 | P90 | P50 | P90 | P50 | P90 |
| 32 | 50 | 15 | -3 | 0.76 % | 1.76 % | 1.35 % | 3.53 % | 1.33 % | 3.61 % |
| 32 | 50 | 15 | 0 | 0.74 % | 1.56 % | 1.27 % | 3.62 % | 1.28 % | 3.32 % |
| 32 | 50 | 15 | 3 | 0.74 % | 1.48 % | 1.21 % | 3.42 % | 1.24 % | 3.49 % |
| 32 | 50 | 30 | -3 | 0.82 % | 1.77 % | 1.30 % | 3.54 % | 1.35 % | 3.51 % |
| 32 | 50 | 30 | 0 | 0.75 % | 1.54 % | 1.29 % | 3.59 % | 1.33 % | 3.41 % |
| 32 | 50 | 30 | 3 | 0.74 % | 1.48 % | 1.19 % | 3.44 % | 1.17 % | 3.47 % |
| 32 | 100 | 15 | -3 | 1.62 % | 2.96 % | 1.96 % | 4.65 % | 2.09 % | 4.61 % |
| 32 | 100 | 15 | 0 | 1.65 % | 2.93 % | 2.02 % | 4.35 % | 1.89 % | 4.47 % |
| 32 | 100 | 15 | 3 | 1.61 % | 2.95 % | 1.86 % | 4.13 % | 1.73 % | 4.34 % |
| 32 | 100 | 30 | -3 | 1.59 % | 3.00 % | 1.92 % | 4.50 % | 1.83 % | 4.51 % |
| 32 | 100 | 30 | 0 | 1.51 % | 2.89 % | 1.99 % | 4.52 % | 1.78 % | 4.43 % |
| 32 | 100 | 30 | 3 | 1.49 % | 2.84 % | 1.88 % | 4.15 % | 1.82 % | 4.21 % |
| 256 | 50 | 15 | -3 | 0.18 % | 0.43 % | 0.30 % | 2.98 % | 0.31 % | 3.03 % |
| 256 | 50 | 15 | 0 | 0.17 % | 0.38 % | 0.25 % | 2.95 % | 0.27 % | 2.99 % |
| 256 | 50 | 15 | 3 | 0.17 % | 0.38 % | 0.23 % | 2.89 % | 0.25 % | 2.95 % |
| 256 | 50 | 30 | -3 | 0.19 % | 0.41 % | 0.27 % | 3.00 % | 0.31 % | 3.03 % |
| 256 | 50 | 30 | 0 | 0.17 % | 0.39 % | 0.26 % | 2.98 % | 0.25 % | 2.95 % |
| 256 | 50 | 30 | 3 | 0.18 % | 0.38 % | 0.23 % | 2.90 % | 0.25 % | 2.94 % |
| 256 | 100 | 15 | -3 | 0.23 % | 0.59 % | 0.38 % | 3.15 % | 0.39 % | 3.11 % |
| 256 | 100 | 15 | 0 | 0.22 % | 0.53 % | 0.34 % | 3.07 % | 0.33 % | 3.11 % |
| 256 | 100 | 15 | 3 | 0.21 % | 0.50 % | 0.30 % | 3.05 % | 0.31 % | 3.04 % |
| 256 | 100 | 30 | -3 | 0.24 % | 0.56 % | 0.41 % | 3.13 % | 0.38 % | 3.14 % |
| 256 | 100 | 30 | 0 | 0.22 % | 0.51 % | 0.35 % | 3.10 % | 0.34 % | 3.08 % |
| 256 | 100 | 30 | 3 | 0.23 % | 0.51 % | 0.32 % | 3.04 % | 0.35 % | 3.10 % |

Phase requirements

Phase offset estimation error values as degrees from 0 to 180 are shown in Table 3.

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the subcarriers carrying TRS symbols.

- Scenario 3: Data on all the OFDM symbols; except for the TRS symbols.

Table 3: Phase offset estimation error 90th percentile in degrees from 0 to 180.

| Estimation and quantization error in degrees from 0 to 180 |  |  | Scenario 1 |  |  | Scenario 2 |  |  |  | Scenario 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quant. Levels | SCS | SNR | No averaging | 3 samples | 5 samples | No averaging | 3 samples | 5 samples | No averaging |  | 3 samples | 5 samples |  |
| 16 | 15 | -3 | 10,25 | 10,37 | 10,25 | 22,57 | 18,24 | 14,48 | 24,95 |  | 17,9 | 13,88 |  |
| 32 | 15 | -3 | 5,37 | 5,62 | 5,2 | 20,69 | 15,76 | 11,26 | 21,87 |  | 15,6 | 10,96 |  |
| 64 | 15 | -3 | 3,13 | 3,61 | 3,02 | 20,71 | 15,32 | 10,2 | 21,82 |  | 15,2 | 9,47 |  |
| 256 | 15 | -3 | 1,76 | 2,38 | 1,51 | 20,66 | 15,22 | 9,85 | 21,5 |  | 15,24 | 9,39 |  |
| 1024 | 15 | -3 | 1,64 | 2,36 | 1,37 | 20,75 | 15,18 | 9,77 | 21,69 |  | 15,14 | 9,44 |  |
| 16 | 15 | 0 | 10,35 | 10,31 | 10,31 | 15,43 | 13,59 | 11,75 | 16,09 |  | 14,27 | 11,79 |  |
| 32 | 15 | 0 | 5,09 | 5,42 | 5,09 | 12,78 | 9,91 | 8,02 | 14,07 |  | 11,32 | 7,97 |  |
| 64 | 15 | 0 | 2,75 | 3,02 | 2,74 | 11,66 | 9,1 | 6,4 | 13,01 |  | 9,46 | 6,6 |  |
| 256 | 15 | 0 | 1,22 | 1,72 | 1,1 | 11,62 | 8,49 | 5,81 | 12,57 |  | 9,53 | 5,99 |  |
| 1024 | 15 | 0 | 0,99 | 1,66 | 0,94 | 11,82 | 8,59 | 5,78 | 12,41 |  | 9,4 | 5,84 |  |
| 16 | 15 | 3 | 10,23 | 10,15 | 9,92 | 12,28 | 12,12 | 10,82 | 13,04 |  | 11,95 | 10,93 |  |
| 32 | 15 | 3 | 5,05 | 5,17 | 5,17 | 8,7 | 7,9 | 6,5 | 9,26 |  | 8,06 | 6,66 |  |
| 64 | 15 | 3 | 2,62 | 2,75 | 2,55 | 7,14 | 6,32 | 4,66 | 7,65 |  | 6,81 | 4,57 |  |
| 256 | 15 | 3 | 0,92 | 1,23 | 0,95 | 6,72 | 6 | 3,79 | 7,45 |  | 6,18 | 3,98 |  |
| 1024 | 15 | 3 | 0,64 | 1,07 | 0,65 | 6,64 | 5,85 | 3,75 | 7,41 |  | 6,24 | 3,83 |  |