3GPP TSG-RAN WG4 Meeting #113	R4-2502150

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	7.28.2

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on RRM requirements for XR enhancements. The main contributions of this paper are:

- We proposed the formulas on how to capture the measurement delay extension due to measurement skipping

- We demonstrated that UAI related to channel occasions cannot be determined by the UE in a reliable manner for non-serving cell measurements, and made analysis of what details are needed in case of UAI is agreed.

- We demonstrated that DCI-based approach provides better capacity gains, which increase with small time offset, and proposed to follow RAN1 decision on Alt 1 with a small time offset for increased performance.

Scenarios for measurement skipping

L3 Measurements without gaps with scheduling restrictions

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]::

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported - Option 1a: Support skipping of measurements without gaps - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported - Option 2a: Measurement skipping does not apply for measurements without gaps - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

As for measurements without gaps, they may cause scheduling restrictions in some situations. One typical situation for scheduling restrictions of measurements without gaps is when performing measurements in FR2 because of beam sweeping. For FR1 the cases with scheduling restrictions are mostly limited to cases with mixed numerology.

For measurements without gaps, another aspect to consider is whether interruptions are needed or not. Interruptions will cause the UE to lose some data/control at some time before and after SMTC occasions to be measured. This will typically be more problematic for XR traffic than for normal eMBB traffic. When XR traffic is affected by interruption, that lost data or DCI scheduling data will cause extra delay which can be large in relation to the total delay budget available for the UE. Another reason why interruptions need to be avoided for XR traffic is that they may cause the UE to miss commands for measurement skipping.

Some measurements without gaps scenarios cause interruptions. Interruptions will cause scheduling problems for latency sensitive traffic.

RAN4 has defined RRM requirements for interruptions for measurements without gaps in Rel-18 NR_MG_enh. During this work item, it was defined that interruptions are not restricted to a location. That means that although interruptions are expected to happen close to a SMTC to measure, there is no requirement of how close they should be. During this discussion, some companies pointed out that it was not clear whether Rel-16 UEs supporting NeedForGaps-r16 are causing interruptions.

If measurement skipping requirements are to be defined for measurements without gaps with interruptions, it is important that some method is defined in order to limit the interruptions over XR traffic. As an example a window where interruption is avoided could be defined. This would avoid the problems described above.

Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

So far our view was that measurements without gaps do not need to be skipped, however, in order to promote progress we are willing to consider this case as well. In this case we would like that the considerations above are taken into account, otherwise the benefits of measurements skipping would not be achieved.

Measurement skipping does apply for measurements without gaps for the following scenarios:

  - Intra-frequency without gap for SSB within active BWP in FR2-1

  - For UE supporting NeedForInterruptions-r18

For the requirements of measurements without gaps, reuse agreement on time offset requirement for measurement gap skipping

If interruptions are needed for measurements without gaps, skipping indication cancel the interruptions as well as scheduling restrictions.

DCI skipping command applies for an entire SMTC.

L1 measurements and L1-L2 triggered mobility measurements

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures. - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures. - Option 2a: Measurement skipping does not apply for L1 measurements and procedures - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact. ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM - Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping. - Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

L1 measurements, such as L1-RSRP and L1-SINR, are for serving cell only and refer to L1 procedures, i.e. RLM, BFD and CBD procedures. Skipping of these measurements will increase the measurement delay and hence may yield a degradation of these procedures. The operation of these procedures should be agnostic of the XR operation. Thus, these L1 measurements should not be subject to measurement skipping during XR operation.

Measurement skipping does not apply for L1 measurements and L1 procedures.

As for L1 measurements above, the operation of the LTM procedure should be agnostic of the XR operation. Thus, these L1/L2 triggered mobility measurements should not be subject to measurement skipping during XR operation.

Measurement skipping does not apply for L1 measurements and LTM procedures.

Impact of measurement skipping

How to capture the impact of measurement skipping

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

# Topic #3: XR skipping requirements impact ## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay <Way forward>: Proposals related to measurement delay requirements: - Option 1: Proposals without distinction of XR and legacy requirements - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended. - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Option 2: Proposals with distinction of conditions where measurement impact is not needed - Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. - Option 2b: - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

In the previous RAN4 meetings, some companies wanted to discuss UAI in combination to the measurement delay caused by skipping. In the view of some companies, we need to define some cases where measurement delay extension is not needed, because of the UAI information. On the other hand, we need to have in mind that even if UAI is agreed, it will be probably an optional UE capability, and therefore we cannot avoid specifying the requirements for the case where UAI is not defined. Additionally, it is also a network decision whether UAI is configured.

Even if RAN4 agreed on UAI, it will be probably optional UE capability, so RAN4 needs to define requirements without UAI first.

Discuss first the requirements without UAI consideration.

Principles for measurement delay calculation

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Options based on scaling factor - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor. - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor). - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

In our view, both options are appropriate during XR operation, but it will be clearer if we start discussion the formulas for measurement delay calculation.

The Rel-18 requirements of measurement period with measurement gaps in FR1 are referred as one example for the analysis of the measurement delay extension. The requirements without DRX on Table 9.3.5-1 are represented by the formula:

- Max(200 ms, Ceil(8 * Kgap)  Max(MGRP , SMTC period))  CSSFinter

Where

- 200 ms is the lower bound for the measurement delay calculation

- 8 is the maximum number of samples for inter-frequency measurements

- MGRP is the gap periodicity

- SMTC period is the period for the SMTC window

- CSSF is the carrier specific scaling factor.

- Kgap is used to take into account the overlap of concurrent gaps

Considering a simple example of a single carrier measurement (CSSF=1), without concurrent gaps (Kgap=1), and MGRP=SMTC period, we can simplify the formula above as

- Max(200 ms, 8  ´ MGRP)

So if we skip N samples on a measurement cycle, we could extend the formula as:

- Max(200 ms, (8 + N)   MGRP )

That would still keep the 200 ms as lower bound, and in some cases the measurement delay is not extended. If we follow that logic, we can extend the measurement delay formula as done in NR-U requirements:

- Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

One problem of this simple extension arises if we consider multiple frequency layers with CSSFinter > 1. In that case, every skipped sample would result in an extension of CSSFinter  Max(MGRP,SMTC) according to the round robin approach. Therefore, we propose one alternative solution that allows for extending the measurement delay just by a single SMTC/MRGP

- Max(200 ms  CSSFinter, Ceil( 8 * Kgap)  Max(MGRP , SMTC period)  CSSFinter + Nskip  Max(MGRP , SMTC period))

Instead of the round robin approach for applying the CSSF between different frequency layers, this stops the round robin at the skipped MG occasion and continues it at the next available MG occasion. Thus, the overall delay can be remarkably reduced.

RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Max(200 ms  CSSFinter, Ceil( 8  * Kgap)  Max(MGRP , SMTC period)  CSSFinter + Nskip  Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Additionally, we also have to consider the case where multiple frequency layers are measured. When multiple layers are configured, the current RAN4 assumption is that the UE is using a single searcher for measurements with gaps, and a single frequency layer is measured at every given measurement occasion. In some network deployments, the importance of certain frequencies is not equal in relation to others. Therefore, it is desirable that the measurement skipping is not affecting equally the measurement delay for all the frequency layers. This can be achieved by the UE prioritizing higher priority frequency layers to be measured first in case a measurement gap is skipped, while low priority gaps may be measured on remaining gap occasions and hence experience longer measurement delays.

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

One example is if one frequency layer is used for mobility, while another is more often used for additional carriers in carrier aggregation. Figure 1 shows one example where one carrier is assigned a high priority. In this example the skipping of few SMTCs is compensated by measuring more often for the high priority carrier, while the low priority carrier has its delay extended.

Figure 1 – Example assigning priority to measure one frequency layer, where measurement delay is not extended to that layer, while it is extended to the low priority layer

When that approach is used, we can consider the high priority carriers to have unchanged measurement delay as long as the number of skipped measurement occasions in the measurement cycle is Nskip < 8  (CSSF - Nhigh) for inter-frequency measurements and Nskip < 5  (CSSF - Nhigh) for intra-frequency measurements.

Examples if Nskip and the resulting measurement skipping ratio are shown on Table 1 for different values of CSSF and high priority layers. In some examples with small number of Nhigh the maximum skipping ratio that can be achieved without impact to high priority layers can get between 50% to 80%.

It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

  - Nskip ≤8  (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5  (CSSF - Nhigh) for intra-frequency measurements

  - where Nhigh is the number of high priority layers.

Table 1 Calculation of maximum skipping ratio without high priority delay extension

| Number of inter-freq samples (per frequency layer) | CSSF | Nhigh | Maximum Nskip | Maximum skipping ratio % |
| --- | --- | --- | --- | --- |
| 8 | 2 | 1 | 8 | 50 |
| 8 | 3 | 1 | 16 | 66,7 |
| 8 | 3 | 2 | 8 | 33,3 |
| 8 | 4 | 1 | 24 | 75 |
| 8 | 4 | 2 | 16 | 50 |
| 8 | 4 | 3 | 8 | 25 |
| 8 | 5 | 1 | 32 | 80 |
| 8 | 5 | 2 | 24 | 60 |
| 8 | 5 | 3 | 16 | 40 |
| 8 | 5 | 4 | 8 | 20 |

Conditions for measurement skipping

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: max number of occasions cancelled during a measurement period shall be defined. - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. - Option 3: discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping), - maximum MGL (e.g., MGL≤6 ms), - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc. - Option 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

In this issue few companies proposed that a limitation condition where measurement skipping may be used. These included maximum number of occasions to be skipped or measurement gap configurations.

Regarding Option 1 of issue 3-3, we understand that some companies have concerns on potential impacts if measurement skipping exceeds a certain limit. However, it is not very clear how to define a limit to measurement gap skipping that is general enough to account for all possible scenarios.

- Example 1: 2 different networks configure the UE to monitor different number of frequency layers:

  - Network 1: UE only monitors 1 frequency layers, and CSSF = 1

  - Network 2: the UE monitors 4 frequency layers, and CSSF = 4

  - Measurement period calculation: Max(200ms, Ceil(8 * Kgap) × Max(MGRP , SMTC period)) × CSSFinter, with Kgap = 1

| RRM parameter | Network 1 | Network 2 |
| --- | --- | --- |
| T_SSB | 40ms | 40ms |
| MGRP | 40ms | 40ms |
| CSSF inter | 1 | 4 |
| Measurement period | 320ms | 1280ms |

  - In this example the impact of measurement skipping is not the same for Network 1 and Network 2.

    - If a UE in network 1 skips 75% of the gaps, it should be capable of achieving the same measurement delay as the UE in network 2.

Therefore, we think that it is natural that in the network deployment there is a limitation on the maximum number of skipping. However, this is an optimization that can be determined case by case, for example considering the typical mobility of XR UEs in a given location, the ISD, or layers to monitor. If we define a limit in the specification, we will simply limit available options on the deployment and the benefit of the feature will be impacted.

When considering Option 3, it is not clear what is the relationship between the measurement gap length and the impact to measurement skipping. We understand that the measurement skipping will have a direct impact on measurement delay requirements, however those requirements do not depend on the measurement gap length.

Limiting the maximum number of occasions that can be skipped in the specification will have an impact on the network deployment flexibility.

No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

Mobility impacts due to measurement skipping

For the case when measurement delay in RRC_CONNECTED will increase, due to measurement skipping, as discussed in sections 5.1 and 5.2, L3 measurement delay may impact mobility procedures such as handover. For instance, if the UE is moving towards cell edge and a neighbor cell is becoming stronger than the serving cell, then a larger measurement delay may impact handover success. Additionally, the skipped measurement opportunities might cause the freeze in L3 filter update and the effect of outdate measurements in the L3 filter will persist for a longer time than the case when measurements are not skipped. If the UE is configured with an A3 event, which is triggered when the neighbor cell becomes stronger than the serving cell by a certain offset, then the reporting may be delayed due to the increased measurement delay, i.e. the handover may be delayed, or even may fail, in case serving cell quality drops rapidly while measurement skipping is applied. This impact due to measurement skipping is assumed to increase if several measurements for a frequency layer in a measurement period are skipped and/or if subsequent measurements for a frequency layer in the measurement period are skipped. This mobility impact due to measurement skipping for L3 measurements should be considered by RAN4. One way is to limit the skipping ratio, but this will impact the XR service quality. Another option is to define adjusted offsets for mobility procedures such as handover to reduce the handover delay. For instance, if A3 offset is somewhat reduced in case of observed measurement skipping, then the difference in signal strength between neighbor and serving cell triggering A3 event can be detected at an earlier point in time and hence additional handover delay can be compensated.

Measurement skipping will result in an additional L3 measurement delay, thus impacting both handover preparation and execution, thus the handover will be executed with an additional delay .

RAN4 to consider mobility impacts due to measurement skipping and to start with the handover procedure.

RAN4 to discuss how to mitigate the impact of measurement delay extension in mobility events.

Time offset for measurement skipping

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

Regarding the FFS point of the optional capability of 3 ms, there were a company that wanted to check whether conditions for the 3 ms to apply are needed. In our view we already agreed that the UE is required to measure SRVCC is not supported by RAN4. That was done in order to easy the implementation, in case of 2G-3G measurements, and should suffice in guaranteeing the 3 ms condition.

No condition is needed for the optional capability of 3 ms.

Need/feasibility of UAI

UAI related to measurement occasions

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 5-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Proposals related to measurement occasions - Option 1: Define UAI related to measurement occasions - Option 1a: number of occasions which can be skipped - Option 1b: number of measurement occasions that are needed for measurement - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation. - Option 1d: patterns of gaps positions of MG that can be skipped - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured. - Option 1f: Maximum number of consecutive MGs that can be skipped - Option 2: - Option 2a: Do not define UAI related to measurement occasions. - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI. - Option 3: The information about gaps not used for measurement are reported as UE capability. - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. - Proposals related to channel conditions - Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

RAN1 sent LS to RAN4, requesting RAN4 to consider whether or not to introduce UE assistance information related to measurement occasions .. As part of this LS, the RAN1 agreement included FFS points related to measurement occasions, channel conditions, traffic, and UE mobility.

As part of the proposals for UAI, many options relate to the number of SSBs that need to be available in a time window. The argument is that the UE is able to determine how many samples it need without extending the measurement delay based on the SNR. The UE might know the SNR of cells that were previously measured, and assume a smaller number of samples is needed for those based on previous information. However, it is impossible for the UE to estimate the SNR for cells that are unknown (i.e., never sampled in the past) and not yet detected.

In Figure 2 we show one example where the UE can detect 2 different cells at 2 different time instants. In Figure 2 (a) a blocker is placed such that neighbour 1 is not visible for the UE, while Neighbour 2 can be detected. In the same scenario, if either the UE moves or the environment changes (e.g. the blocker moves) neighbour 1 can be detected while neighbour 2 cannot be detected. If we relate this example to the argument that some companies made that the UE can determine the number of samples based on the SNR, how can the UE determine how many samples it would be needed to detect neighbour 1 between time instants (a) and (b)?

The UE cannot determine SNR and number of samples needed for a cell that is not yet detected.

| (a) | (b) |
| --- | --- |

Figure 2 Example of UE monitoring serving cell and neighbour cells, where (a) neighbour 1 is not detectable for the UE and neighbour 2 can be detected, and (b) neighbour 1 can be detected and neighbour 2 is not detectable.

In another proposal, some companies would like the UE to report which measurement occasions are not used and might be used for measurement skipping. The argument of those companies is that the UE is not always performing measurements on every measurement occasion. Therefore, the occasions that are not measured may be used for measurement skipping with reduced impact to the measurement requirements. One challenge with that approach is that if you have latency sensitive traffic, it is not possible to align that traffic to the measurement occasions that the UE would be reporting using UAI. Figure 3 shows one example where there is an XR traffic with a given pattern that would result in 3 out of 9 gaps to require skipping. If a UE reports that it prefers to skip one of every 3 gaps, the resulting desired skipping pattern is only matching 1 of the gaps that require skipping. Therefore, the information cannot be used by the network in a reliable way to improve the XR used experience.

There is no benefit in the UE indicating preference for measurement skipping that is not matching with the XR traffic.

Figure 3 Example of UE reporting preference for measurement occasions for skipping which are not aligned with XR traffic

Therefore, for the UAI information related to measurement occasions, RAN4 has to provide analysis that UAI is beneficial and feasible. Moreover, UAI is optional and network-controlled behavior is required, as the network has best knowledge of both mobility performance and XR traffic quality. Being an optional feature, UAI would make complex to define of measurement requirements in terms of delay and accuracy.

Considering the aforementioned reasons,we still think that UAI is a solution that will bring negligible or no gains.

Do not define UAI related to measurement skipping in Rel 19.

If UAI is agreed by RAN4, there are many details have to be clarified that are not included in the options of Issue 5-1. For example, if the measurement gap is not yet configured, the UE would need to provide the report based on each measurement object. In another example, if there are 2 per FR gaps configured, and other measurements to be performed without gaps, how can the network distinguish if the desired skipping ratio relates to the measurements without gaps, which layers, or which measurement gaps. As each MO may be mapped differently to a gap or SMTC, a single report cannot be enough for guiding the network on informed decisions about skipping. If we agree with L1 measurement skipping, should we also distinguish between RLM, BFD, CBD, L1 RSRP?

If UAI related to measurement occasions is agreed, the report should be given as maximum skipping ratio without measurement impact for each MO in a list of MO configured by the network.

Additional skipping schemes

In the RAN4 #113 meeting, the following issue was left for discussion in the agreed WF [1]:

## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: Follow RAN1 working assumption and consider DCI only. - Option 2: also consider RRC/semi-static approach. - Option 2a: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. - Option 2b: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. - Option 2c: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. - Option 2d: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2e: In addition to DCI solution, it is proposed to introduce semi-static schemes. - Option 3: - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

In our view, measurement gap skipping should be fully under network control. The network has all information available on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Hence Alt 1-1 selected by RAN1 based on DCI scheduling is appropriate for this.

The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Additionally, many companies had concerns regarding Alt 1-1 because of the potential short time for reaction between DCI and measurement gap to be skipped. We already addressed in the last RAN4 meeting when we agreed with a time offset for measurement skipping of 5 ms as baseline.

RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Overall, the performance of semi-static schemes for XR is judged inferior versus the DCI based scheme, as shown in our simulation paper  in RAN4 #113 R4-2418890.As shown in Figure 4 and 5 of R4-2418890, the capacity gain of the semi static scheme (Alt-3) in relation to measurements with gaps is very minor in comparison to the dynamic (DCI-based) scheme. This is also reasoned by the fact that Alt-3 is highly depending on prediction (of configured gaps subject to be skipped) and hence generates useless skipping, whilst XR traffic is highly dynamic, has jitter and hence cannot be matched with a predicted traffic pattern.

Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Another problem related to Alt-3 is that it relies on good traffic pattern prediction before the network has enough information to configure the skipping pattern. Figure 4 shows one example of what happens during the call start of a XR traffic stream. If Alt 3 approach is used as in Figure 4 a), the network needs a training phase in order to determine the XR traffic pattern. Once the prediction is ready at time t1, the network can determine the skipping pattern to be used that matches the XR traffic, and send an RRC command configuring the UE to use that pattern at t2. Therefore, before t2, the UE would experience worst performance related to the XR traffic. On the other hand, Figure 4 b) shows the example of using dynamic approach, as decided by RAN1. When using this approach, the traffic prediction training is not a mandatory step, and the network can start skipping measurement gaps from the start of the XR traffic in t0.

Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

a)

b)

Figure 4 Comparison of XR traffic quality variation over time when using a) semi-static approach (Alt 3) and b) dynamic approach as the decided by RAN1 working assumption (Alt 1-1)

Next, when using semi-static schemes, the network cannot quickly react to changes in channel condition. Consider the example where a UE is configured with a semi-static pattern with 50% skipping and approaches the cell edge. In that situation, once the network notices that the UE is moving toward cell edge, it would need to send RRC reconfiguration to change the pattern. Only after that the UE could perform measurements with short delay. In that case the Alt 1-1 can in a very dynamic way adapt the skipping ratio and prioritize mobility measurements.

Semit static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

Furthermore, RAN1 has already made a down-selection between DCI and RRC schemes

RAN1 already made down selection between DCI and RRC schemes

Follow RAN1 working assumption and consider DCI scheme only.

Conclusion

This paper presents Nokia’s views on RRM requirements for XR measurement skipping. As part of the discussion in the paper, the following observations and proposals are made:

Observation 1: Some measurements without gaps scenarios cause interruptions. Interruptions will cause scheduling problems for latency sensitive traffic.

Observation 2: Requirements for measurements without gaps with interruptions using NeedForInterruptions-r18 do not limit interruption location in time.

Observation 3: Measurements without gaps based on NeedForGaps-r16 do not have interruption requirements and restrictions.

Proposal 1: Measurement skipping does apply for measurements without gaps for the following scenarios:

a.	Intra-frequency without gap for SSB within active BWP in FR2-1

b.	For UE supporting NeedForInterruptions-r18

Proposal 2: For the requirements of measurements without gaps, reuse agreement on time offset requirement for measurement gap skipping

Proposal 3: If interruptions are needed for measurements without gaps, skipping indication cancel the interruptions as well as scheduling restrictions.

Proposal 4: DCI skipping command applies for an entire SMTC.

Proposal 5: Measurement skipping does not apply for L1 measurements and L1 procedures.

Proposal 6: Measurement skipping does not apply for L1 measurements and LTM procedures.

Observation 4: Even if RAN4 agreed on UAI, it will be probably optional UE capability, so RAN4 needs to define requirements without UAI first.

Proposal 7: Discuss first the requirements without UAI consideration.

Proposal 8: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

a.	Max(200 ms  CSSFinter, Ceil( 8  * Kgap)  Max(MGRP , SMTC period)  CSSFinter + Nskip  Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

b.	Use this formula as example and follow similar approach for the other cases.

Proposal 9: RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

Observation 5: It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

Proposal 10: High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

a.	Nskip ≤8  (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5  (CSSF - Nhigh) for intra-frequency measurements

b.	where Nhigh is the number of high priority layers.

Observation 6: Limiting the maximum number of occasions that can be skipped in the specification will have an impact on the network deployment flexibility.

Proposal 11: No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

Proposal 12: RAN4 to consider mobility impacts due to measurement skipping and to start with the handover procedure.

Proposal 13: RAN4 to discuss how to mitigate the impact of measurement delay extension in mobility events.

Proposal 14: No condition is needed for the optional capability of 3 ms.

Observation 8: The UE cannot determine SNR and number of samples needed for a cell that is not yet detected.

Observation 9: There is no benefit in the UE indicating preference for measurement skipping that is not matching with the XR traffic.

Proposal 15: Do not define UAI related to measurement skipping in Rel 19.

Observation 10: If UAI related to measurement occasions is agreed, the report should be given as maximum skipping ratio without measurement impact for each MO in a list of MO configured by the network.

Observation 11: The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Observation 12: RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Observation 13: Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Observation 14: Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

Observation 15: Semit static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

Observation 16: RAN1 already made down selection between DCI and RRC schemes

Proposal 16: Follow RAN1 working assumption and consider DCI scheme only.

- References

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, Qualcomm

- R4-2419326, General aspects for XR enhancements, Nokia

- R4-2419327, Discussion on XR ph3 RRM requirements, Nokia

- R4-2418890, XR simulation results, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024