3GPP TSG-RAN4 Meeting #114	R4-2502164

# Athens, Greece, 17th – 21st February, 2025

Source: 	Qualcomm Incorporated

Title: 	 FR2-1 L3 measurement delay by optimizing RX beam sweeping factor

Agenda item:	 7.17.2.1

Document for:	Discussion

# Introduction

In this paper, we are presenting view for R19 RRM enhancement phase-5 WI.

# Discussion

## Prior Agreements

A subset of agreements and open issues from recent meetings [1][4] are listed below.

Applicability requirement: Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. UE Power class: Agreement in online session: Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: Agreement in online session: Remove {FFS: “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”}. Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception. Issue 1-2-2: FFS: UE’s mobility status, e.g., whether HST is precluded or not Agreement in online session: - RAN4 to consider UE in non-HST case as first priority. - Note: whether or how HST case could use the outcome of the WI discussion can be FFS after concluding on non-HST case. These extra FFS parts will NOT delay the WI completion. Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion. Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment. Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF Agreement: - RAN4 is to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - The detailed definition can be postponed to the end of the core part discussion. Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement. Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability

## Impact of Channel Condition on Triggering Fast Beam Sweeping

RAN4 has agreed to use serving cell’s signal quality (RSRP and/or RSRQ) based triggering condition to activate/deactivate multi-RX for L3 measurement. It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info to change SMTC config for the UE. For example, a UE that can support fast beam sweeping can be configured with longer SMTC periodicity to increase throughput.

Observation 1: It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info to change SMTC config for the UE.

Existing RLM/BFD relaxation mechanisms have introduced signaling and procedures to report RLM/BFD relaxation status. These signaling and procedures are shown in appendix A. These reports for conveying RLM/BFD relaxation status can be reused or served as a baseline for conveying beam sweeping status.

UE should not immediately report to the network after “not cell-edge” conditions are fulfilled. UE’s condition may change, and immediate reporting may lead to a ping-pong effect. UE should wait for a certain time to report to the network after “not cell-edge” conditions are fulfilled. TTT-like time window or N310 like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge” is observed by the UE, can be configured to avoid frequent status transitions and reports.

Observation 2: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 3: After “cell-edge” condition is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

RAN4 can adopt proposal 1 to allow UE to inform its beam sweeping status to network.

Proposal 1: Network configures report for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “cell-edge” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

## RX Beam Sweeping Factor

Rel-19 multi-RX capable UE should, at least, support a fast beam sweeping (FBS) factor of 4. Other FBS factors can be FFS.

Proposal 2: At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS.

# Conclusions

Observation 1: It is useful for network to know UE’s fast beam sweeping related multi-RX status. Network could utilize this info in different purposes, e.g., to change SMTC config for the UE.

Observation 2: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 3: After the triggering condition for fast beam sweeping factor is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

Proposal 1: Network configures report for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “cell-edge” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

Proposal 2: At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS.

# Appendix A: Reporting of BFD/RLM Relaxation State via UAI

UE’s reporting regarding BFD and RLM relaxation status can be summarized through the following:

- bfd-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of BFD measurements.

- rlm-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of RLM measurements.

The requirements for providing RLM/BFD relaxation status are defined as follows in RAN2 and RAN4 specs [2] [3].

NR UE employs the following timers to report RLM/BFD relaxation status [2].

# References

[1] R4-2414026, Way Forward for NR_RRM_Ph5, Apple, RAN4 #112

[2] 38.331 V 17.7.0

[3] 38.133 V 17.11.0

[4] R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 #112bis.