3GPP TSG-RAN4 Meeting #114	R4-2502166

# Athens, Greece, 17th – 21st February, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Core Requirements and Testability Issues

Agenda item:	7.19.2

Document for:	Discussion

# Introduction

In this paper, we focus on CSI prediction and address the issues that are needed to define work item related requirements

# Prior Agreements

Following agreement happened during the last meeting [1].

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

# Discussion

38.843 has emphasized to ensure consistency between network and UE side training [2]

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied

RAN1 has introduced the concept of associated ID to ensure consistency between training and inference regarding network side additional conditions.

Agreement (9.1.1) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement (9.1.1) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Observation 1: 38.843 has emphasized to ensure consistency between network and UE side training. RAN1 has introduced the concept of associated ID to ensure consistency between training and inference regarding network side additional conditions.

3GPP has approved LCM and generalization of one-sided models as a work item. However, RAN4 should start with defining a basic AI-ML CSI prediction test that focuses on a specific scenario.  Hence, RAN4 can simply assume network side additional condition (e.g., configuration, antenna layout, scenario) to be same for both model training and inference in Rel-19 CSI prediction tests. So, RAN4 should use the same network side additional conditions to define the requirements that RAN4 plans to specify in the tests.

Proposal 1: RAN4 uses the same network side additional condition (scenario, configuration, antenna layout, etc.) that would be specified for tests to define the test requirements.

Some companies in RAN4 have proposed to down-select the codebook for AI-ML based CSI report for Rel-18. RAN1 is still discussing down-selecting Rel-18 Doppler codebook or Rel-16 eTypeII codebook.  As following RAN1 agreement shows, RAN1 allowed both Rel-18 Doppler codebook and Rel-16 eTypeII codebook to be used for evaluating CSI report of AI-ML based CSI prediction.

Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

Observation 2: Some companies proposed RAN4 to assume R18 eType II Doppler codebook for CSI report for AI-ML prediction.

Observation 3: RAN1 is still discussing whether R18 eTypeII Doppler codebook or R16 eTypeII codebook would be used for CSI report for AI-ML prediction. RAN1 allowed both Rel-18 Doppler codebook and Rel-16 eTypeII codebook to be used for evaluating CSI report of AI-ML based CSI prediction.

Proposal 2: RAN4 waits until RAN1’s decision to define the codebook for CSI report for AI-ML prediction tests.

# Conclusions

Observation 1: 38.843 has emphasized to ensure consistency between network and UE side training. RAN1 has introduced the concept of associated ID to ensure consistency between training and inference regarding network side additional conditions.

Observation 2: Some companies proposed RAN4 to assume R18 eType II Doppler codebook for CSI report for AI-ML prediction.

Observation 3: RAN1 is still discussing whether R18 eTypeII Doppler codebook or R16 eTypeII codebook would be used for CSI report for AI-ML prediction. RAN1 allowed both Rel-18 Doppler codebook and Rel-16 eTypeII codebook to be used for evaluating CSI report of AI-ML based CSI prediction.

Proposal 1: RAN4 uses the same network side additional condition (scenario, configuration, antenna layout, etc.) that would be specified for tests to define the test requirements.

Proposal 2: RAN4 waits until RAN1’s decision to define the codebook for CSI report for AI-ML prediction tests.

# References

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- 3GPP TR 38.843 V 18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- R1 116bis Chairman’s notes, Hunan, China, April 2024.