3GPP TSG-RAN4 Meeting #114	R4-2502170

# Athens, Greece, 17th – 21st February, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for low band carrier aggregation via switching

Agenda item:	7.8.3

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106. We discuss the RRM related topics that might be potentially impacted through this work item.

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remain under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner.

The work item document discussed following objectives of this work item [1]

Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] Objective of Performance part WI - Specify RRM test cases

Objective 1: Low band carrier aggregation via switching work item is expected to specify RRM requirements due to semi-static switching between TX/RX on FDD carrier 1 and RX on SDL carrier 2.

The RRC configured semi-static switching pattern between case 1 and case 2 can impact, at least, following areas of RAN4 RRM spec:

- RLM, BFD, L1-RSRP and L3 intra-frequency/inter-frequency measurements

- SCell activation

Proposal 1: RAN4 investigates, at least, the following topics as potential impact areas of “low band carrier aggregation via switching” work item:

- RLM, BFD, L1-RSRP and L3 intra-frequency/inter-frequency measurements

- SCell activation

# Conclusions

Objective 1: Low band carrier aggregation via switching work item is expected to specify RRM requirements due to semi-static switching between TX/RX on FDD carrier 1 and RX on SDL carrier 2.

Proposal 1: RAN4 investigates, at least, the following topics as potential impact areas of “low band carrier aggregation via switching” work item:

- RLM, BFD, L1-RSRP and L3 intra-frequency/inter-frequency measurements

- SCell activation

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.