3GPP TSG-RAN4 Meeting #114	R4-2502173

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title: 	Discussion on Ku-band VSAT output power and different antenna sizes

Agenda Item:	7.11.4

Document for:	Discussion

# Introduction

In the 3GPP RAN4 Meeting#113 it was agreed in [1] that

- The 20cm x 20cm, 40cm x 40cm and 60cm x 60cm size of the VSAT antenna models can be considered for the VSAT types with electronic steering antenna in the Ku band normative work

  - Detail parameters can be further checked and FFS on mapping to the 38.101-5 VSAT types and conducting the coexistence study

and in addition, it was also agreed that

- For the Ku band with uplink 14 – 14.5 GHz (Earth-to-Space) concerning FCC requirements, consider the following as the starting point:

  - The current additional spectrum emission mask requirement for n511, n510 can be applied to the Ku band.

- Consider the off-axis EIRP density limits requirement in 38.101-5 based on the Table 1-2-7.5~1-2-7.7.

Table 1-2-7.5: Off-axis EIRP density limits for co-polarized transmissions in the plane tangent to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 15 – 25log10(θ) | 4 |
| 7° ≤ θ ≤ 9.2° | -6 | 4 |
| 9.2° ≤ θ ≤ 19.1° | 18 – 25log10(θ) | 4 |
| 19.1° < θ ≤ 180° | -14 | 4 |

Table 1-2-7.6: Off-axis EIRP density limits for co-polarized transmissions in the plane perpendicular to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 3° ≤ θ ≤ 19.1° | 18 – 25log10(θ) | 4 |
| 19.1° < θ ≤ 180° | -14 | 4 |

Table 1-2-7.7: Off-axis EIRP density limits for cross-polarized transmissions in the plane tangent to the GSO arc and in the plane perpendicular to the GSO arc for the conventional Ku-band

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.5° ≤ θ ≤ 7° | 5 – 25log10(θ) | 4 |

In this contribution, we discuss the VSAT output power ranges for the above-mentioned antenna sizes and off-axis emission requirements.

# Discussion

## Comparison of FCC Ka-band and Ku-band proposed Off-axis EIRP density requirements

The purpose of the NTN VSAT Off-axis EIRP density requirements is to ensure that the UE transmit beam has narrow enough shape vs. the transmitted output power so that other satellites in the same area with the target satellite are not interfered. During the Ka-band work, it was also discussed in RAN4 that meeting the Off-axis requirements with antenna matrix using electronic beam steering may require reduction of the output power although no MPR requirements were defined. This is also captured into the TS 38.101-5 [2] definition of the NTN VSAT device types as shown below.

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

In order to compare the Ka- and Ku-band Off-axis EIRP emission density requirements the main requirement items for cross-polarized Off-axis EIRP emission density for band n510 and n511 and FCC requirements for conventional Ku-band are collected into the table below. The comparison is done using the limits for cross-polarized transmissions only to understand the scale and delta of the differences and impact to VSAT output power. The limits for co-polarized transmission have similar delta between Ku and Ka-bands, but allow about 10dB higher PSD level than the cross-polarized one with θ ≤ 7°.

Table 1

| Parameter | Ka-band (n511 and n510) | Ka-band (n512) | Ku-band under discussion |
| --- | --- | --- | --- |
| Frequency range | 27.5 – 30GHz |  | 14 – 14.5GHz |
| Minimum peak EIRP GSO | 70 dBm (Fixed and Mobile VSAT) |  |  |
| Minimum peak EIRP LEO | 61 dBm (Fixed VSAT only) |  |  |
| Measurement bandwidth for Off-axis EIRP emission density | 1MHz | 40kHz | 4kHz |
| Range of θ value of | 2.0° ≤ θ ≤ 7° | 1.8° ≤ θ ≤ 7° (Fixed) 2.0° ≤ θ ≤ 7° (Mobile) | 1.5° ≤ θ ≤ 7° |
| Maximum EIRP level | 52.5 – 25log10(θ) dBm | 39 – 25log10(θ) – K dBm | 5 – 25log10(θ) dBW |

The main differences between the requirements on Ka- and Ku-band are the measurement bandwidth, where 1MHz bandwidth is used for Ka-band and relatively narrow 4kHz bandwidth is used for Ku-band, range of θ, where Ku-band requirement starts already from angle of 1.5° and EIRP levels that follow the same function, but there’s large offset between them. The offset between the EIRP levels is partly resulting from dBW vs. dBm units and measurement bandwidth difference, but even with same unit and measurement bandwidth the allowed emissions levels for Ku-band are about 6-6.5dB higher than those on Ka-bands n511, n510 and n512. However, it’s to be noted here that, the analysis here only applies for signals with flat power spectral density (PSD) and usually the emission limits with narrow measurement bandwidth are more difficult to achieve than the ones with wider one.

The emission limits for different values of θ are shown in more detail in figure below.

Figure 1. Comparison of cross-polarized EIRP emission limits.

Observation 1: The Off-axis EIRP limits for conventional Ku-band start already from 1.5° θ-angle, but allow 6-6.5dBm higher emissions than on Ka-band.

Observation 2: FCC Ku-band requirements are defined using 4kHz measurement bandwidth, which is significantly narrower than 1MHz used for bands n511 and n510 (FCC) and 40kHz used for n512 (CEPT). Emission limits with narrow measurement bandwidth are usually more difficult to achieve that the ones with wider one.

## VSAT output power estimation for different sized antenna matrix

The EIRP output power of the VSAT is a function of the conducted power (TRP), that is fed into the antenna system and the gain of the antenna system. For the system simulations following assumptions are agreed and these can be used to estimate the maximum achievable EIRP for the VSAT with different sized antennas as shown below.

Conducted power per feed: 4.44dBm

Element antenna gain: 3.5dBi including the implementation losses.

Table 2

|  | Small | Medium | Large |  |
| --- | --- | --- | --- | --- |
| Physical size | 20x20 | 40x40 | 60x60 | cm |
| Diamater (~aperture size) | 28 | 57 | 85 | cm |
| N-elements X/Y | 15/15 | 30/30 | 45/45 |  |
| N-elements | 225 | 900 | 2025 |  |
| Antenna gain | 27.0 | 33.0 | 36.6 | dB |
| TRP (sum power of all feeds) | 28.0 | 34.0 | 37.5 | dB |
| Maximum Peak EIRP | 55.0 | 67.0 | 74.1 | dBm |

The Maximum Peak EIRP shown above is achieved in this kind of panel antennas towards boresight direction, while Minimum Peak EIRP in 38.101-5 is defined for the manufacturer declared minimum elevation angle supported for transmitting and is within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.2-1 is used.

If electrical steering is needed to reach the supported minimum elevation angle the achievable EIRP drops and RAN4 should take this into account when defining the output power requirements for VSAT for this kind of antennas. This drop in EIRP with 70 degrees electronic tilt and elevation of 20 degrees using the coordinate systems in TS 38.101-5 Figure 9.2.1.2-1 is shown in figure below using the antenna parameters that are used in Ku-band system simulations. Additional implementation margins may also be necessary to support different types of implementations but at the same time the minimum link budget for targeted service should be guaranteed. It may be also possible for VSAT to compensate for the drop in antenna gain by boosting the conducted input power to the antenna system when operating the lower elevation angles and this needs to be also considered for definition of TRP requirement.

Observation 3: Using the Ku-band system simulation model the Maximum Peak EIRP for 20x20, 40x40 and 60x60cm antennas is 55, 67 and 74.1dBm respectively. However, as Minimum Peak EIRP in 38.101-5 is defined for the manufacturer declared minimum elevation angle supported for transmitting the minimum requirement could be lower than this due to antenna gain and power degradation caused by electronic antenna tilting. Additional implementation margins may also be necessary to support different types of implementations but at the same time, the minimum link budget for targeted service should be achieved. It may be also possible for VSAT to compensate for the drop in antenna gain by boosting the conducted input power to the antenna system when operating these lower elevation angles and this needs to be also considered for definition of TRP requirement.

Figure 2. Antenna radiation patterns with different Etilt values

## VSAT output power vs. proposed Off-axis EIRP density requirements

As already captured into the TS 38.101-5 sub-clause 9.2, NTN VSAT with electronic steering antenna may need power reduction to comply with OFF-axis EIRP requirements. The compliance of the different sized antennas vs. FCC conventional Ku-band Off-axis EIRP density requirement is analyzed in figures below for 20x20cm (Small); 40x40cm (Medium) and 60x60cm (Large) antenna panels using the same system simulator antenna model as in previous section.

As the Off-axis requirement is a density / power spectral density requirement the VSAT performance changes with the used transmission bandwidth.

Observation 4: Even if the proposed Off-axis PSD limits for Ku-band are higher than those of Ka-band none of the antennas used complies with the requirement with SCS= 60kHz single RB transmission.

Observation 5: 20x20cm antenna complies with the proposed limits with 11RBs (10MHz CBW). 40x40cm antenna complies with the limits with 66RBs (50MHz CBW). 60x60cm antenna complies with the limits with 132RBs (100MHz CBW).

Observation 6: Analysis here is done only for electronic tilt of 0 degrees (90degrees elevation) using the system simulation antenna model and hence further analysis taking also other aspects, including the different elevations may be necessary, but based on the results shown here it’s possible to conclude that also the Ku-band NTN VSAT with electronic steering antenna may need power reduction to comply with OFF-axis EIRP requirements.

Proposal 1: It’s proposed to use the same approach for the Off-axis power for Ku-band NTN VSAT with electronic steering antenna as already used for similar VSAT device type on Ka-band in the Table 9.2.1.0-1: The definitions of NTN VSAT Types.

Figure 3. 90degrees elevation Off-axis performance for 20x20cm, 40x40cm and 60x60cm antenna

# Conclusions

Observation 1: The Off-axis EIRP limits for conventional Ku-band start already from 1.5° θ-angle, but allow 6-6.5dBm higher emissions than on Ka-band.

Observation 2: FCC Ku-band requirements are defined using 4kHz measurement bandwidth, which is significantly narrower than 1MHz used for bands n511 and n510 (FCC) and 40kHz used for n512 (CEPT). Emission limits with narrow measurement bandwidth are usually more difficult to achieve that the ones with wider one.

Observation 3: Using the Ku-band system simulation model the Maximum Peak EIRP for 20x20, 40x40 and 60x60cm antennas is 55, 67 and 74.1dBm respectively. However, as Minimum Peak EIRP in 38.101-5 is defined for the manufacturer declared minimum elevation angle supported for transmitting the minimum requirement could be lower than this due to antenna gain and power degradation caused by electronic antenna tilting. Additional implementation margins may also be necessary to support different types of implementations but at the same time, the minimum link budget for targeted service should be achieved. It may be also possible for VSAT to compensate for the drop in antenna gain by boosting the conducted input power to the antenna system when operating these lower elevation angles and this needs to be considered for definition of TRP requirement.

Observation 4: Even if the proposed Off-axis PSD limits for Ku-band are higher than those of Ka-band none of the antennas used complies with the requirement with SCS= 60kHz single RB transmission.

Observation 5: 20x20cm antenna complies with the proposed limits with 11RBs (10MHz CBW). 40x40cm antenna complies with the limits with 66RBs (50MHz CBW). 60x60cm antenna complies with the limits with 132RBs (100MHz CBW).

Observation 6: Analysis here is done only for electronic tilt of 0 degrees (90degrees elevation) using the system simulation antenna model and hence further analysis taking also other aspects, including the different elevations may be necessary, but based on the results shown here it’s possible to conclude that also the Ku-band NTN VSAT with electronic steering antenna may need power reduction to comply with OFF-axis EIRP requirements.

Proposal 1: It’s proposed to use the same approach for the Off-axis power for Ku-band NTN VSAT with electronic steering antenna as already used for similar VSAT device type on Ka-band in the Table 9.2.1.0-1: The definitions of NTN VSAT Types.

# References

[1] Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance (Release 18)