3GPP TSG-RAN WG4 Meeting #114	R4-2502181

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.17.2.2

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing CSSF outside gap in CA/DC scenarios for L3 measurement delay reduction.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 further discussed three tentative solutions to optimize CSSF outside gap in R19 for L3 measurement delay reduction. The first one proposes to measure only one serving CC per band if multiple serving CCs are in the same band. The second one is whether to use a variable ratio to share the searchers with different CCs. The third one is whether to introduce a third searcher to optimize CSSF. Those solutions are captured below WF [2].

Issue 2-4-1: Tentative solution 1: measure one serving CC per band [Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs [Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

For solution 1 in Issue 2-4-1, the CSSF is optimized by measuring one serving CC per band if multiple serving CCs are in the same band. This means instead of sharing a searcher with multiple CCs in the same band as in the legacy, the whole searcher (or more percentage of the searcher) can now be used only for one of the CCs in that band. The method of how UE selects the CC to be measured when multiple CCs are configured in the same band is another issue need to be discussed. It is known that when NW provides the MOs for the configured CCs, UE is required to measure those CCs. Therefore, NW can indicate which CC to be measured per band by providing the MO for that specific CC (i.e., without providing MOs for the other CCs not to be measured in the same band). If NW provides MOs for multiple CCs in the same band, then UE can select one of them based on the following rule:

•	Measure PCC if PCC in band or

•	Measure PSCC if PSCC in band or

•	Measure SCC with neighbour cell MO or

•	Otherwise, up to UE implementation

Proposal 1: Specify solution#1 to optimize CSSF: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 2: NW can provide single MO for the CCs to be measured in a single band. If multiple MOs are provided for multiple CCs in the same band, then UE can select to

- Measure PCC if PCC in band or

- Measure PSCC if PSCC in band or

- Measure SCC with neighbour cell MO or

- Otherwise, up to UE implementation

For solution 2 in Issue 2-4-2, some companies proposed to change the searcher occupancy ratio of measurement among the measured CCs. In legacy, 1 searcher was dedicated to PCC as it is considered higher priority to maintain than other SCCs, while other SCCs share the same searcher equivalently. With solution 2, some CC(s) can have higher ratio of the searcher than the others. This solution will also require NW indication to indicate how much percentage of the searcher to be used by specific CC. Therefore, we think this solution is more complicated than solution 1 and we prefer solution 1 over 2. However, if RAN4 agree to introduce solution 2, i.e., change the searcher occupancy ratio, PCC’s searcher should not be impacted as it is higher priority to maintain; and how to change the second searcher for SCCs can be FFS

Proposal 3: Solution#1 is preferred over Solution#2. However, if RAN4 agree to introduce Solution#2 then PCC’s searcher should not be impacted.

For solution 3 in Issue 2-4-3, RAN4 further discussed the issue of introducing the third searcher to optimize CSSF. Although it is mentioned in the WID that 2 searchers are the baseline assumption in this WI, some companies proposed to consider 3 searchers for CSSF optimization. Given Solution#1 where UE measures one serving CC per band if multiple serving CCs are in the same band, we don’t see the benefit of the third searcher if all the CCs are configured within two bands. The third searcher might be useful when there is a CC to be measured in a third band. However, since there were different views on whether to introduce 3 searchers or not, we can compromise to limit 3 searchers for FR1 scenario only, otherwise RAN4 to prioritize optimizing CSSF outside gap based on 2 searchers.

Proposal 4: If 3 searchers is supported to optimize CSSF, RAN4 to limit the scenarios for FR1 only, otherwise RAN4 to prioritize optimizing CSSF based on 2 searchers.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Specify solution#1 to optimize CSSF: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 2: NW can provide single MO for the CCs to be measured in a single band. If multiple MOs are provided for multiple CCs in the same band, then UE can select to

- Measure PCC if PCC in band or

- Measure PSCC if PSCC in band or

- Measure SCC with neighbour cell MO or

- Otherwise, up to UE implementation

Proposal 3: Solution#1 is preferred over Solution#2. However, if RAN4 agree to introduce Solution#2 then PCC’s searcher should not be impacted.

Proposal 4: If 3 searchers is supported to optimize CSSF, RAN4 to limit the scenarios for FR1 only, otherwise RAN4 to prioritize optimizing CSSF based on 2 searchers.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.