3GPP TSG-RAN WG4 Meeting #114									                        R4-2502191

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.1.1.3.4

Source:	Ericsson

Title:	On the SRS IL imbalance reporting details for the SRS-AS

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, and in this contribution we only focus on the study of the issue of insertion loss imbalance across SRS ports:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - …

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Due to divergent views from different companies, progress on this topic is slow. However, in the last two meetings, there was a convergence towards a candidate solution framework which gathers different proposals, where the exact details are still for further study (see the approved WF [2]).

Same as in our contribution for the previous meeting, we will not repeat the detailed analysis on the general considerations for the SRS IL imbalance issue nor the justification for the introduction of reporting. Instead, we will only repeat the main observations and refer to the corresponding contributions where more details can be found.

Regarding the existing UE behavior, according to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations (see [3] for more details).

Regarding the impact of SRS IL imbalance on NW performance degradation, based on system-level simulation results, we have demonstrated that the SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss (see [3] for more details).

In [4], we have provided lab testing results of a commercial UE to verify the reciprocity of the Tx and Rx imbalance. We observed that the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance. We have also observed that contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior.

There are also other issues which have been observed for multiple commercial UEs during the lab testing, including the Rx chains being switched off (presumably for power saving reasons) during the periods when there is no DL transmission. In such cases, the SRS has not been transmitted from the corresponding Rx ports, which breaks the specifications.

In general, UL RMCs and parameters for verifying SRS requirements in TS38.101-1, clause 6.2.4 with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching' or ‘codeBook’ are not specified. This means that UE conformance with SRS transmitter power requirements and the ΔTRxSRS for antenna switching cannot be verified. In our contribution [5] (as a part of TEI-17), we have proposed RMCs for SRS transmissions that should be introduced to TS38.101-1, where the rest could be handled by RAN5.

# Discussion

In the approved WF [2], the following candidate solution framework has been captured, where the details of SRS IL self-compensation and assistance framework are still FFS:

Candidate solution framework - UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact. - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others - Details of SRS IL self-compensation and assistance framework are FFS

According to the WF, companies are “encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed”.

In the following, we will propose the details of the SRS IL self-compensation and assistance framework, and then we will discuss the impact on the different specifications (on high-level).

First, we make the following proposals:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

## Details of the proposed candidate solution

### Details of the SRS IL self-compensation part of solution

As discussed in the introduction and captured by the WF [2], the UE shall perform self-compensation for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations. Since PCMAX takes into account the insertion loss by allowing the maximum relaxation ΔTRxSRS in its lower bound and is defined for each SRS resource separately, each SRS transmission power below PCMAX shall be feasible. Thus, the UE should perform the self-compensation of the SRS IL up to the maximum configured output power for the given SRS resource.

Since this behavior is already captured in the specifications and no changes have been proposed, no additional RAN4 performance requirements would need to be introduced.

Observation 1: No additional RAN4 performance requirement needs to be introduced to verify that the UE performs the IL self-compensation.

Observation 2: For the SRS IL self-compensation part of solution, there is no impact on RAN1 specifications since the IL is not considered in the SRS power control equations.

### Details of the assistance framework part of solution

Since the UE cannot compensate for the IL at the maximum configured output power, it should provide assistance to the network on the SRS IL compensation.

One candidate reporting parameter is the “SRS insertion loss value (per SRS resource or per UE)”. In our understanding, this is related to the actual insertion loss of different branches/SRS ports. In general, this could be a promising solution as long as the imbalance between the actual SRS transmission powers is equal to the actual insertion loss differences, which should be the case if the UE transmits at the maximum configured output power PCMAX at each SRS port.

However, as it can be concluded from the online discussions, the UE behavior in the field on the IL compensation may be unpredictable. If the network takes into account the reported actual “SRS insertion loss value” and the UE compensates for the insertion loss, that may introduce even more inaccuracy for the DL CSI estimation.

Even if the network knew that the UE behaves as it is supposed to, i.e. pre-compensating for all the powers below the maximum configured output power, there is no way for the network to know if the UE has been able to compensate for the insertion losses (UE being near the cell center vs. being near the cell edge). Only if the PHR were transmitted for each SRS resource together with the actual insertion loss, the network would know whether to take into account the information on the actual insertion loss (whether the SRS transmission output power is the equal on all SRS resources/ports or it differs by the amount of the actual insertion losses).

In addition, there is no guarantee that the UE will set the maximum configured output power according to the actual insertion loss (for example, the chosen PCMAX could be “overoptimistic”), and in that case only a partial compensation of the IL may occur.

Finally, the mapping between the SRS ports and the antenna connectors can change with the time, and the “SRS insertion loss value” may become outdated.

Observation 3: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Our proposed solution for the “UE assistance to the network” part is that reporting should include the configured maximum output power per SRS resource and the power headroom (PH) per SRS resource, as well as the information on the power class fallback or the return to the supported power class (ΔPPowerClass). In that way the network can have a relatively precise information on each SRS output power at the connector. Note that reporting of all three of these parameters is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”.

The configured maximum output power includes the actual insertion loss (Tx/Rx) imbalance (for resources for which this is allowed) on top of the MPR used for the SRS resource. If the MPR is the same between the different ports (which is expected), by inspecting the differences between the configured maximum output power per SRS port the information on the IL imbalance could be obtained. On the other hand, by transmitting in addition the power headroom (PH) report per SRS resource, the network can be aware of whether a UE operates at its maximum power and it can obtain the information on the actual output power per SRS resource by subtracting the power headroom from the configured maximum output power (in that way it can observe whether the IL compensation has been performed). So, both cases when UEs perform or do not perform a compensation for the ILs are covered by the proposed solution.

One of the motivations to include ΔPPowerClass in the reporting is the discussion under the Rel-18 maintenance agenda for “Further RF requirements enhancement for NR frequency range 1 (FR1)” WI, where the ΔPPowerClass requirements for AS for 4Tx are currently still incomplete. To see a detailed argumentation from our side, one could check our contribution [6]. Here, we will only reiterate that the solution similar to the one specified for 2Tx would be sub-optimal (4Tx adds a lot of complexity compared with 2Tx) and that the solution for that issue could be further improved by introducing the reporting of related to the SRS IL.

The expected benefit from such solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of the DL MIMO, both Single User and Multi-User.

Proposal 3: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

Observation 4: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

### Reporting per SRS resource or per SRS port?

According to TS38.213 [7], clause 7.3, if the non-TDMed SRS resource consists of multiple ports then its transmission power is equally divided across all ports (at all power levels): “- else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the SRS ports of each SRS resource of an SRS resource set in a symbol for SRS transmission.” In that case, the mapping between the different SRS ports and the actual antenna connectors in the UE is up to UE implementation (but different pairs should be used for each resource).

From that perspective, if all the UEs in the field were following the specifications (which is mandatory) and no tolerances were allowed, there would be no need to for the insertion loss reporting per SRS port. The reporting per SRS resource would be sufficient, where if the SRS resource consists of more than one port, the network would know that the estimated transmitted SRS output power, summed from all SRS ports (obtained by subtracting PHR from PCMAX), should be divided by the number of ports to estimate the SRS output power per port.

However, in practice, according to our lab testing it turns out that for some UEs there is up to 4 dB difference between two SRS ports of the same SRS resource. In some cases, the ΔTRxSRS requirement is not met (the relaxation is much larger), and for that reason, it is very important to introduce conformance tests for the existing SRS transmitter power requirements and the ΔTRxSRS for antenna switching, as mentioned in the introduction (see our contribution [5]).

On the other hand, to increase even further the performance benefit of the reporting related to the insertion loss in practical scenarios, it would be helpful if the reporting was per SRS port: both the configured maximum power for SRS transmission PCMAX and the Power Headroom (PHR) would be defined and reported per port. This should work regardless of compensation as long as the PCMAX,f,c value is at the antenna connector, the PH is a relative measure. The PH will be larger in case the UE does not compensate (then the PH is w r t the sounding PA output power before the diversity antenna arrangement with the additional IL) but would never exceed the max power at the antenna connector (essentially “PA power class” – additional IL) in any case. The PH will be smaller below max power in case the UE compensates and sets the power according to TS38.213. UEs not compensating would not violate RAN4 power-control requirements below the max power, as the tolerances are huge.

However, the reporting per SRS port would increase the impact on RAN1 and RAN2 specifications, as well as on the control signaling overhead.

Obviously, in the case of single port SRS transmissions and “t1ry” (e.g. “t1r6”) reported SRS Antenna Switching capabilities, reporting per SRS resource is equivalent to reporting per SRS port. Only for multi-port transmissions (e.g. “t2r4”) this question would matter.

Observation 5: To address the issue observed in practice of non-equal transmitted power split between the SRS ports in a multi-port SRS resource, it would be even more beneficial if the reporting were per SRS port, i.e. both power headroom reporting and the configured maximum output power reporting per SRS port. However, such reporting would increase the impact on RAN1 and RAN2 specifications, as well as on the control signaling overhead.

Still, the reporting per SRS resource would bring a lot of benefits for the performance of reciprocity-based channel estimation due to a very relaxed ΔTRxSRS requirement, which is not tested and not always respected in practice. Also, the Tx imbalance is bigger and non-reciprocal to the imbalance on the receiving side (due to the IL). Such reporting is very well suited for the “t1ry” SRS-AS and is sufficiently good for multi-port SRS.

## Detailed impact on the specifications

### RAN4 specifications: TS38.101-1

By adopting Proposal 3 there would be no/very limited impact on TS38.101-1 specification.

### RAN1 specifications: TS38.213

The Type 3 power headroom report is defined in clause 7.7.3 of TS38.213[7].

For insertion loss reporting per SRS resource, the only required change would be to allow Type 3 power headroom report for the UE configured for PUSCH transmissions on carrier f of serving cell c (for both an actual and a reference SRS transmission). In the following, we highlight the exact places in the specifications where the applicability of Type 3 power headroom is limited only to “PUSCH-less” carriers, e.g. SDL. This restriction should be lifted at least for a new MAC-CE, named for example “SRS resource PHR” (see clause 2.2.3), which should be straightforward to incorporate into the text.

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier of serving cell c and if the UE is not configured for PUSCH transmissions on carrier of serving cell c and the resource for the SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where , , , , and are defined in clause 7.3.1 with corresponding values provided by SRS-ResourceSet. If the UE determines that a Type 3 power headroom report for an activated serving cell is based on a reference SRS transmission then, for SRS transmission occasion on UL BWP of carrier of serving cell c, and if the UE is not configured for PUSCH transmissions on UL BWP of carrier of serving cell c and a resource for the reference SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where is an SRS resource set corresponding to SRS-ResourceSetId = 0 for UL BWP and , , and are defined in clause 7.3.1 with corresponding values obtained from SRS-ResourceSetId = 0 for UL BWP . is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB and TC =0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5].

One possible way to do it is the following:

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier f of serving cell c and if the UE is not configured for PUSCH transmissions on carrier f of serving cell c and the resource for the SRS transmission is provided by SRS-Resource [or if the UE is configured for PUSCH transmissions on carrier c of serving cell f and the resource for the SRS transmission is provided by SRS-Resource with usage antennaSwitching], the UE computes a Type 3 power headroom report as

### RAN2 specifications: TS38.321

Power Headroom (PH) framework specified in TS38.321 [8] could be used for the proposed reporting solution. PH used for the SRS resource can be a Type 3 but used for a new MAC-CE, for example called “SRS resource PHR”. A similar approach as for the “Multiple Entry PHR MAC CE” could be used (clause 6.1.3.9 in [8]), where multiple PH (power headroom), PCMAX (maximum configured output power) and DPC (ΔPPowerClass) fields (each set corresponding to a different SRS resource) could be transmitted in a single MAC CE. The maximum number of entries could be eight, which corresponds to 8 Rx case and 1T8R AS, where an SRS Resource Set consists of eight single port SRS resources. A new MAC-CE specified for SRS power reporting should be dynamic since SRS AS power levels can change.

The “SRS resource PHR MAC CE” could be defined similar to following:

-	Ci: This field indicates the presence of a PH field for the SRS resource with index i. The Ci field set to 1 indicates that a PH field for the SRS resource with index i is reported. The Ci field set to 0 indicates that a PH field the SRS resource with index i is not reported;

-	V: This field indicates if the PH value is based on a real transmission or a reference format. The V field set to 0 indicates real transmission on SRS and the V field set to 1 indicates that an SRS reference format is used. Furthermore, the V field set to 0 indicates the presence of the octet containing the associated PCMAX,f,c field and the DPC field, and the V field set to 1 indicates that the octet containing the associated PCMAX,f,c field and the DPC field is omitted;

-	Power Headroom (PH): This field indicates the power headroom level for the SRS resource i. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in TBD;

-	P: This field indicates whether power backoff is applied due to power management (as allowed by P-MPRc as specified in TS 38.101-1, TS 38.101-2, and TS 38.101-3). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c i: If present, this field indicates the PCMAX,f,c (as specified in TS 38.213) for the SRS resource i used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in TBD (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133);

-	DPC: If dpc-Reporting-FR1 is configured, and the Serving Cell operates on FR1, this field indicates the ΔPPowerClass for SRS resource i, as specified in TS 38.101-1[14] and TS 38.101-3 [16]. This field indicates an index to Table 6.1.3.8-4 and the corresponding measured values of DPC levels in dB are specified in TS 38.133 [11], the DPC field is set to 0 if the criteria to report DPC is not met. The length of the field is 2 bits.

| C7 | C6 | C5 | C4 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P | V | PH (Type 3, SRS Resource 0) |  |  |  |  |  |
| DPC0 |  | PCMAX,f,c 0 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 1) |  |  |  |  |  |
| DPC1 |  | PCMAX,f,c 1 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 2) |  |  |  |  |  |
| DPC2 |  | PCMAX,f,c 2 |  |  |  |  |  |

…

| P | V | PH (Type 3, SRS Resource 7) |
| --- | --- | --- |
| DPC7 |  | PCMAX,f,c 7 |

Figure 1: Possible configuration of “SRS resource PHR MAC CE” with the highest number of SRS resources equal to 8 (corresponding to ‘t1r8’ AS)

Such PHR configuration would be suitable even if the IL reporting were per SRS port, since the maximum total number of SRS ports over all SRS resources remains equal to eight (for 8Rx case).

The exact details of the PHR triggering conditions would also have to be discussed.

# Conclusion

In the previous sections, we made the following observations:

Observation 1: No additional RAN4 performance requirement needs to be introduced to verify that the UE performs the IL self-compensation.

Observation 2: For the SRS IL self-compensation part of solution, there is no impact on RAN1 specifications since the IL is not considered in the SRS power control equations.

Observation 3: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Observation 4: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

Observation 5: To address the issue observed in practice of non-equal transmitted power split between the SRS ports in a multi-port SRS resource, it would be even more beneficial if the reporting were per SRS port, i.e. both power headroom reporting and the configured maximum output power reporting per SRS port. However, such reporting would increase the impact on RAN1 and RAN2 specifications, as well as on the control signaling overhead.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Proposal 3: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2420376, “WF on 6Rx requirements”, AT&T, RAN4#113.

- R4-2413360, “On need for SRS insertion loss imbalance reporting”, Ericsson, RAN4#112.

- R4-2415313, “On the SRS insertion loss imbalance reporting”, Ericsson, RAN4#112-bis.

- R4-2501566, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, Ericsson, RAN4#114.

- R4-2502187, “(NR_ENDC_RF_FR1_enh2-Core) On DeltaP_PowerClass for SRS AS for 4Tx”, Ericsson, RAN4#114.

- 3GPP TS38.213 V18.5.0, “NR; Physical layer procedures for control”.

- 3GPP TS38.321 V18.4.0, “NR; Medium Access Control (MAC) protocol specification”.