3GPP TSG-RAN WG4 Meeting #114									                        R4-2502194

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.10.2.2.2

Source:	Ericsson

Title:	On UE RF Rx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Rx requirements for NR-NTN HPUE from the approved WF in RAN4#113 [2].

# Discussion

In the approved WF [2], the following issues are for further study:

Issue 3-1-1: General considerations

- Proposals

  - Option 1: For power class 2, there is no differentiation of handheld and non-handheld UE. (P2b/R4-2418201)

  - Option 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE. (P2/R4-2419717)

In TS38.101-1 [3], the UL configuration for the REFSENs test does not depend on the applied Power Class. So, the same approach could be adopted for NTN.

Proposal 1: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE.

Issue 3-2: RSD requirements for PC1.5

- Proposals

  - Option 1: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE (P1/R4-2417815)

  - Option 2: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only (P3/R4-2418201)

  - Option 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)

  - Option 4: [2.2] dB RSD for PC1.5 with 2Tx architecture for n256.

Issue 3-3: RSD requirements for PC1

- Proposals

  - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (P4a/R4-2418201)

    - Option 1-1: FFS for PC1 with 2 or 3 Tx cases (P4b/R4-2418201)

  - Option 2 no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)

For TN, the RSD requirements are not specified for PC1.5 and PC1. FDD bands do not support PC1.5, while PC1 UEs are not targeted for smartphone form factor. For the NTN, no RSD for PC1.5 and PC1 can be taken as the starting point.

Proposal 2: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1.

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE.

Proposal 2: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2420403, “WF on NR NTN HPUE”, Samsung, RAN4#113.

- 3GPP TS 38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”