3GPP TSG-RAN WG4 Meeting #114	R4-2502231

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.3.1

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# Introduction

For release 19 work item on Evolution of NR duplex operation: Sub-band full duplex (SBFD), the main objective is to specify the sub-band full duplex operation. In addition to that, enhancements to CLI handling scheme is another WID [1] objective that is expected to have impact on RRM requirements. RAN4 has already started discussions on the RRM requirements and reached a few high-level agreements [2,3,4] in the last meetings. In this paper, we further discuss various issues related to L1-CLI requirements.

# L1 based UE-to-UE CLI measurement requirements

As we know, RAN1 has agreed to define L1-SRS-RSRP and L1-CLI-RSSI as two new measurement report quantities and has finalized upon method #1, #2 and#3 to be used to perform CLI measurements.

Until the last RAN4 meeting, only Method#1 and#2 were agreed in RAN1, while Method#3 got agreed during the last RAN1 meeting. RAN4 made some preliminary agreements to specify RRM requirements considering Method#1 and #2 and now also needs to consider Method#3 we well.

- RAN4 shall specify L1 CLI measurement requirements at least for below methods:

  - Method#1: UE measures RSSI within DL subband

  - Method#2: UE measures RSRP of aggressor UE within UL subband

- Additional methods can be considered based on further RAN1 agreements.

- Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Next, we discuss some of the RRM requirements to be specified for the two new measurement metrics considering the agreed methods.

Scheduling Restrictions

We have the following agreement from the last RAN4 meeting:

- For scheduling restriction,

  - For L1-SRS-RSRP,

    - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption

    - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption

  - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing.

Let’s first discuss the scheduling restrictions for L1-SRS-RSRP. RAN4 sent an LS to RAN1 asking whether L1 UE-to-UE CLI measurements are applicable only to intra-cell scenarios or inter-cell scenarios as well. RAN1 has sent a reply LS [5] mentioning that RAN1 has not made a distinction between the two deployment scenarios (intra-cell or inter-cell). RAN1 further mentions that although L1 UE-to-UE CLI measurement and reporting methods specified by RAN1 can be applied for both application scenarios, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

With the above information from RAN1, we think RAN4 can consider timing error to be limited to intra-cell scenarios and not consider cell-phase synchronization error between gNBs. However, it is likely that in future, 3GPP may specify SRS resource set configurations information exchange among gNBs to enable SBFD operation for inter-cell scenarios as well. With that, RAN4 may need to revisit these requirements and update the scheduling restrictions. In order to save the time and effort, we are also okay with specifying the scheduling restrictions considering the worst case scenario, i.e., inter-cell deployment.

Observation 1: RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement).

Observation 2: For inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs but specification impact of this in Xn/F1 has been excluded in the WID.

Observation 3: It is likely that 3GPP may, in future, specify the SRS resource set configurations information exchange among gNBs.

Next, let’s look at the exact number of symbols for scheduling restrictions.

RAN4 has agreed that when the UE measures L1-SRS-RSRP, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

| FR | Tc | NTA-Offset | Tc* NTA-Offset | Margin (Intra-cell) | Terror_SRS_RSRP (Intra-cell) | Margin (Inter-cell) | Terror_SRS_RSRP (Inter-cell) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 0.51 | 25600 | 13.0304 | 1.67 | 14.7004 | 4.67 | 17.7004 |
| FR1 | 0.51 | 39936 | 20.327424 | 1.67 | 21.997424 | 4.67 | 24.997424 |
| FR2 | 0.51 | 13792 | 7.020128 | 0.67 | 7.690128 | 3.67 | 10.690128 |

| SCS | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| Symbol duration | 71.35 | 35.68 | 17.84 | 8.92 |
| Scheduling restriction (Inter cell) | 1 symbol for FR1 | 1 symbol for FR1 | 2 symbols for FR1 1 symbol for FR2 | 2 symbols for FR2 |
| Scheduling restriction (Intra cell) | 1 symbol for FR1 | 1 symbol for FR1 | 2 symbols for FR1 1 symbol for FR2 | 1 symbol for FR2 |

As analysed above, the scheduling restriction requirements remain unchanged for FR1 irrespective of whether intra-cell or inter-cell deployment scenario is considered. However, for FR2, the scheduling restriction for 120kHz SCS may be reduced from 2 symbols to 1 symbol, but this is cutting too close to the symbol duration. So we recommend using existing requirements for scheduling restrictions.

Proposal 1: RAN4 to reuse R16 scheduling restrictions requirements for L1-SRS-RSRP measurements irrespective of the deployment scenario, i.e., inter-cell or intra-cell for both FR1 and FR2.

For L1-CLI-RSSI measurements, since RAN4 agreed to use DL timing, we only need scheduling restrictions on the OFDM symbols to be measured and no scheduling restrictions on the symbols before and after the

Proposal 2: RAN4 to define scheduling restrictions only on the OFDM symbols to be measured for L1-CLI-RSSI measurements, i.e., no restrictions on the symbols before and after the measurement symbols.

Furthermore, R16 CLI requirements also considered UE capabilities to perform DL reception and CLI measurements in FDM manner. We think similar capabilities can also be used for L1 CLI measurements.

Proposal 3: RAN4 to discuss whether to define UE capabilities (e.g., cli-SRS-RSRP-FDM_DL and cli-RSSI-FDM-DL) for FDM DL and L1 CLI measurements.

Measurement Restrictions

RAN4 made the following agreement on measurement restrictions during the last meeting:

- Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

  - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

While the above agreement addresses the measurement restrictions when the L1 CLI measurement resources overlap with other measurement resources configured for L1 or L3 measurements, the case of overlapping L1 CLI measurement resources is not clear. Based on the recent agreement from RAN1, we may have the following cases of L1 CLI measurement collisions:

- Case 1: Method#1 and Method#2

  - L1-CLI-RSSI resources configured in DL sub-band collide with L1-SRS-RSRP measurement resources configured in UL sub-band

- Case 2: Method#2 and Method#3

  - L1-SRS-RSRP resources configured in UL sub-band collide with L1-CLI-RSSI measurement resources configured in UL sub-band

- Case 3: Method#1 and Method#3  Invalid case based on RAN1 agreement

  - L1-CLI-RSSI resources configured in DL sub-band collide with L1-CLI-RSSI measurement resources configured in UL sub-band

We think Case 1 & 2 are valid cases and may be discussed in RAN4 while Case 3 is invalid based on RAN1 agreement:

A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Observation 4: L1-CLI-RSSI resources (configured according to Method#1 or Method#3) may collide with L1-SRS-RSRP resources configured for L1 CLI measurements

Proposal 4: RAN4 to discuss how to handle the collisions of L1-CLI-RSSI resources with L1-SRS-RSRP resources configured for L1 CLI measurements, i.e., which one of the two should the UE prioritize?

Measurement requirements

For measurement capability, similar to R16 CLI requirements, the L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined within the DL active BWP of the UE, so that the UE is not required to perform any RF retuning for such measurements.

Proposal 5: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Moving on to the side conditions and corresponding measurement delays and accuracy requirements, we think the R16 SRS configurations can be assumed for SBFD as well, but the side conditions on SRS Es/Iot need to be re-evaluated. Based on the RAN1 reply LS, since these measurements are intra-cell only, RAN4 should consider higher values for the SRS Es/Iot values as compared to 1db value specified for R16 SRS-RSRP measurements. The reason is that low SNR of the SRS will cause very little leakage to the DL sub-bands and hence may not cause any interference to the DL.

Proposal 6: Reuse R16 assumptions on SRS configuration for L1 CLI measurements.

Proposal 7: RAN4 to consider higher side conditions on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (>1db). Exact value FFS.

For measurement delays, RAN4 made the following agreement for L1-SRS-RSRP during the last meeting:

- For L1-SRS-RSRP, further discuss the following options based on simulation results:

  - Option 1: requirement specified with [1] sample measurement assumption

  - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

We are okay with both the options, and in general support at-least one set of measurement delay requirement based on a single sample. This can be achieved by re-using the existing measurement accuracy but evaluated at a higher SRS Es/Iot which is what we proposed above. Another reason to specify requirements based on a single sample is that it reduces the SRS transmission overhead on the interfering UE.

Proposal 8: For L1-SRS-RSRP, the baseline measurement delay requirements should be specified with 1 sample. Requirements based on multiple samples (e.g., 3 samples) can be further considered according to NW configuration, e.g. timeRestrictionForChannelMeasurement.

# Conclusion

Observation 1: RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement).

Observation 2: For inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs but specification impact of this in Xn/F1 has been excluded in the WID.

Observation 3: It is likely that 3GPP may, in future, specify the SRS resource set configurations information exchange among gNBs.

Proposal 1: RAN4 to reuse R16 scheduling restrictions requirements for L1-SRS-RSRP measurements irrespective of the deployment scenario, i.e., inter-cell or intra-cell for both FR1 and FR2.

Proposal 2: RAN4 to define scheduling restrictions only on the OFDM symbols to be measured for L1-CLI-RSSI measurements, i.e., no restrictions on the symbols before and after the measurement symbols.

Proposal 3: RAN4 to discuss whether to define UE capabilities (e.g., cli-SRS-RSRP-FDM_DL and cli-RSSI-FDM-DL) for FDM DL and L1 CLI measurements.

Observation 4: L1-CLI-RSSI resources (configured according to Method#1 or Method#3) may collide with L1-SRS-RSRP resources configured for L1 CLI measurements

Proposal 4: RAN4 to discuss how to handle the collisions of L1-CLI-RSSI resources with L1-SRS-RSRP resources configured for L1 CLI measurements, i.e., which one of the two should the UE prioritize?

Proposal 5: The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Proposal 6: Reuse R16 assumptions on SRS configuration for L1 CLI measurements.

Proposal 7: RAN4 to consider higher side conditions on SRS Es/Iot for L1-SRS-RSRP measurements as compared to R16 L3 SRS-RSRP measurements (>1db). Exact value FFS.

Proposal 8: For L1-SRS-RSRP, the baseline measurement delay requirements should be specified with 1 sample. Requirements based on multiple samples (e.g., 3 samples) can be further considered according to NW configuration, e.g. timeRestrictionForChannelMeasurement.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104

[2] R4-2413900, “WF on SBFD RRM”, Huawei, RAN4#112

[3] R4-2416858, “WF on SBFD RRM”, Huawei, RAN4#112bis

[4] R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, RAN4#113

[5] R1-2410890, “Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation”, Huawei, RAN1#119