3GPP TSG-RAN WG4 Meeting #114                                   	R4-2502232

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:			7.11.1 General aspects

Source:	Eutelsat Group

Title:	Addition of structure for support for mobile VSAT served by NGSO

Document for:	Discussion

# Introduction

Following the decision in [1] to define support for mobile VSAT in the Ku bands WI, this paper discusses how to introduce the mobile VSAT in the VSAT definitions.

# Discussion

The current definition of VSAT types is given in 38.101-5 as:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

Due to currently open regulatory aspects, the addition of mobile VSAT served by NGSO is currently limited to the Ku bands. This somewhat complicates the way in which this additional capability is specified.

The scope of the changes required for mobile VSAT is small and essentially limited to UL timing requirements as discussed in [2] and [3].

It does not seem essential to define mobile VSAT served by NGSO for a mechanically steered antenna although this paper assumes this will be done.

Modifying types 4 and 5 to add LEO (NGSO) with limit to Ku bands would be:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: Support for LEO is only defined for Ku bands (TBD) |  |  |

An alternative is to add new types, restricted to Ku bands, which could in the future be extended to Ka bands. This approach however would require two new types one for mechanically steered antenna and the other for electrically steered antenna.

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
|  | 63 | Mobile VSAT communicating with GSO and LEO with mechanically steering antenna. |
|  | 73 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: Support for LEO is only defined for Ku bands (TBD) |  |  |

Given the very small difference in the specifications between Fixed VSAT served by NGSO and Mobile VSAT served by NGSO which require no changes to the UE, it is proposed to modify types 4 and 5, restricted to Ku bands.

# Conclusions

Proposal 1: Add support for mobile VSAT served by NGSO by modifying types 4 and 5, restricted to Ku bands.

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD) |  |  |

# References

[1] RP-243314, “Revised Ku band WID” (Eutelsat Group) RAN #106 Dec 2024

[2] R4-2502205, “UL Timing modifications to enable mobile VSAT served by NGSO” (Eutelsat Group) RAN5=4 #114 Feb 2025

[3] R4-2502207, “CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO by NGSO” (Eutelsat Group) RAN5=4 #114 Feb 2025