3GPP TSG RAN WG4 Meeting #114		R4-2502238

Athens, Europe, February 17th-21st, 2024

Agenda item:	7.14.3.1

Source:	Telecom Italia S.p.A., Vodafone, Deutsche Telekom, Orange

Title:	Proposals for Rel-19 TRP TRS requirements topic

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#113 meeting, discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution provides some proposals related to some of the issues captured in the WF.

# 2	1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

Agreements:

  - RAN4 further discuss how to specify requirements for Size 2 (narrow) UE.

  - Further align the understanding on target of preliminary measurement campaign.

    - Option 1: for defining the offset value, on top of size 1 requirements

    - Option 2: for collecting data to generate CDF and define requirements based on CDF ( this is traditional measurement campaign, the difference is only less number of devices required)

The following observations can be made:

Observation 1: Size 1 and Size 2 handheld devices are based on completely different design so that it is not possible to define a numerical approach (i.e., offset or more complex formula) to corelate the related performances.

Observation 2: The performances are dependent on the considered bands, meaning that for each one a specific offset should be defined.

Observation 3: The measurements performed on two Size 2 devices are not sufficient to determine any offset with respect to the minimum performance requirements defined for the Size 1 devices being based on a pool of 80 samples.

Based on the above observations, it is proposed the following:

Proposal 1: Adopt Option 2 and specify the minimum performance requirements for Size 2 devices based on a complete measurement campaign.

Proposal 2: The number of minimum devices to be measured (and related thresholds) is FFS and depends on the available number of such device currently present on the market.

Issue 4-2-5: Working procedure for 1Tx requirements related performance measurement campaign

Agreements:

  - On top of agreed working procedure in WF R4-2416590, the following aspect is added:

    - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Further discuss on the preferrable measurement grid methods to be used for the performance measurement campaign.

The following observations can be made:

Observation 4: Regarding the testing lab procedures for 1Tx measurements, the results provided for Rel-18 activity were based on the legacy grid providing in high reliability of the measured values.

Observation 5: Same level of confidence and alignment with the Rel-18 should be guaranteed for Rel-19.

Based on the above:

Proposal 3:

- Legacy grid shall be used for TRP TRS measurement

Issue 4-2-7: How to specify 1Tx requirements in Rel-19

Agreements:

  - Further discuss whether a proper methodology (e.g., threshold) is needed to identify and exclude outlier results of TRP TRS data pool.

Proposal 4: For TRP, a measurement result is identified as outlier if it is X dBm lower than the average TRP value. For TRS, a measurement result is identified as outlier if it is Y dBm higher than the average TRS value. X and Y values are FFS.

# 3	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Observation 1: Size 1 and Size 2 handheld devices are based on completely different design so that it is not possible to define a numerical approach (i.e., offset or more complex formula) to corelate the related performances.

Observation 2: The performances are dependent on the considered bands, meaning that for each one a specific offset should be defined.

Observation 3: The measurements performed on two Size 2 devices are not sufficient to determine any offset with respect to the minimum performance requirements defined for the Size 1 devices being based on a pool of 80 samples.

Proposal 1: Adopt Option 2 and specify the minimum performance requirements for Size 2 devices based on a complete measurement campaign.

Proposal 2: The number of minimum devices to be measured (and related thresholds) is FFS and depends on the available number of such device currently present on the market.

Observation 4: Regarding the testing lab procedures for 1Tx measurements, the results provided for Rel-18 activity were based on the legacy grid providing in high reliability of the measured values.

Observation 5: Same level of confidence and alignment with the Rel-18 should be guaranteed for Rel-19.

Proposal 3:

- Legacy grid shall be used for TRP TRS measurement

Proposal 4: For TRP, a measurement result is identified as outlier if it is X dB lower than the average TRP value. For TRS, a measurement result is identified as outlier if it is Y dB higher than the average TRS value. X and Y values are FFS.

# 4	References

- RP-243277, “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, December 2024

- R4-2419808, “WF for [113][325] TRP_TRS_Ph3”, vivo, November 2024