3GPP TSG-RAN4 Meeting #114	R4-2502250

# Athens, Greece, 17th – 21st February, 2025

Title:	Cross Link Interference (CLI) handling impact on MPR for SBFD operation

Source:	Qualcomm Incorporated

Agenda item:	7.23.1

WI code:	NR_duplex_evo-Core

Release:	Rel-19

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

In the RAN plenary #104 meeting, RAN-P approved some schemes for CLI handling under the WI of evolution of NR duplex operation [1]. One of the schemes for handing inter-gNB CLI is based on UL resource muting of the PUSCH for both CP-OFDM and DFT-s-OFDM waveform. In RAN1 #119 meeting, an LS was sent out to RAN4 to check the impact on transmit signal quality/MPR requirement due to UL resource muting [2].

RAN1 has made the following agreements on UL resource muting:

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

# 2. 	Discussion on PUSCH with RE muting

In this section, we describe how UL resource muting (RM) is applied to PUSCH waveform. The muting pattern in frequency domain is based on comb-2 structure, where every other RE is reserved and not used for PUSCH resource mapping. The UL-RM is applied only to two OFDM symbol within the PUSCH.

Figure 0-1: UL resource muting for PUSCH

In CP-OFDM, the muting is applied in frequency domain (pre IFFT) in the muted PUSCH OFDM symbols. Since, half of the REs are muted, the amplitude of the remaining REs are multiplied by sqrt (2), to keep the same power. This will lead to higher PSD in the muted symbols that may affect emissions which could in turn impact MPR

Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR.

For DFT-s-OFDM waveform, the muting can be implemented by using one of the followings methods.

- Method #1: The DFT size on the muted symbol is half compared to the non-muted OFDM symbols. Then, the complex valued symbols after DFT are oversampled by factor of 2 (i.e. mapped to every other RE) at the input of the IFFT.

- Method #2: The DFT size is the same, however, the input samples are repeated to generate a comb-2 RE muting post DFT.

From the recommendation above of resource muting, it can be observed that in time domain muting happens only for two symbols and not for the entire slot. This raises the question of how should the emissions be calculated now for SBFD operation i.e still at a slot level or a symbol level?

Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  might need to be revisited.

# 3. 	Analysis

Based on the waveform description in section 2, simulation is performed for a power class 2 (PC2) transmitter with following assumptions.

Waveform : DFT-s-OFDM QPSK

Channel BW = 100 MHz with SCS = 30 kHz

UL # RBs = 1 with Start Index = 0

For the generated waveforms, linearity metrics namely ACLR, IBE, EVM are analyzed.

For a waveform starting at the edge of channel bandwidth which is limited by SEM, it can be seen that there is no loss of power for resource muted waveform.

# Conclusion

We discussed an approach to construct a waveform to combat CLI in SBFD operation with resource muting and its impact on UE Tx performance.

Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR.

Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  might need to be revisited.

Observation 3: Resource muting has minimal impact on the transmit waveform quality.

# Reference

- 3GPP TSG RAN Meeting #104, RP-241614 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”

- R1-2410889, LS on impact on transmit signal quality/MPR requirement