3GPP TSG-RAN WG4 Meeting RAN4#114      				R4-2502254 Athens, Greece

April 17-21, 2025

Agenda Item:	7.1.1.2.1

Source:	Lenovo

Title:	A-MPR Handling with Bandwidth Extension

Document for:  Approval

- Introduction

In RAN4#113, there were proposals for the handling of A-MPR with the use of the extended channel bandwidth. During the meeting, it was noted that there could be an issue with the proposed approach in that A-MPR tables may not include values for RB allocations in cases in which the A-MPR value is less than or equal to the MPR value for the same allocation. This contribution addresses this issue.

- A-MPR with Bandwidth Extension

In RAN4#113, a method was proposed for handling the case in which bandwidth extension is used and additional emissions constraints were applied. Specifically, in [1] it was proposed that in cases where additional emissions requirements should be met, the UE performs the max(MPR,A-MPR) with A-MPR based on the UE channel bandwidth and MPR based on the extended channel bandwidth as in the following:

Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function with:

- A-MPR based on the LCRB and RBstart and frequency based on the UE CBW (NRB and RBstart parameters)

- MPR based on the extended UE BW and position (BS_NRB and RB0shift parameters).

It was subsequently noted that in some cases the A-MPR tables may not include values for RB allocations for which the A-MPR is less than or equal to the MPR for the same allocation. As a result, in the way forward [2], the proposal was modified as the following:

In cases where additional emission should be met, the UE performs the max(MPRenh, MPR, A-MPR) function with:

- MPRenh here means the approach converting outer allocation to inner allocation i.e. the method that is discussed in this WI

- Companies are encouraged to propose further refinement

In examining the proposal in the way forward, can be observed that for a given RB allocation X, it is always true that

MPRenh(X) ≤ MPR(X) .

From this, we have that

max(MPRenh(X), MPR(X), A-MPR(X)) = max(MPR(X), A-MPR(X))

and thus it can be observed that in the case with additional emissions constraints, the benefit of the enhanced channel bandwidth would be lost.

There are at least two alternative ways to address this case where additional emissions requirements apply.

Alternative 1:

If A-MPR is specified for a given RB allocation X, then the UE performs

max(MPRenh(X), A-MPR(X)) .

If A-MPR is not specified for a given RB allocation X, then the UE performs

max(MPR(X), A-MPR(X)) .

Alternative 2:

For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which

A-MPR(X)≥minX∈SMPR(X)

where S is the set of all possible RB allocations for the given carrier, then	 the UE performs

max(MPRenh(X), A-MPR(X)) .

It can be noted that minX∈SMPR(X) depends on both the transmission mode and the modulation type.  For example, for single antenna transmission of CP-OFDM, minX∈SMPR(X) is 1.5 dB, 2 dB, 3.5 dB, and 6.5 dB for QPSK, 16 QAM, 64 QAM, and 256 QAM, respectively.

- Summary

In RAN4#113, methods were proposed for handling the case in which bandwidth extension is used and additional emissions constraints were applied. It was subsequently noted that in some cases the A-MPR tables may not include values for RB allocations for which the A-MPR is less than or equal to the MPR for the same allocation.  In this contribution, we propose the following alternative methods:

Alternative 1:

If A-MPR is specified for a given RB allocation X, then the UE performs

max(MPRenh(X), A-MPR(X)) .

If A-MPR is not specified for a given RB allocation, then the UE performs

max(MPR(X), A-MPR(X)) .

Alternative 2:

For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which

A-MPR(X)≥minX∈SMPR(X)

where S is the set of all possible RB allocations for the given carrier, then	 the UE performs

max(MPRenh(X), A-MPR(X)) .

# References

- R4-2418015,”Redefining allocation types for improved MPR with BS BW larger than UE CBW,” Skyworks Solutions Inc., RAN4#113

- R4-2420527, “WF on Power domain enhancement,” Huawei, HiSilicon, RAN4#113.