3GPP TSG-RAN4 Meeting #114	R4-2502255

# Athens, GR, 17th – 21st February, 2025

Source:	LG Electronics

Title:	UE RF requirements and UE capability of LB carrier aggregation

Agenda item:	7.8.2

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirement for low band CA [1] which was approved in RAN#106.

Discussion

Regarding the following objectives of approved LB CA WID [1], we would like to discuss UE RF requirements.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

Figure 2-1 shows the switching between {case1, case 2} with example band combination : CA_5A-n29A.

Figure 2-1: switching between {case1 , case 2} : CA_5A-n29A

As seen in Figure 2-1, carrier switching from Tx/Rx in n5 to Rx in n29 needs to be considered.

Existing UL switching in NR inter-band CA.

- 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

  - 6.3A.3.3.1	General

  - 6.3A.3.3.2	Time mask for switching between two uplink carriers

  - 6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

  - 6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

  - 6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors

  - 6.3A.3.3.6	Time mask for switching across up to four uplink bands

  - 6.3A.3.3.6a	Additional requirements for three-band switching with dual TAG

Compared with the exiting UL switching in NR inter-band CA, the carrier switching between {case1, case2} seems be new switching.

According to the objectives, RAN4 is supposed to specify the switching delay and time mask for carrier switching and define the corresponding UE capability. Therefore, it is necessary to discuss whether to reuse the existing requirement, UE capability and RRC signalling for the carrier switching. Then, RAN4 have to specify the corresponding requirements including time mask, UE capability and NW information for carrier switching.

In the existing uplink switching for inter-band CA, the following UE capability, RRC signalling, and time mask requirement can be considered as starting point for carrier switching.

- UE capability

  - ULTxSwitchingBandPair

    - bandIndexUL1, bandIndexUL2

    - uplinkTxSwitchingPeriod

    - uplinkTxSwithing-DL-Interruption

- RRC signaling

  - UplinkTxSwitching

    - uplinkTxSwitchingPeriodLocation

    - uplinkTxSwitchingCarrier

- Time mask requirement, e.g., 6.3A.3.3.2

Proposal 1: Consider whether to define new UE capability of carrier switching for LB CA with SDL.

Proposal 2: Consider whether to define switching period and switching location of carrier switching for LB CA with SDL.

Proposal 3: Consider whether to define time mask requirement of carrier switching for LB CA with SDL.

Conclusion

In this contribution, we proposed our view on UE RF requirement for low band CA.

Proposal 1: Consider whether to define new UE capability of carrier switching for LB CA with SDL.

Proposal 2: Consider whether to define switching period and switching location of carrier switching for LB CA with SDL.

Proposal 3: Consider whether to define time mask requirement of carrier switching for LB CA with SDL.

- References

[1] RP-243315, “New WID on low band carrier aggregation via switching”, Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE