3GPP TSG-RAN WG4 Meeting # 114	R4-2502259

# Athens, GR, 17th – 21th February, 2025

Source:	LG Electronics

Title:	Discussion on Power domain enhancements for single carrier

Agenda item:	7.1.1.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #113, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

- Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation Agreement in main session: - Extended CBW based approach - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE - Both scenario 1 and 2 should be covered - The network should ensure the co-existence be met when signaling the extended CBW. - Issue 1-1-2: Approaches of calculating the new inner RB region - Options - Option 1: The NW signals the extended number of RBs and RB shift towards the UE configured CBW or RRC signalling to indicate the extended RB number at each side of the UE CBW for the UE to calculate the new inner, outer, and edge regions - The extended number of RBs may not necessarily be the same as those in the BS CBWs - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 2: Based on the fixed extension ratio, it is up to the NW to judge the gap width on both sides of the UE and grant the permission to UE using the reduced MPR for the new inner region - The extension ratio on both sides, 0.5* UE CBW should be stipulated in the spec - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - Option 3: For a UE supporting [capability] and configured with a UE channel bandwidth within a carrier with a bandwidth greater than the transmission bandwidth configuration of this UE channel bandwidth, NRB = N´RB is the said carrier bandwidth as indicated by carrierBandwidth in [SIB1] and the condition for an inner RB allocation is modified accordingly - The intersection of the Inner region relative to the NW CBWs and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE Examples of new inner region just for illustration purpose: Example 1: single CC scenario; Example 2: multi-CC scenario. - WF - FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec - Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: The extended RBs on one side should be equal to NRB/2, where NRB is the maximum transmission bandwidth configuration of the original UE CBW - Option 2: The extended RB number for each side can be different and the value could vary from zero to the one specified in Table 5.3.2-1 in TS 38.101-1, but it is no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz. - Option 3: Width of Extended UE channel bandwidth can be up to BS channel bandwidth but no more or it can be narrower than BS CBW - Other options are not precluded - WF - Extension on each side of UE CBW could be different - FFS on extended RB numbers/channel bandwidth, which are up to X on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios - FFS whether extended RB number/channel bandwidth should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW - Fixed extended RB number/channel bandwidth to X is not precluded. - Extension is not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond X on each side does not further reduce the MPR. - FFS on X value - Sub-topic 1-2: Applicable requirements - Issue 1-2-1: Where to use extended IBE in the larger BS CBW - Proposals - Proposal 1: IBE should be used between edges of UE CBW and extended UE CBW. (vivo, ZTE, Huawei, Qualcomm, China Telecom, Skyworks) - Alt 1: extension of the IBE is NOT used in the guard band of UE CBW. - Alt 2: extension of the IBE is also used in the guard band of UE CBW. - Alt 3: extension of the IBE applies in the guard band of the UE CBW but not in the guard band of the extended UE CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS the application of IBE in guard band of UE CBW and/or extended UE CBW - Issue 1-2-2: How to derive the extended IBE - Proposals - Proposal 1: (vivo) - For IBE of both original CBW and extended CBW, NRB should be based on the original UE CBW, ΔRB should also be limited within the original UE CBW. (vivo, ZTE, Huawei) - The IBE requirement of the extension portion should be equal to that of the last non-allocated RB in the original UE CBW. (vivo, ZTE, Huawei) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified to: IBE_general = max⁡(PRB - 30 dB, max⁡{-25, 20*log10EVM -3 +5LCRB, -57dBm + 10log10(SCS/15kHz)- PRB}) - Proposal 2: (ZTE) - Use the IBE limit of -25 dB between edges of UE CBW and extended UE CBW for full RB allocation scenario - Proposal 3: (Huawei) - For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1). The equation for IBE derivation is: max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB - Proposal 4: (China Telecom) - For full RB allocation, if the extended UE CBW exists, the IBE should be applied. - Proposal 5: (Qualcomm) - The IBE between the edge of the UE BW and the edge of the extended UE BW should be an extension of the IBE for the signal within the UE BW. - WF - FFS the method to derive the extended IBE, which should cover both cases for partial RB allocation and full RB allocation in the UE CBW - Issue 1-2-3: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW. (Nokia, Qualcomm, vivo, ZTE, LGE, OPPO, Huawei, China Telecom) - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW Alt 1. Alt 2. - WF - ACLR and SEM should be applicable from the edge of extended UE CBW. - FFS the following alternatives - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW - Issue 1-2-4: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. (vivo, LGE) - Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz. (Nokia) - Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW (Qualcomm, OPPO) - WF - FFS with consideration of Issue 1-2-3 - Sub-topic 1-3: Signaling and others - Issue 1-3-2: Others - Proposals - Proposal 1: (Skyworks) - Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function with: - A-MPR based on the LCRB and RBstart and frequency based on the UE CBW (NRB and RBstart parameters) - MPR based on the extended UE BW and position (BS_NRB and RB0shift parameters). - WF - In cases where additional emission should be met, the UE performs the max(MPRenh, MPR, A-MPR) function with: - MPRenh here means the approach converting outer allocation to inner allocation i.e. the method that is discussed in this WI - Companies are encouraged to propose further refinement

Discussion

Approaches of calculating the new inner RB region

- FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec

Network indication method can be one of simple method without additional simulation and used to cover outer RB allocation to inner RB allocation while including the above aspects in WF. So far as we have discussed with extended UE CHBW. In this discussion, unified solution converting outer RB allocation to inner RB allocation is coming. Because MPR value in the spec was made under the condiction that there is adjacent allocated channel. At this aspect, if network indicate some information (e.g. extended UE CHBW and RBoffset) to UE. UE can calculate the new MPR value with converting outer RB allocation to inner RB allocation.

Figure1. Extension UE CHBW

Figure2. Operation based on BS indication

If the RB allocation region is divided based on the new NRB_new as above, the MPR value can be calculated with the inner region expanded than the previous one. NRB_new is calculated as follow NRB_new=RBoffset_left+ RBoffset_right . Then, if the network indicates the NRB_newand RBoffset_left values to the UE, the UE can apply the MPR value of the new region based on the expanded UE CHBW with improved values. The above method can reduces the MPR by the number of unallocated RBs next to the UE CHBW including many aspect in WF. The outer RB allocation can be modified to an inner RB allocation if the Network indicate NRB_newand RBoffset_leftto the UE.

We consider network indication when converting outer RB allocation to inner allocation as below

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Proposal 1: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Proposal 2: Description of deriving the RB allocation region inside the UE CBW

- The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW

Width/RB numbers of extended CBW at each side of UE CBW

Actually,when half of the UE CHBW on each side was added, the existing outer RB allocation of UE CHBW totally comes into the inner. However, there is no harm when the extension is extended beyond that. So. It is possible to freely decide the width of extended UE CBW on the network

Proposal 3: Width of Extended UE CBW can be freely decided on the network .

Boundary to apply ACLR and SEM

if integral region and boundary of OOBE is based on extended UE CBW, spurious emission will not satisfy the ITU-R radio regulations. so The integral region and the boundary of OOBE should be based on UE CBW.

Proposal 4: The OOBE requirements is necessary based on UE CBW

- 2. Conclusion

In this contribution, we want to share some views on Power domain enhancements for single carrier and proposals are made as following:

Proposal 1: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset_left  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Proposal 2: Description of deriving the RB allocation region inside the UE CBW

- The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW

Proposal 3: Width of Extended UE CBW can be freely decided on the network .

Proposal 4: The OOBE requirements is necessary based on UE CBW

- 3. Reference

[1] R4-2420527, “WF on Power domain enhancements”