# 3GPP TSG-RAN WG4 Meeting #114	R4-2502278

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:			7.10.4

Source:	Samsung

Title:	Way Forward for [114][325] NTN_testing_NGSO_channel_model

Document for:	Approval

Introduction

This t-doc provides WF for [114][325] NTN_testing_NGSO_channel_model.

WF

Sub-topic 1-1 TE emulated channel model for satellite mobility

Issue 1-1-1 Satellite position/velocity estimation based on equation of motion

- WF

- RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102

  - Note: the details models for Kepler’s equation and equation of motion can be referred as in Appendix

- RAN4 will introduce the accuracy requirement applied for two models compared with GMAT

- Companies are encouraged to provide the calibration and comparison results with two models before RAN4#114bis meeting, based on calibration and comparison results, further discuss how to define the related accuracy requirement in the next meeting.

- FFS how to capture the TE-emulated time varying doppler shift and propagation delay model in to specification

  - Option 1: capture into the normative part of Appendix, details need to be further discussed

Issue 1-1-2 Correction of Propagation Delay Calculation

- WF

- RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay

- Considering the earth rotation, the propagation delay, DelayERtR can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR,  K  is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b

- The proponent is encouraged to provide the detail information, e.g., reference, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting.

Issue 1-1-4 Assumption of TE emulated channel model

Issue 1-1-4-1 Accuracy requirement to solve Kepler’s equation based on New-Raphson method

- Proposals

- Option 1: E-E0> ϵ=10-8

- Option 2: Considering the trade-off of complexity and accuracy

  - ϵ=10-7

  - TE vendors are encouraged to provide the input about the implementation feasibility

Issue 1-1-4-2 Number of iterations to solve Kepler’s equation based on New-Raphson method

- Proposals

- Option 1: 4

- Option 2: Do not consider the number of iterations as a stopping criterion for Newton-Raphson

Issue 1-1-4-3 Interpolation method to obtain the propagation delay and Doppler shift trajectory

- Proposals

- Option 1: Cubic interpolation

- Option 2: linear interpolation

- Option 3: Open to not restrict the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation

- Option 4: With the assumption of UE updates the satellite location/velocity at least every 1 second. The TE may update Doppler Shift and Propagation delay every slot linear interpolation or cubic spline interpolation. It is update to TE implementation which interpolation method is used.

Issue 1-1-4-4 Accuracy requirement of maximum delay errors to obtain the propagation delay and Doppler shift trajectory

- Proposals

- Option 1

  - Max |delay-error|~5e-10 s

  - Max |delay-rate error|~1e-10 s/s

- Option 2

  - RAN4 to define a simulation assumption/TE requirement on interpolation accuracy when quantifying the NGSO demodulation for dynamic conditions so all test equipment can be provided equivalent results

- Option 3

  - Do not consider accuracy requirements for the estimation of propagation delay and Doppler Shift

- Option 4

  - For the accuracy requirement for the delay-error and delay-rate error if interpolation method is used, Max |delay-error| < 3e-13 s, Max |delay-rate error| < 3.28e-12 should be fulfilled

Issue 1-1-4-5 Overall procedure of update timing of the satellite position/velocity, ephemerisInfo, Doppler shift and propagation delay

- WF

- Agreed the general procedure of TE emulated channel model including Satellite position and velocity estimation, as well as Doppler shift and propagation delay generation.

  - Tsib=10.24 (sec) for RF and demodulation test and Tsat < Tsib

  - Tsib=2.56 (sec) for RRM test, and Tsat < Tsib

Issue 1-1-4-6 Update granularity for estimated satellite location/velocity

- Proposals

- Option 1: Adjust the interval from 1 second to 1.024 seconds

- Option 2: TE should update the satellite location/velocity at least every 1 second

- WF

- TE should update the satellite location/velocity at least every 1.024 second

Issue 1-1-4-7 Update granularity for the estimation of propagation delay and Doppler shift

- Proposals

- Option 1: update once per 1ms

- Option 2: updated once per slot in FR1,

- Option 3: RAN4 not to relay on a slot granularity for the definition of the channel model representing the NGSO propagation model.

- WF

  - Option 2: updated once per 1ms

  - Note: updated timing is not necessary to be synchronized with slot boundary

Issue 1-1-4-8 Assumption of Initial ephemeris information at time 0

- Proposals

- Option 1:

  - Considering NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point for initial ephemeris information at time 0 to specify the TE-emulated Doppler shift and propagation delay model. Pending on the test purpose, the existing condition can be updated during individual test.

- Option 2:

  - Decide initial ephemeris information based on the example files provided in R4-2413333 and the outputs in the discussion of elevation angle range.

- Option 3

  - To select appropriate initial ephemeris which is specified in the test specification to ensure the initial elevation is within the appropriate range.

Issue 1-1-4-8 Assumption of Elevation angle range

- Proposals

- Option 1:

  - Considering NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point for initial ephemeris information at time 0 to specify the TE-emulated Doppler shift and propagation delay model. Pending on the test purpose, the existing condition can be updated during individual test.

  - For IoT NTN demodulation test, RAN4 should solve the problem of long test time

- Option 2:

  - For demodulation test, to align with Rel-17 NTN channel model discussion, the elevation angle with [30°~150°] can be considered for testing. Large testing period, i.e., [10°~170°] can be considered for IoT NTN testing. For RF test, [10°~170°] can be considered

- Option 3

  - To consider [30°~150°] for both NR and IoT NTN for demodulation test

- Option 4

  - Consider 30-degree initial elevation angle for both RRM and demod test cases

- WF

- Initial ephemeris information and 	Elevation angle at time 0

  - Derive the initial ephemeris information based on the example files provided in R4-2413333 with associated elevation angle to specify the TE-emulated Doppler shift and propagation delay model 

  - Note: pending on the test purpose, the initial ephemeris with associated elevation angle can be different

  - For RRM and Demod test

    - Initial Evaluation angle: 30°

- Table 1	 Example of initial ephemeris information starting from elevation angles of 30 degrees for NGSO LEO 600

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | [-3451.1273] |
| positionY-r17 | km | [5703.5635] |
| positionZ-r17 | km | [2071.0809] |
| velocityVX-r17 | km/s | [0.87360] |
| velocityVY-r17 | km/s | [-2.06700] |
| velocityVZ-r17 | km/s | [7.21231] |

- FFS whether to define the ephemeris information for NGSO LEO 1200

  - For RF test

    - Evaluation angle: FFS

    - Initial ephemeris information: FFS

- FFS on how to solve the problem of long testing time for IoT NTN

  - Option 1: Define the test applicability rule for the tests which are applied for TE-emulated time varying channel model testing

  - Option 2: The case under testing within the visible range of satellite

Sub-topic 1-2 RAN4 requirements

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- Proposals

  - Option 1: Consider Keplerian model for UE assumption on the propagator model

  - Option 2:

    - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

  - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information

  - Option 3a: RAN4 may further discuss how to incorporate the TE emulated channel model for simulation

  - Option 4: A common propagation model for all NTN UEs should be standardized to allow the optimization of the NTN performance

Issue 1-2-2 Impact on RAN4 requirements

- WF

  - Further discuss the impact on RAN4 requirement pending on channel model discussion progress. The impact of RF, RRM and demodulation requirement will be discussed in separate session.

Issue 1-2-3 Impact on RAN4 demodulation requirements

- Proposals

  - Option 1:

    - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

  - Option 2:

    - Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin

  - Option 3

    - Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

- WF

  - Further discuss whether to assume UE of interpolation method to estimate the Doppler shift and Propagation delay

  - Further discuss whether to consider frequency shift error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply specified in TS 38.101-5 and TS 36.102

  - Frequency shift error

    - Option 1: 70Hz

    - Option 2: 200Hz

  - Time error

    - Option 1: 3.5us

    - Option 2: 0.94us

  - Based on simulation results, further discuss whether additional margin to consider the estimation/pre-compensation errors with TE-emulated varying Doppler shift and propagation delay models

Sub-topic 1-3 Inclusion in Specfication

Issue 1-3-1 Specification of channel Model

- WF

  - Option 1: How to capture into specification can be discussed in the CR drafting

  - Option 2:

    - FFS to specify the detail formulas directly into specification for NTN NGSO channel model, including both Keplerian model-based estimation and Equation of Motion based estimation-method  considering the Keplerian definition motion bass method was pubic materials, can be referred the public book or academic paper

    - RAN4 can further discuss whether it is feasible to specify the detail procedure for each step to generate the time varying doppler shift and propagation delay, without explicitly specifying the detail formula, capture the output for each step, and final doppler shift trajectory and propagation delay under agreed NGSO test conditions.

- Reference

[1] R4-2500586, “Topic summary for [114][325] NTN_testing_NGSO_channel_model “, Moderator (Samsung)

- Appendix (for information)

  - A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

A.1.1	Keplerian model based estimation

Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

A.1.2	Equation of Motion based estimation

Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if the satellite position/velocity is updated every 1 second.

Step 2	Solve the equation of motion with the 4th-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

Where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

  - A.2	Varying Doppler shift and propagation delay generation

A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since it is assumed that UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |

ρtECEF=ρt,xECEIρt,yECEIρt,zECEI=rt,xECEF-UEt,xrt,yECEF-UEt,yrt,zECEF-UEt,z

Range(t)=ρtECEI=ρt,xECEI2+ρt,yECEI2+ρt,zECEI2

RangeRate(t)=ρtECEF⋅vtECEFRnage(t)=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFRange(t)

Delay(t)=Range(t)c

Doppler(t)=-RangeRate(t)×fcc

Additionally, the elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0