3GPP TSG-RAN WG4 Meeting #114	R4-2502295
Athens, GR, 17th – 21st February, 2025

Agenda item:	7.14.3.2

Source:	CAICT

Title:	Framework and time plan for Rel-19 MIMO OTA performance requirements development (Feb 2025)

Document for:	Approval

1	Introduction

This contribution provides a framework and time plan for Rel-19 MIMO OTA performance requirements development, based on the revised WID [1], the workplan [2], Rel-17 and Rel-18 MIMO OTA lab alignment outcomes [3, 4], and the latest approved framework [5].

The framework can be further updated and refined according to discussion outcomes in future meetings.

2	Discussion

At the last RAN #106 plenary, the WID for TRP_TRS_MIMO_OTA_Ph3 was revised [1]. The updates include target bands for MIMO OTA performance requirements, as shown below:

4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Objectives for this performance part work item are as follows - Specify TRP and TRS requirements: - Specify FR1 TRP and TRS requirements for NR bands for UE based on operators demand - Handheld UEs with 1Tx at new bands n3, n5, n7, n8, n20 are first priority. n38 and n77 are second priority. - 4 bands are targeted for introducing requirements depending on measurement data availability - Handheld UEs with 2Tx (non-coherent UL MIMO) at n41 and n78 depending on measurement data availability are first priority. Additionally, n77 and n79 are second priority. - Headworn XR OTA requirements will not be considered until development of suitable head phantom and sufficient number of commercial devices become available - Coherent UL MIMO UE OTA requirements will not be considered until sufficient number of commercial devices become available - Size 2 Handheld UEs with 1Tx at legacy bands n1, n28, n41, n78 are first priority - RC harmonization at low band frequency, e.g., n28 Note: Handheld UEs includes Size 1 (wide, width >72mm and ≤92mm) and Size 2 (narrow, width ≥56mm and ≤72mm); - Specify MIMO OTA requirements: - Specify FR1 MIMO OTA requirements for new bands for handheld UE based on operators demand (2 layers or 4 layers) - Band n3, n7, n8, n20, n77, are first priority - 4 bands are targeted for introducing requirements depending on measurement data availability - The requirements are specified based on Rel-17 static MPAC test method

To be aligned with the revised WID, we updated the framework in Section 3.

3	Framework for Rel-19 MIMO OTA performance requirements development (for approval)

3.1 Overall work flow

The overall work flow of Rel-19 MIMO OTA performance requirements development for bands <1GHz (n8/n20) and bands >1GHz (n3/n77/n7) are illustrated in Figs. 1 (a) and (b), respectively. The requirements will be specified based on the Rel-17 static MPAC test method.

(a) for bands <1GHz (n8/n20)                           (b) for bands >1GHz (n3/n77/n7)

Fig. 1.  Work flow of Rel-19 MIMO OTA performance requirements development

In general, Rel-19 MIMO OTA performance requirements will be derived based on a measurement data pool of commercial devices per band. To establish valid and trustable measurement data pools for defining FR1 MIMO OTA performance requirements, the following activities are required.

  - For bands <1GHz (n8/n20)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Labs that submitted channel model validation results for band n8/n20 in Rel-18 (as captured in TR 38.761) do not need to submit the results again.

    - Measurement Campaign: Collect measurement data from Rel-18 aligned labs (Apple, CAICT, CMCC&BUPT, MediaTek, OPPP, Xiaomi) for defining FR1 MIMO OTA requirements for bands < 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

  - For bands >1GHz (n3/n77/n7)

    - Channel Model Validation: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Working procedures of the channel model validation are provided in Section 3.2.1.

      - Note: Rel-17 aligned labs do not need to submit channel model validation results for band n77/n7.

    - Measurement Campaign: Collect measurement data from Rel-17 aligned labs (Apple, CAICT, CMCC&BUPT, Huawei, MediaTek, Xiaomi) for defining FR1 MIMO OTA requirements for bands > 1GHz. Working procedures of the measurement campaign are provided in Section 3.2.2.

Detailed working procedures for Rel-19 MIMO OTA performance requirements development are described in Section 3.2.

3.2 Detailed working procedures

3.2.1 Channel Model Validation

- The purpose of Channel Model Validation is to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone of the MPAC system.

- The channel model validation measurements shall be performed as described in Annex C.3 of TS 38.151, including:

  - Power Delay Profile (PDP)

  - Doppler/Temporal correlation

  - Spatial correlation

  - Cross-polarization

  - Power validation

- Test band: n3, n8; Test frequency: as specified in Tables C.3.1-1 and C.3.1-2 of TS 38.151

- Channel model: as specified in Annex C.1 of TS 38.151

  - For band n8: FR1 UMi CDL-C (2x2 MIMO)

  - For band n3: FR1 UMa CDL-C (4x4 MIMO)

- Pass/fail limits: as defined in Annex C.4 of TS 38.151

3.2.2 Measurement Campaign

- The purpose of Measurement Campaign is to collect measurement results of commercial devices from permitted labs for specifying FR1 MIMO OTA performance requirements.

- Test Methodology: the static FR1 MIMO OTA test method based on Multi-Probe Anechoic Chamber (MPAC) test system. Test plan: 3GPP TS 38.151.

- Test cases for Rel-19 FR1 MIMO OTA Measurement Campaign:

  - Test bands: n3, n7, n8, n20, n77

    - 4 bands are targeted for introducing requirements depending on measurement data availability

  - MIMO layer:

    - For band n8, n20: 2x2 MIMO (with FR1 UMi CDL-C channel model)

    - For bands n77, n3, n7: 4x4 MIMO (with FR1 UMa CDL-C channel model)

  - Use scenario: Free space

  - Operation mode: SA (first stage)

  - Form factor: Smart phone

- Commercial device (Smartphone) selection criteria for FR1 MIMO OTA Measurement Campaign:

  - DUT capability: supporting all the bands listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - DUT variety: the selection of commercial devices should cover various of devices in the market. The following selection criteria can also be considered:

    - Year of production: 2021-2025

    - Brand variety

    - Price range (to cover different price ranges, including High/Mid/Low-end products)

    - Popularity

    - Number of bands supported

  - For band n3, UEs equipped with 4 Rx antenna ports shall be selected for the Measurement Campaign. Labs can try to identify 4Rx or 2Rx UEs in any methods listed below:

    - Method 1: If a UE can be connected to BS simulator with 4x4 MIMO, the UE can be confirmed as a 4Rx UE

    - Method 2: Obtain the MIMO layer information from BS simulator, e.g., check the IE maxNumberMIMO-LayersPDSCH

    - Method 3: Directly collect the information from OEMs.

    - Other methods are not precluded

- Commercial devices provision:

  - Test labs can prepare and collect commercial devices by themselves based on the above selection criteria.

  - Any 3GPP member can work with the selected test labs to provide devices

    - A test lab shall measure only one UE model in case different samples are provided

    - Same UE model supporting different sets of bands can be measured. For this case, the UE model should be marked as different model, e.g., model A-1, model A-2. (guidance on how to manage this case are provided in the spreadsheet in R4-2321191)

    - For band n3, make sure the devices are 4Rx UEs

    - The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects. Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

- Measurement results submission:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: labs use the spreadsheet in R4-2321191 to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

  - Labs use the worksheet template in [TBD, use Rel-18 template R4-2403019 as starting point] to submit the measurement results for Rel-19 3GPP FR1 MIMO OTA performance data pool. Do NOT apply any systematic offset to the measurement results.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3, the maximum number is 15. Meanwhile, labs are encouraged to provide as much data as possible within 15

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to 3GPP RAN4

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB, GCF, NAL/CTA (Chinese Network Access Licensed Test), FCC, CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is not a prototype

- Percentage of the devices that are certified for each certification body (for information only)

3.2.3 Specifying Performance Requirements

- Only measurement data from the aligned labs with channel model validation completed will be considered for specifying requirements, as described in Section 3.1.

- Thresholds for specifying FR1 MIMO OTA performance requirement:

  - Minimum amount of data per band: 15

  - Minimum amount of UE models per band: 12

- Method of requirements derivation: per-band Data driven approach

- The value at [85%] percentile of the CDF curve can be selected as the starting point for requirement discussion

- Performance part of the work will proceed in a contribution-driven manner.

3.3 Time plan

1. Finalize the working procedures and time plan for Rel-19 MIMO OTA requirements development at RAN4 #113 (Nov 2024).

2. Measurement Campaign for bands n77/n8/n7/n20 can start at an early stage with the Rel-17/Rel-18 aligned labs that completed channel model validations for these bands.

3. Labs should submit channel model validation results for bands n3/n8 by formal T-docs no later than RAN4 #115 (May 2025) to participate in the Measurement Campaign at the corresponding bands. Earlier submission is encouraged.

4. To properly plan the Measurement Campaign and preliminarily check measurement data availability per band, the following actions are requested no later than RAN4#115 (May 2025):

  - The rapporteur checks with the volunteer labs the amount of DUTs (minimum 3, maximum 15) they expect to be able to measure per band AND how many DUTs they can accommodate from 3GPP members per band

  - The 3GPP members providing devices for the measurement campaign (if any) check how many samples they intend to provide (in terms of maximum number)

  - Planning of the measurement campaign could be reviewed based on the above points

5. The 3GPP members (if any) should provide the devices to volunteer labs no later than RAN4#116 (Aug 2025).

6. Labs should submit measurement results of commercial devices by formal T-docs no later than RAN4#117 (Nov 2025), also submit the devices information to RAN4 Secretary.

7. Conclude FR1 MIMO OTA performance requirements no later than RAN4#118 (Feb 2026).

Proposal 1: Approve the updated framework in Section 3 of this contribution for Rel-19 MIMO OTA performance requirements development. Further refinement is not precluded.

4	Conclusion

In this contribution, the framework is updated based on the latest revised WID.

Proposal 1: Approve the updated framework in Section 3 of this contribution for Rel-19 MIMO OTA performance requirements development. Further refinement is not precluded.

References

- RP-243277, Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, 3GPP RAN#106, Dec. 2024.

- R4-2412056, Updated Workplan of Rel-19 FR1 OTA WI, vivo, CAICT, 3GPP RAN#112, Aug. 2024.

- R4-2214360, WF on NR MIMO OTA (Rel-17), CAICT, vivo, 3GPP RAN4#104-e, Aug. 2022.

- R4-2406083, Way Forward for [110bis][337] NR_MIMO_OTA_enh, CAICT, 3GPP RAN4#110-bis, Apr. 2024.

- R4-2419813, Updated Framework and time plan for FR1 MIMO OTA performance requirements development (Nov 2024), CAICT, 3GPP RAN4#113, Nov. 2024.