TSG-RAN Working Group 4 (Radio) meeting #114	R4-2502318

Athens, EU, 17th to 21th February 2025

Source:	Ericsson

Title:	TP to TR 38.908: Conformance test aspects for EEIRP

Agenda item:	7.13.4.1

Document for:	Approval

# Introduction

This is a text proposal for Clauses in TP 38.908

-----Start of text proposal-----

# Background

The frequency band of 6425 to 7125 MHz and parts thereof was identified for International Mobile Telecommunications (IMT) use by WRC 23 in different ITU region as in the RR Footnotes 5.457D, 5.457E and 5.457F with associated technical condition of limits on the Expected Equivalent Isotropic Radiated Power (EEIRP) spectral density of IMT base-stations for protecting Earth-to-Space Fixed Satellite Services (FSS) as in the ITU Radio Regulation Resolution 220 (WRC-23). The EEIRP is a new regulatory requirement specified as a mask for over-the-horizon emissions. The Annex to the Resolution 220 (WRC-23) [2] outlines a theoretical calculation of the EEIRP of an IMT base station for assessing the compliance of the IMT base station equipment with the limit on EEIRP.

The aim of this report is the following:

- To capture the technical background information relevant for the core requirement in BS RF core specification .

- To capture the technical background information relevant for the test procedures in the conformance test specification.

Having the requirement included in a 3GPP standard would guarantee a harmonized terminology and conformance test methodologies. Eventually, the concept for conformance testing can be adopted to similar situations involving co-channel spectrum sharing between IMT and FSS UL.

The ITU Radio Regulations were updated at WRC-23 with Resolution 220 that contains this EEIRP technical requirement for a single IMT BS where it calculates the average radiating power for each of the seven elevation bins. Since the protection of FFS UL receiver is based on aggregated power from many base stations oriented in different directions over a large satellite footprint, the technical requirement can be easily broken down to a single BS based on the average EIRP emissions from a single BS, as used during normal operation. WRC-23 decided to adopt a concept where the average EIRP emissions are extracted based on measuring the EIRP emissions for several test beams over the specified elevation bins. The choice of this test beam set should be representative of the BS normal operation.

## 6.3	Test beams

### 6.3.1	General

The BS capability to provide coverage in different deployment situations is declared by the BS manufacturer in the form of a steering range determined by minimum and maximum angles along the  and  axis, as visualised in Figure 6.3-1. It is possible to declare multiple steering ranges for different types of deployment scenarios.

# Figure 6.3-1: Declaration of steering range

Evaluation of Expected EIRP is based on the assumption that maximum carrier power of the BS is utilized together with the narrowest beam supported, generating the maximum EIRP. As the BS antenna array contains a large number of elements the number of narrow beams that can be generated in this steering range is also large.

Ideally, EEIRP would be the result of averaging the contributions of all beams generated by the BS, but such approach is impractical, as it will lead to an excessive testing effort. For this reason, in the following section, we are presenting the results of averaging a smaller number of representative beams, in a effort to determine the test beams set for which overall averaging of the EEIRP is sufficiently accurate (as described in the Annex to the ITU Resolution 220).

### 6.3.2	Test beam set evaluation

The evaluation of the EEIRP estimation accuracy is highlighted as a function of number of test beams considered within the supported steering range.

### 6.3.2.1	Evaluation method

The following method is utilized for determining the optimal set of test beams through simulations:

- We assume a number of antenna array configurations that are supposed to reflect future realistic product implementation. We use the antenna modelling in TR38.803 to create the radiation pattern per test beam. Beams will be generated within declared steering range.

- We define a reference case, consisting of a very large number of test beams, that will completely describe the BS beamforming capability in the declared steering range. Two reference cases are considered, one assuming a rectangular steering range and one assuming an more realistic ellipsoidal steering range. We calculate the reference EEIRP for this reference case according to the EEIRP formula as described in section 6.2.

- We consider for evaluation a number of test beam sets in the range from 9 (3x3) to 25 (5x5) in different constellations. We calculate the EEIRP according to the EEIRP formula as described in section 6.2 for each of these limited sets. Each of the beams in their respective set have equal weight.

- For each test beam set, we calculate the EEIRP difference relative to the reference case and draw conclusions about the accuracy of the EEIRP estimation.

#### 6.3.2.2	Ericsson results

In the evaluation we consider a BS steering range -60 to 60 degrees along azimuth axis and 0 to 16 degrees along vertical axis. The beam sets are compared to a reference beam set defined as 17x121 beam constellation (2057 beams) within the steering range.

In our evaluation we have included 9 different beam constellations (4x3, 4x4, 6x3, 5x4, 4x5, 6x4, 5x5, 5x5 no corners, 17 beams in ellipsoid and 20 random beams), visualised in Figure 6.3.2.2-1, Figure 6.3.2.2-2 and Figure 6.3.2.2-3.

# Figure 6.3.2.2-1: Beam constellation BS1 (left), BS2 (middle) and BS3 (right)

# Figure 6.3.2.2-2: Beam constellation BS4 (left), BS5 (middle) and BS6 (right)

# Figure 6.3.2.2-3: Beam constellation BS7 and BS7nc (left), BS8 (middle) and BS9 (right)

The beam sets have been constructed so that uniform (BS1, BS2, BS5) and non-uniform (BS3, BS4, BS6, BS7, BS8 and BS9) distributions with different number of total beams are included.

To include variations due to array antenna configuration several configurations including different array size and sub-array size were considered, as listed in Table 6.3.2.2-1.

Table 6.3.2.2-1: Array antenna configuration

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 |
| Element | Vertical HPBW (°) | 90 | 90 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | - | - | - | - |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 |
|  | Element power (dBm/100 MHz) | 0 | 0 | 0 | 0 |
| Array configuration | Number of rows | 16 | 32 | 8 | 8 |
|  | Number of columns | 8 | 16 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 4x1 |
|  | Sub-array pre-tilt (°) | - | - | 6 | 6 |
|  | Vertical element separation (λ) | 0.5 | 0.5 | 2.1 | 2.8 |
|  | Horizontal element separation (λ) | 0.5 | 0.5 | 0.5 | 0.5 |

The simulation results are listed in Table 6.3.1.1-1, Table 6.3.1.1-2, Table 6.3.1.1-3 and Table 6.3.1.1-4. The rectangular reference case is considered as a baseline for all configurations. Additionally, for the BS7nc there are two set of values: the first one is for the rectangular reference case and the second one for the elipse reference case.

Table 6.3.2.2-1: Config 1

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS7nc |  | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<<5 | -0,963 | -0,993 | -0,261 | 0,05 | -0,999 | -0,291 | 0,044 | -0,66 | 0,407 | -0,504 | -0,2 |
| 2 | 5<<10 | -0,21 | -0,233 | -0,275 | -0,393 | -0,238 | -0,298 | -0,398 | -0,311 | -0,301 | -0,042 | 0,02 |
| 3 | 10<<15 | -0,389 | -0,403 | -0,142 | -0,021 | -0,406 | -0,155 | -0,024 | 0,044 | 0,105 | -0,001 | -0,013 |
| 4 | 15<<20 | 0,012 | 0,014 | 0,12 | 0,183 | 0,018 | 0,123 | 0,186 | 0,052 | 0,172 | -0,076 | -0,039 |
| 5 | 20<<30 | -0,048 | -0,027 | -0,077 | -0,071 | -0,016 | -0,057 | -0,059 | 0,093 | -0,058 | 0,027 | 0,009 |
| 6 | 30<<60 | 0,208 | 0,069 | 0,219 | 0,082 | 0,049 | 0,08 | 0,06 | 0,23 | 0,015 | 0,039 | -0,057 |
| 7 | 60<<90 | -0,338 | 0,246 | -0,369 | 0,273 | 0,089 | 0,271 | 0,087 | 0,629 | 0,055 | 0,064 | -0,311 |
| Average deviation |  | 0,32 | 0,30 | 0,21 | 0,16 | 0,27 | 0,18 | 0,12 | 0,3 | 0,16 | 0,11 | 0,09 |
| Max over-estimation |  | 0,21 | 0,25 | 0,22 | 0,27 | 0,09 | 0,27 | 0,19 | 0,63 | 0,41 | 0,06 | 0,02 |
| Max under-estimation |  | 0,96 | 0,99 | 0,37 | 0,39 | 1,00 | 0,30 | 0,40 | 0,66 | 0,3 | 0,50 | 0,31 |
| RMS el_bin |  | 0,33 | 0,31 | 0,21 | 0,16 | 0,29 | 0,18 | 0,13 | 0,3 | 0,16 | 0,11 | 0,10 |

Table 6.3.2.2-2: Config 2

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS7nc |  | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<<5 | -3,351 | -3,36 | -0,6 | -0,018 | -3,372 | -0,608 | -0,03 | -0,729 | 1,568 | -0,421 | -0,302 |
| 2 | 5<<10 | -0,505 | -0,517 | -0,33 | -0,47 | -0,527 | -0,341 | -0,481 | -0,582 | -0,208 | -0,079 | 0,062 |
| 3 | 10<<15 | 0,285 | 0,269 | -0,141 | -0,05 | 0,263 | -0,156 | -0,057 | -0,041 | -0,07 | -0,017 | -0,092 |
| 4 | 15<<20 | -0,537 | -0,55 | 0,245 | 0,344 | -0,557 | 0,231 | 0,34 | 0,377 | 0,331 | -0,024 | -0,021 |
| 5 | 20<<30 | 0,177 | 0,175 | -0,162 | -0,241 | 0,161 | -0,163 | -0,257 | -0,181 | -0,317 | 0,036 | 0,025 |
| 6 | 30<<60 | 0,093 | -0,012 | 0,204 | 0,121 | -0,04 | 0,098 | 0,086 | 0,23 | -0,098 | 0,028 | -0,058 |
| 7 | 60<<90 | 0,011 | -0,056 | -0,356 | 0,493 | 0,224 | 0,393 | 0,086 | 0,53 | -0,383 | 0,008 | -0,09 |
| Average deviation |  | 0,87 | 0,87 | 0,29 | 0,25 | 0,89 | 0,29 | 0,19 | 0,39 | 0,45 | 0,09 | 0,09 |
| Max over-estimation |  | 0,29 | 0,27 | 0,24 | 0,49 | 0,26 | 0,39 | 0,34 | 0,53 | 1,37 | 0,04 | 0,06 |
| Max under-estimation |  | 3,35 | 3,36 | 0,60 | 0,47 | 3,37 | 0,61 | 0,48 | 0,73 | 0,38 | 0,42 | 0,30 |
| RMS el_bin |  | 1,08 | 1,08 | 0,30 | 0,26 | 1,10 | 0,29 | 0,20 | 0,39 | 0,49 | 0,09 | 0,09 |

Table 6.3.2.2-3: Config 3

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS7nc |  | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<<5 | -3,234 | -3,242 | -0,539 | -0,001 | -3,255 | -0,548 | -0,014 | -0,65 | 0,83 | -0,39 | -0,269 |
| 2 | 5<<10 | -0,802 | -0,814 | -0,936 | -1,043 | -0,824 | -0,948 | -1,054 | -1,265 | -0,655 | -0,048 | -0,727 |
| 3 | 10<<15 | -0,422 | -0,445 | -0,099 | 0,088 | -0,447 | -0,121 | 0,085 | -0,646 | 0,213 | -0,606 | -0,214 |
| 4 | 15<<20 | 0,042 | 0,015 | -0,031 | -0,095 | 0,018 | -0,058 | -0,092 | 0,556 | -0,104 | 0,327 | 0,444 |
| 5 | 20<<30 | -0,418 | -0,424 | -0,042 | 0,114 | -0,446 | -0,05 | 0,095 | -0,332 | 0,226 | -0,443 | 1,064 |
| 6 | 30<<60 | -0,039 | -0,262 | 0,101 | 0,032 | -0,44 | -0,081 | -0,113 | -0,202 | -0,106 | 0,02 | 2,849 |
| 7 | 60<<90 | -0,557 | 0,224 | -0,297 | 0,843 | -0,517 | 0,612 | -0,592 | 0,055 | -0,685 | 0,366 | 6,869 |
| Average deviation |  | 0,93 | 0,92 | 0,30 | 0,34 | 0,98 | 0,36 | 0,31 | 0,55 | 0,41 | 0,32 | 2,51 |
| Max over-estimation |  | 0,04 | 0,22 | 0,10 | 0,84 | 0,02 | 0,61 | 0,10 | 0,56 | 0,83 | 0,37 | 6,87 |
| Max under-estimation |  | 3,23 | 3,24 | 0,94 | 1,04 | 3,26 | 0,95 | 1,05 | 1,27 | 0,68 | 0,61 | 0,73 |
| RMS el_bin |  | 1,10 | 1,10 | 0,32 | 0,36 | 1,15 | 0,37 | 0,32 | 0,56 | 0,42 | 0,32 | 3,42 |

Table 6.3.2.2-4: Config 4

| Bin | Vertical range | BS1 | BS2 | BS3 | BS4 | BS5 | BS6 | BS7 | BS7nc |  | BS8 | BS9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0<<5 | -4,447 | -4,455 | -1,105 | -0,585 | -4,467 | -1,113 | -0,597 | -1,348 | 0,478 | -0,355 | -0,99 |
| 2 | 5<<10 | 0,415 | 0,405 | 0,047 | 0,011 | 0,396 | 0,037 | 0,003 | -0,425 | -0,19 | -0,474 | 0,182 |
| 3 | 10<<15 | 1,211 | 1,194 | 0,088 | 0,355 | 1,188 | 0,07 | 0,349 | 1,03 | 0,205 | 0,66 | 0,165 |
| 4 | 15<<20 | 0,914 | 0,894 | -0,058 | -0,263 | 0,895 | -0,077 | -0,265 | -0,702 | -0,575 | -0,509 | 0,605 |
| 5 | 20<<30 | -0,833 | -0,837 | -0,299 | -0,138 | -0,854 | -0,305 | -0,151 | -0,819 | 0,109 | -0,532 | 0,722 |
| 6 | 30<<60 | -0,472 | -0,443 | -0,088 | -0,052 | -0,391 | -0,152 | -0,022 | 0,491 | 0,156 | -0,173 | 3,088 |
| 7 | 60<<90 | -0,567 | 0,348 | -0,73 | 0,805 | 0,822 | 0,109 | 1,234 | 1,958 | 1,07 | 1,68 | 6,215 |
| Average deviation |  | 1,51 | 1,48 | 0,36 | 0,32 | 1,53 | 0,28 | 0,39 | 1,00 | 0,41 | 0,65 | 2,30 |
| Max over-estimation |  | 1,21 | 1,19 | 0,09 | 0,81 | 1,19 | 0,11 | 1,23 | 1,96 | 1,07 | 1,68 | 6,22 |
| Max under-estimation |  | 4,45 | 4,46 | 1,11 | 0,59 | 4,47 | 1,11 | 0,60 | 1,35 | 0,58 | 0,53 | 0,99 |
| RMS el_bin |  | 1,83 | 1,81 | 0,38 | 0,33 | 1,85 | 0,30 | 0,41 | 1,03 | 0,42 | 0,68 | 3,02 |

#### Calculation methodology:

The EEIRP is calculated for each of the (seven) different elevation bins for each considered array antenna configuration and beam-set (BS1 to BS9) including the rectangular reference beam set with 2057 beams (17 along elevation axis and 121 along azimuth axis). An elipsoidal reference case is also considered, by removing the corner areas of the rectangular reference case.

Figure 6.3.2.2-3 Illustration of reference cases

The EEIRP difference between each beam set and the reference grid is calculated for each of the elevation bins.

For every combination of array configuration and beam set, three metrics are then considered based on this difference.

- The average deviation “AVG deviation” is the linear average for each elevation bin (average of 7 values, one for each elevation bin) and is intended to represent the deviation of the calculated EEIRP compared to the reference beam set.

- The maximum over-estimation “Max over-estimation” is the maximum positive value over the 7 bins and is intended to represent the largest error that is observed ABOVE the levels of the reference EEIRP.

- The maximum under-estimation “Max under-estimation” is the minimum negative value over the 7 bins and is intended to represent the largest error that is observed BELOW the levels of the reference EEIRP.

The aggregated results per beam-set as presented in Table 2.2-2 are the WORST values observed over the different array configurations, i.e. the highest values of “AVG deviation”, “Max over-estimation” and “Max under-estimation” between all considered antenna arrays.

The fourth metric “RMS_TBS_dBm” is the RMS representation of the error per beam set, over all antenna configurations and elevation bins where the error is considered as the difference of the beam-set specific EEIRP to that of the reference beam set.

i.e., for Beam Set (BS) 1 to 9,  RMSbs=  10log10mMnNEEIRPnmbs-EEIRPnmref2NM 
where EEIRPnm is the calculated EEIRP for antenna configuration m and elevation bin n.

The difference between the two variations of RMS (“on dB diff”, “on linear diff”) is that the EEIRP difference, EEIRPnmbs-EEIRPnmref, is taken in linear scale in one case and on dB scale (and converting afterwards to linear) on the other.

In Table 6.3.2.2-4, the simulation results are presented for each considered Beam Set (BS1 to BS9). The Beam Set is defined as Mcolumn x Nrow. The rectangular reference case is considered as a baseline for all configurations. Additionally, for the BS7nc there are two set of values: the first one is for the rectangular reference case and the second one for the elipse reference case.

Table 6.3.2.2-4: Simulation results

|  | BS1 (4x3) |  |  | BS2 (4x4) |  | BS3 (6x3) |  | BS4 (5x4) |  | BS5 (4x5) |  | BS6 (6x4) |  | BS7 (5x5) |  | BS7nc (5x5 nc) | BS8 (Star) |  | BS9 (Random) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVG deviation | 1,51 |  |  | 1,48 |  | 0,36 |  | 0,34 |  | 1,53 |  | 0,36 |  | 0,39 |  | 1,00 / 0,41 | 0,65 |  | 2,51 |  |  |  |
| Max over-estimation | 1,21 |  |  | 1,19 |  | 0,24 |  | 0,84 |  | 1,19 |  | 0,61 |  | 1,23 |  | 1,96 / 1,07 | 1,68 |  | 6,87 |  |  |  |
| Max under-estimation | 4,45 |  |  | 4,46 |  | 1,11 |  | 1,04 |  | 4,47 |  | 1,11 |  | 1,05 |  | 1,35 / 0,68 | 0,61 |  | 0,99 |  |  |  |
| RMS_TBS_dBm | 0,56 |  |  | 0,55 |  | 0,15 |  | 0,14 |  | 0,57 |  | 0,14 |  | 0,13 |  | 0,59 / 0,33 | 0,15 |  | 0,97 |  | On dB diff |  |
| RMS_TBS_dBm | -6,8 | -6,8 | -12,1 | -15,3 | -6,8 | -12,0 | -15,2 | -10,7 / -13,1 | -13,0 | -12,7 | On Linear diff |

The result is colour coded to visualize good and bad beam sets. The simulation result shows BS3, BS4, BS6, BS7 and BS8 produce acceptable deviations with respect to the reference case. Note that the of beams along the vertical axis is essential to reduce the deviation [R4-2501648].

#### 6.3.1.2	Huawei results

##### 6.3.1.2.1	Average Metric

3 different metrics have been used in the evaluation:

RMS average of the error in dB

RMSdB=E0-52+E5-102+E10-152+E15-202+E20-302+E30-602+E60-90270.5

ABSAVdB=E0-5+E5-10+E10-15+E15-20+E20-30+E30-60+E60-907

Average of the absolute dB values

RealAVdB=10*log1010E0-510+10E5-1010+10E10-1510+10E15-2010+10E20-3010+10E30-6010+10E60-90107

RMS average of the absolute error in linear units (displayed in dB)

The 3 method all use the absolute error in dB and form slightly different averages. Considering the 3 metrics, in the range of dB errors are experiencing (approx. 0 to 2dB), the difference in the resulting average is not huge as such it is probably not very important which is used.

In addition to the average metric the overestimate and underestimate values are also compared to ensure that no excessive error in individual angular bins are experienced.

##### 6.3.1.2.2	Test cases

The test cases are based on ANT4 (see table 6.3.1.2.2-1) with 2 different steering range configurations.

Table 6.3.1.2.2-1: Antenna parameters

The steering range configurations are:

Steering range 1

Mechanical tilt = 0deg

Electrical steering range = 0 to -12deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

Steering range 2

Mechanical down-tilt = 4deg

Electrical steering range = 0 to -16deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

The test beam direction sets currently under consideration are shown in figure 6.3.1.2.2-1:

12 point (5x3)

Figure 6.3.1.2.2-1. Test beam direction sets

The result is listed in Table 6.3.1.2.2-2, Table 6.3.1.2.2-3, Table 6.3.1.2.2-4 and Table 6.3.1.2.2-5

Table 6.3.1.2.2-2: Results for ANT4, Steering range 1

Table 6.3.1.2.2-3: Results for ANT 4, Steering range 2

Table 6.3.1.2.2-4: Results for ANT 1, Steering range 1

Table 6.3.1.2.2-5: Results for ANT 1, Steering range 2

Given these results the main conclusion is that it is very difficult to identify any test beam direction set as being significantly better than any other. As it has been seen previously, it seems  at least 5 points in elevation for the the errors to be small.

#### 6.3.1.3	Nokia results

##### 6.3.1.3.1 Simulation results

The EEIRP calculation involves calculating the average EIRP over a defined set of elevation angles (around the base station, i.e. 360° azimuth) for the full range of beamforming directions. Two Reference Sets (RefSet1 and RefSet2) of the full range of beamforming directions is shown in Figure 6.3.1.3.1-1 and Figure 6.3.1.3.1-2, with the vertical steering range set to -16° ≤ θetilt ≤ 0° and -12° ≤ θetilt ≤ 0° respectively. The number of beamforming directions for these examples are 2057 and 1573 respectively, for a 1° steering resolution (i.e. 17 x 121 and 13 x 121). These figures are from the viewpoint of the UE into the BS antenna panel.

Figure 6.3.1.3.1-1: Reference Set 1 (RefSet1) of beamforming directions

Figure 6.3.1.3.1-2: Reference Set 2 (RefSet2) of beamforming directions

##### 6.3.1.3.2 Test Beams Sets

The following Test Beams Sets (TBSs) shown in Figure 6.3.1.3.2-1 and Figure 6.3.1.3.2-2 are evaluated against RefSet1 and RefSet2 respectively.

Figure 6.3.1.3.2-1: Test Beams Sets for RefSet1

Figure 6.3.1.3.2-2: Test Beams Sets for RefSet2

##### 6.3.1.3.3 Antenna configurations

These TBSs are evaluated over six different antenna configurations. The antenna parameters for these configurations are shown in Table 6.3.1.3.3-1 where Config 1 and 2 are non-sub-array antennas, whilst Config 3 to 6 contains sub-arrays.

| Parameter |  | Antenna |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| Element | Vertical HPBW (°) | 90 | 65 | 65 | 65 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | 5.5 | 5.5 | 6.4 | 6.6 | 6.6 | 6.8 |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 | 30 | 30 |
| Array configuration | Number of rows | 16 | 16 | 8 | 8 | 8 | 16 |
|  | Number of columns | 8 | 8 | 16 | 8 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 2x1 | 3x1 | 2x1 |
|  | Sub-array pre-tilt: (°) | - | - | 6 | 0 | 0 | 0 |
|  | Vertical spacing (λ) | 0.5 | 0.7 | 2.1 | 1.4 | 2.1 | 1.4 |
|  | Horizontal spacing (λ) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

Table 6.3.1.3.3-1: Antenna parameters

NOTE: These parameters are just some example antenna configurations for the purpose of evaluating the suitability of the Test Beams sets rather than for evaluating against the specific EEIRP values required for the different elevation ranges. In addition, the TRP for each of the configuration is set to 20 dBm/MHz and no specific antenna techniques such as sidelobe suppression are used where these techniques could potentially lower the EIRP level at various vertical and horizontal angles.

##### 6.3.1.3.4 Evaluation of Test Beams Sets

The Test Beams sets in clause 6.3.1.3.2 are used to evaluate against the antenna configurations in clause 6.3.1.3.3. The Reference Sets illustrated in Figure 6.3.1.31-1 are based on 1° steering resolution, and the calculation of the EEIRP above the horizon (upper hemisphere) is based on 0.5° sampling resolution in order to capture the beam as accurately as possible.

The antenna pattern generated from these configurations were normalised, and just to note that the normalisation factor is only up to around 1.5 dB. The results shown in Table 6.3.1.3.4-1 to Table 6.3.1.3.4-10 is in the form of absolute difference (delta) in linear comparing the EEIRP generated from those Test Beams set to the Reference Set. This delta calculation is as shown below:

Deltabin1,…,7_linear=10EEIRPTBS_dBm -EEIRPref_dBm10    ,

where Deltabin1,…,7_linear is the linear multiple/fractional delta for elevation bin 1 to 7 (bin1 to bin7), and EEIRPTBS_dBm -EEIRPref_dBm is the absolute value of the difference between the EEIRP generated from the Test Beams set to the particular Reference Set in logarithmic scale. Just to note that the closer this delta to linear value of ‘1’, the better it represents the Reference Set.

The results benchmarking against Reference Set 1, RefSet1, is shown in Table 6.3.1.3.4-1 to Table 6.3.1.3.4-5.

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 1: 12 Beams (3 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.40 | 2.12 | 3.22 | 2.03 | 2.96 | 2.87 |
| 2 | 5<q<10 | 1.00 | 1.13 | 1.12 | 1.11 | 1.13 | 1.05 |
| 3 | 10<q<15 | 1.22 | 1.07 | 1.15 | 1.08 | 1.03 | 1.01 |
| 4 | 15<q<20 | 1.04 | 1.05 | 1.17 | 1.17 | 1.10 | 1.04 |
| 5 | 20<q<30 | 1.02 | 1.03 | 1.17 | 1.10 | 1.03 | 1.03 |
| 6 | 30<q<60 | 1.04 | 1.06 | 1.08 | 1.07 | 1.01 | 1.19 |
| 7 | 60<q<90 | 1.02 | 1.15 | 1.03 | 1.11 | 1.13 | 1.03 |

Table 6.3.1.3.4-1: Difference of Test Beams Set 1 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 2: 12 Beams (4 x 3) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.00 | 1.15 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.00 | 1.14 | 1.01 | 1.08 | 1.06 |
| 4 | 15<q<20 | 1.00 | 1.00 | 1.02 | 1.06 | 1.00 | 1.08 |
| 5 | 20<q<30 | 1.01 | 1.00 | 1.15 | 1.12 | 1.07 | 1.08 |
| 6 | 30<q<60 | 1.05 | 1.03 | 1.00 | 1.09 | 1.08 | 1.02 |
| 7 | 60<q<90 | 1.08 | 1.05 | 1.01 | 1.08 | 1.24 | 1.13 |

Table 6.3.1.3.4-2: Difference of Test Beams Set 2 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 3: 16 Beams (4 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.01 | 1.15 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.01 | 1.14 | 1.01 | 1.07 | 1.06 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.06 | 1.00 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.01 | 1.14 | 1.11 | 1.07 | 1.07 |
| 6 | 30<q<60 | 1.03 | 1.01 | 1.05 | 1.09 | 1.01 | 1.02 |
| 7 | 60<q<90 | 1.01 | 1.00 | 1.03 | 1.00 | 1.02 | 1.03 |

Table 6.3.1.3.4-3: Difference of Test Beams Set 3 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 4: 20 Beams (4 x 5) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.01 | 1.14 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.01 | 1.13 | 1.00 | 1.07 | 1.06 |
| 4 | 15<q<20 | 1.01 | 1.01 | 1.02 | 1.05 | 1.01 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.02 | 1.15 | 1.11 | 1.07 | 1.08 |
| 6 | 30<q<60 | 1.02 | 1.00 | 1.11 | 1.08 | 1.02 | 1.02 |
| 7 | 60<q<90 | 1.03 | 1.00 | 1.12 | 1.00 | 1.02 | 1.03 |

Table 6.3.1.3.4-4: Difference of Test Beams Set 4 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 5: 20 Beams (5 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.22 | 1.39 | 1.66 | 1.36 | 1.60 | 2.84 |
| 2 | 5<q<10 | 1.04 | 1.02 | 1.53 | 1.02 | 1.55 | 1.09 |
| 3 | 10<q<15 | 1.08 | 1.01 | 1.11 | 1.01 | 1.07 | 1.05 |
| 4 | 15<q<20 | 1.01 | 1.00 | 1.05 | 1.05 | 1.05 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.01 | 1.09 | 1.10 | 1.05 | 1.15 |
| 6 | 30<q<60 | 1.02 | 1.02 | 1.06 | 1.09 | 1.01 | 1.12 |
| 7 | 60<q<90 | 1.01 | 1.00 | 1.04 | 1.01 | 1.10 | 1.03 |

Table 6.3.1.3.4-5: Difference of Test Beams Set 5 from RefSet1

The results benchmarking against Reference Set 2, RefSet2, is shown in Table 6.3.1.3.4-6 to Table 6.3.1.3.4-10.

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 1: 12 Beams (3 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.25 | 1.51 | 2.74 | 1.49 | 2.61 | 3.40 |
| 2 | 5<q<10 | 1.02 | 1.01 | 1.09 | 1.01 | 1.10 | 1.12 |
| 3 | 10<q<15 | 1.16 | 1.02 | 2.13 | 1.02 | 2.34 | 1.05 |
| 4 | 15<q<20 | 1.01 | 1.03 | 1.02 | 1.02 | 1.02 | 1.06 |
| 5 | 20<q<30 | 1.00 | 1.04 | 1.12 | 1.40 | 1.10 | 1.90 |
| 6 | 30<q<60 | 1.04 | 1.00 | 1.25 | 1.08 | 1.14 | 1.04 |
| 7 | 60<q<90 | 1.03 | 1.05 | 1.05 | 1.03 | 1.00 | 1.01 |

Table 6.3.1.3.4-6: Difference of Test Beams Set 1 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 2: 12 Beams (4 x 3) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.33 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.09 |
| 3 | 10<q<15 | 1.11 | 1.00 | 1.54 | 1.00 | 1.66 | 1.04 |
| 4 | 15<q<20 | 1.01 | 1.00 | 1.02 | 1.01 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.01 | 1.00 | 1.07 | 1.29 | 1.07 | 1.89 |
| 6 | 30<q<60 | 1.05 | 1.03 | 1.14 | 1.06 | 1.03 | 1.06 |
| 7 | 60<q<90 | 1.09 | 1.02 | 1.01 | 1.05 | 1.09 | 1.11 |

Table 6.3.1.3.4-7: Difference of Test Beams Set 2 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 3: 16 Beams (4 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.34 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.10 |
| 3 | 10<q<15 | 1.10 | 1.00 | 1.54 | 1.00 | 1.66 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.00 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.00 | 1.02 | 1.07 | 1.27 | 1.07 | 1.89 |
| 6 | 30<q<60 | 1.03 | 1.02 | 1.26 | 1.06 | 1.13 | 1.06 |
| 7 | 60<q<90 | 1.02 | 1.01 | 1.05 | 1.01 | 1.12 | 1.02 |

Table 6.3.1.3.4-8: Difference of Test Beams Set 3 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 4: 20 Beams (4 x 5) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.34 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.10 |
| 3 | 10<q<15 | 1.10 | 1.00 | 1.54 | 1.00 | 1.65 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.00 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.01 | 1.02 | 1.07 | 1.27 | 1.07 | 1.90 |
| 6 | 30<q<60 | 1.02 | 1.01 | 1.27 | 1.05 | 1.10 | 1.06 |
| 7 | 60<q<90 | 1.06 | 1.02 | 1.11 | 1.01 | 1.18 | 1.04 |

Table 6.3.1.3.4-9: Difference of Test Beams Set 4 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 5: 20 Beams (5 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.17 | 1.29 | 1.49 | 1.28 | 1.46 | 2.00 |
| 2 | 5<q<10 | 1.02 | 1.02 | 1.14 | 1.02 | 1.13 | 1.01 |
| 3 | 10<q<15 | 1.08 | 1.01 | 1.43 | 1.01 | 1.50 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.00 | 1.04 | 1.01 | 1.00 | 1.07 |
| 5 | 20<q<30 | 1.01 | 1.01 | 1.07 | 1.23 | 1.07 | 1.70 |
| 6 | 30<q<60 | 1.03 | 1.02 | 1.21 | 1.05 | 1.12 | 1.05 |
| 7 | 60<q<90 | 1.02 | 1.00 | 1.05 | 1.01 | 1.12 | 1.08 |

Table 6.3.1.3.4-10: Difference of Test Beams Set 5 from RefSet2

A simple root-mean-square deviation (RMSD) metric can be used as a ‘figure of merit’ to rank the suitability of these Test Beams Sets, and the metric is shown below:

RMSDTBS_dB=10 ×log102RMSDallbins_linear_A12+⋯+RMSDallbins_linear_A626  ,

where the RMSD calculated for each antenna configuration, Ax, (over all elevation bins) is derived as

RMSDallbins_linear_Ax=2Deltabin1_linear_Ax2+⋯+Deltabin7_linear_Ax27

and, Deltabin1_linear_Ax , … , Deltabin7_linear_Ax indicating the linear delta for elevation bin 1 to 7 for the antenna configuration Ax.

| No ` | Test Beams Set | RMSD (dB) |  |
| --- | --- | --- | --- |
|  |  | RefSet1 | RefSet2 |
| 1 | 12 Beams (3 x 4) | 1.41 | 1.49 |
| 2 | 12 Beams (4 x 3) | 1.10 | 0.91 |
| 3 | 16 Beams (4 x 4) | 1.06 | 0.92 |
| 4 | 20 Beams (4 x 5) | 1.07 | 0.93 |
| 5 | 20 Beams (5 x 4) | 0.80 | 0.66 |

Table 6.3.1.3.4-11: Summary of evaluation of TBSs for both Reference Sets

Just to note again that the closer this RMSD to logaritmic (dB) value of ‘0’, the better it represents the Reference Set.

##### 6.3.1.3.5 Additional results

Additional simulations were carried out based on the rectangular Reference Set 2 (RefSet2) as illustrated earlier, and a corresponding elliptical reference set, Reference Set 3 (RefSet3), as shown in Figure 6.3.1.3.5-1.

Figure 6.3.1.3.5-1: Reference Set 3 (RefSet3) of beamforming directions

There are six additional Test Beams Sets being simlated as shown in Figure 6.3.1.3.5-2 where a ‘distributed’ pattern was used in the last Set (shown in bottom right corner in the figure). The ‘distributed’ pattern is known to produce quite accurate results when compared to the Reference Set even with smaller number of Test Beams.

Figure 6.3.1.3.5-2: Additional Test Beams Sets

These additional simulations were carried out using antenna configuration Config 1 to Config 4 where the sub-array pre-tilt for Config 4 changed to 6° to be more common in sub-array designs.

Using the same RMSD formulation in as in the earlier section, the results for both Reference Sets, RefSet2 and RefSet3, is shown in

| Reference Set 2 (Rectangle) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Test Beams Set | RMSD | Max Over-estimation | Min Under-estimation |
| 1 | 25 Beams (5 x 5) | 0.44 | 0.23 | -1.72 |
| 2 | 21 Beams (5 x 5) - NC | 0.82 | 0.90 | -2.55 |
| 3 | 16 Beams (5 x 4) - NC | 0.99 | 1.09 | -2.85 |
| 4 | 15 Beams (5 x 3) | 0.43 | 0.39 | -1.73 |
| 5 | 20 Beams (5 x 4) | 0.43 | 0.20 | -1.72 |
| 6 | 12 Beams (4 x 3) - Distributed | 0.33 | 0.08 | -1.25 |

Table 6.3.1.3.5-1: Summary results from RefSet2

| Reference Set 3 (Ellipse) |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Test Beams Set | RMSD | Max Over-estimation | Min Under-estimation |
| 1 | 25 Beams (5 x 5) | 0.34 | 0.08 | -1.25 |
| 2 | 21 Beams (5 x 5) - NC | 0.63 | 0.62 | -2.08 |
| 3 | 16 Beams (5 x 4) - NC | 0.79 | 0.81 | -2.38 |
| 4 | 15 Beams (5 x 3) | 0.29 | 0.11 | -1.26 |
| 5 | 20 Beams (5 x 4) | 0.32 | 0.07 | -1.25 |
| 6 | 12 Beams (4 x 3) - Distributed | 0.25 | 0.00 | -0.89 |

Table 6.3.1.3.5-2: Summary results from RefSet3

Based on the additional set of results, only the 12 Beams (4 x 3) – Distributed set are able to produce an under-estimation of less than 1 dB when compared to Reference Set 3 while offering the lowest under-estimation when compared to Reference Set 2. These additional simulations conclude that lower number of Test Beams can offer better accuracy than higher number of Test Beams given appropriate test beam pattern.

#### 6.3.1.4	ZTE results

##### 6.3.1.4.1  Methodology on how to create the reference testing beams

In order to get the reference/average EEIRP simulation results, the following extreme number of testing number e.g. 10K testing beams within rectangular steering range and 8k testing beams within ellipse steering range are evaluated. In the following section, the methodology on how to generate the beams within rectangular and ellipse steering range are clarified in details.

##### 6.3.1.4.2. Rectangular reference beam sets

Figure 6.3.1.4.2-1. the illustration of beam steering range in the rectangular coverage

The methodology on how to generate the beams within rectangular steering range as shown in Figure 6.3.1.4.2-1 is described as following:

- To generate the 10k uniform distributed values between 0 degree to 12 degree, say X

- To generate the 10k uniform distributed values between -60 degree to 60 degree, say Y.

- To create rectangular reference beam sets as [X, Y] where Xi is electrical down-tilt of beam i and Yi is electrical scanning of beam i.

##### 6.3.1.4.3. Ellipse reference beam sets

Figure 6.3.1.4.3-1. the illustration of beam steering range in the ellipse coverage

The methodology on how to generate the beams within ellipse steering range as shown in Figure 6.3.1.4.3-1 is described as following:

- To generate the 8k random values for the angle: theta=2 * pi * rand(N, 1) ;

- To generate the 8k random values for r: r=sqrt(rand(N, 1)) ;

- To create the 8k azimuth angle as following

- Y= 0+maximum azimuth steering range * r .* cos(theta);

- To create the 8k elevation angle as following

- X= 1/2* maximum elevation steering range + 1/2* maximum elevation steering range * r .* sin(theta)

- To create ellipse reference beam sets as [X, Y] where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i.

##### 6.3.1.4.4. Candidate testing beams for EEIRP conformance testing

###### 6.3.1.4.4.1.  Random pattern with 20 testing beams

As shown in the figure 6.3.1.4.4.1-1, it is to use the random approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.1.4.4.1-1. random approach to create the testing beams for EEIRP conformance testing

###### 6.3.1.4.4.2. 20 fixed testing beams

As shown in the figure 6.3.1.4.4.2-1, it is proposed to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/20=6 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.1.4.4.2-1. Equally distributed approach to create the 20 testing beams for EEIRP conformance testing

###### 6.3.1.4.4.3. 16 fixed testing beams

As shown in the figure 6.3.1.4.4.3-1, it is proposed to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/20=0.6 degree

- To calculate the step size for azimuth angle as 120/20=6 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.1.4.4.3-1. equally distributed approach to create the 16 testing beams for EEIRP conformance testing

###### 6.3.1.4.4.4.  25 fixed testing beams

As shown in the figure 6.3.1.4.4.4-1, it is proposed to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

Figure 6.3.1.4.4.4-1. equally distributed approach to create the 25 testing beams for EEIRP conformance testing

###### 6.3.1.4.4.5.  Scaling testing beams

As shown in the figure 6.3.1.4.4.5-1/2/3, considering BS capable with different steering range in the elevation angle. For electrical downtilt as 12 degree, 16 testing beams is considered to be sufficient to get the accurate results compared with reference case EEIRP. For the case with 16 degree electrical down-tilt, a larger number of testing beams in the elevation angle achieves better accuracy.

Scaling testing beams is proposed as following with fixed step size in the elevation angle as following. If testing beams within the steering range could be select as conformance testing. e.g. if NR BS is capable of downtilt as 12 degree, then 16 test beams should be tested; if NR BS is capable of downtilt as 16 degree, then 20 test beams should be considered.

Figure 6.3.1.4.4.5-1. Equally distributed approach to create the 16 testing beams for EEIRP conformance testing

Figure 6.3.1.4.4.5-2. Equally distributed approach to create the 20 testing beams for EEIRP conformance testing

Figure 6.3.1.4.4.5-3. Equally distributed approach to create the 24 testing beams for EEIRP conformance testing

###### 6.3.1.4.4.6. 21 testing beam

As shown in the figure 6.3.1.4.4.6-1, it is proposed to use the equally distributed approach to get the sampling for the testing beams. The details could be found as following example:

- To calculate the step size for elevation angle as 12/25=0.48 degree

- To calculate the step size for azimuth angle as 120/25=4.8 degree

- To create option 1a beam sets as [X, Y] according to the above step size with beam peak steering direction located in the center of each indicated finer box where Xi is electrical downtilt of beam i and Yi is electrical scanning of beam i with elevation beam steering range from 0 degree to 12 degree and azimuth beam scanning range from -60 degree to 60 degree.

- 4 corner beams are not considered for the EEIRP evaluation.

Figure 6.3.1.4.4.6-1. 5x5 without 4 corner beam for EEIRP conformance testing

###### 6.3.4.  Simulation results for EEIRP evaluation

The simulation results for above candidate testing beam are summarized in the following table.

Table 6.3.4-1. simulation results for Antenna configuration 4 with electrical downtilt 12 degree

| Candidate test beam | Ant 4 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.29 | 0.30 | -0.64 | 0.67 | 1.53 | -1.40 |
| 5x4 | 0.28 | 0.23 | -0.49 | 0.46 | 0.76 | -0.54 |
| 4x4 | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |
| 5x5 | 0.30 | 0.16 | -0.49 | 0.55 | 0.75 | -1.11 |
| Scaling test beam | 0.48 | 0.26 | -0.87 | 0.48 | 0.57 | -0.75 |
| 5x5 without corner | 0.42 | 0.36 | -0.98 | 0.18 | 0.25 | -0.40 |

Table 6.3.4-2. simulation results for Antenna configuration 4 with electrical downtilt as 16 degree

| Candidate test beam | Ant 4 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.25 | 0.26 | -0.49 | 0.81 | 1.42 | -1.57 |
| 5x4 | 0.47 | 0.52 | -1.20 | 0.59 | 1.05 | -0.79 |
| 4x4 | 0.73 | 0.49 | -2.94 | 0.74 | 0.93 | -1.52 |
| 5x5 | 0.44 | 0.02 | -1.21 | 0.69 | 1.05 | -1.23 |
| Scaling test beam | 0.74 | 0.82 | -1.52 | 0.43 | 0.90 | -0.37 |
| 5x5 without corner | 0.80 | 0.64 | -1.51 | 0.33 | 0.38 | -0.97 |

Table 6.3.4-3. simulation results for Antenna configuration 1 with electrical downtilt as 12 degree

| Candidate test beam | Ant 1 with electrical downtilt as 12 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.07 | 0.21 | -0.05 | 0.46 | 1.32 | -0.90 |
| 5x4 | 0.07 | 0.05 | -0.31 | 0.35 | 0.80 | -0.85 |
| 4x4 | 0.12 | 0.07 | -0.50 | 0.33 | 0.61 | -0.90 |
| 5x5 | 0.07 | 0.05 | -0.32 | 0.34 | 0.79 | -0.84 |
| 5x5 without corner | 0.34 | 0.71 | -0.89 | 0.08 | 0.22 | -0.14 |

Table 6.3.4-4. simulation results for Antenna configuration 1 with electrical downtilt as 12 degree

| Candidate test beam | Ant 1 with electrical downtilt as 16 degree |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rectangular reference |  |  | Ellipse reference |  |  |
|  | Mean error | OverEstimate | UnderEstimate | Mean error | OverEstimate | UnderEstimate |
| Random pattern | 0.08 | 0.25 | -0.01 | 0.50 | 1.52 | -0.83 |
| 5x4 | 0.11 | 0.04 | -0.53 | 0.35 | 0.74 | -0.83 |
| 4x4 | 0.20 | 0.09 | -0.94 | 0.29 | 0.32 | -0.84 |
| 5x5 | 0.11 | 0.04 | -0.53 | 0.34 | 0.73 | -0.81 |
| 5x5 without corner | 0.38 | 0.72 | -1.13 | 0.07 | 0.14 | -0.12 |

Based on the above simulation results, the performance metric for 5x4 and 5x5 is quite close. The underestimate performance for 5x5 is even around 0.5dB worse than 5x4 case if ellipse reference is considered. And performance metric for 5x4 and 4x4 is also quite close when electrical downtilt as 12 degree.

### 6.3.3	Conclusion on test beam set

According to the EEIRP evaluation in clause 6.3.2,  21 testing beam constellation could provide the accurate EEIRP estimation results compared with the reference beam set from both average error and also under-estimate /over-estimate performance.

Given the results presented in clause 6.3.2,  RAN4 decided a the test beams set consisting of 21 beams in a constellation of 5x5 without corners (i.e. 25 – 4=21):

Figure 6.3.3-1 Test beam set for conformance testing

The main reasons for choosing this test beam set are:

- 5 beams in the elevation domain prove to be sufficient for accurate EEIRP estimation

- Exclusion of the corner beams is justified by their low relevance towards real operation scenarios.

- 21 testing beam constellation could provide the sufficient in the declared beam steering range;

- 21 testing beam constellation could allow for the different antenna geometry.

θa,b=aAθmax-θmin-θmax-θmin2+θminφa,b=bBφmax-φmin-φmax-φmin2+φmin

| Beam index | Elevation angle | Azimuth angle | Weighting factor |
| --- | --- | --- | --- |
| 1 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 2 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 3 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 4 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 5 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 6 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 7 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 8 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 9 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 10 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 11 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 12 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 13 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 14 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 | 1/21 |
| 15 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 16 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 17 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |
| 18 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 | 1/21 |
| 19 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 | 1/21 |
| 20 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 | 1/21 |
| 21 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 | 1/21 |