3GPP TSG-RAN WG4 Meeting #114	R4-2502366

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on Spectrum Analyser configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_TRP_TRS_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The parameters for the spectrum analyser were added to this TR in the MU section, which is purely informative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved detailed configuration parameters for the spectrum analyser to the Test Setup section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detailed configuration parameters for the spectrum analyser are not properly captured in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, B.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged Text Deleted >

< Beginning of Changes >

# 7	Anechoic Chamber method (Reference method)

## 7.1	General

This clause defines the test method with Anechoic Chamber system, which is agreed as a reference method for FR1 TRP and TRS testing.

## 7.2	Test setup

### 7.2.1	Test system

For FR1 TRP TRS testing, both Single-antenna and multiple-antennas anechoic chambers can be applied. In Figure 7.2.1-1, an example TRP TRS test system with combined axes system is presented.

Figure 7.2.1-1: Example of a FR1 TRP TRS OTA test system with combined axis

In Figure 7.2.1-2, an example TRP TRS test system with distributed axes system is presented.

Figure 7.2.1-2: Example of a FR1 TRP TRS OTA test system with distributed axis

### 7.2.2	Test equipment configuration

#### 7.2.2.1	Spectrum analyser

If a spectrum analyser is used for TRP measurement as specified in Clause 7.5, to perform either an Integrated Channel Power or a Total Channel Power measurement, the detailed parameters in Table 7.2.2.1-1 shall be used.

Table 7.2.2.1-1: Detailed parameters of spectrum analyser for NR FR1 TRP measurement

| Channel BW (MHz) | SCS [kHz] | NRB | RB Allocation | RB Start | Frequency Offset (MHz) | fspan (MHz) (Note 1) | fGaussian (MHz) (Note 2) | fflat top (MHz) (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | 25 | 12 | 6 | -0.09 | 2.4 | 7 | 3 |
| 10 | 15 | 52 | 25 | 12 | -0.27 | 5 | 14 | 6 |
| 15 | 15 | 79 | 36 | 18 | -0.63 | 7.2 | 20 | 8 |
| 20 | 15 | 106 | 50 | 25 | -0.54 | 10 | 27 | 11 |
| 30 | 15 | 160 | 80 | 40 | 0 | 16 | 44 | 18 |
| 100 | 30 | 273 | 135 | 67 | -0.72 | 54 | 146 | 59 |
| fRBW (Note 1) |  |  |  | 30 kHz |  |  |  |  |
| Tdwell (Note 2) or Tsweep (Note 1) |  |  |  | 100 ms |  |  |  |  |
| Tshort dwell (Note 2) or Tshort sweep (Note 1) |  |  |  | 20 ms |  |  |  |  |
| Pflat (Note 1) |  |  |  | 70% |  |  |  |  |
| Note 1: Used for Integrated Channel Power measurements. Note 2: Used for Total Channel Power measurements. |  |  |  |  |  |  |  |  |

< Unchanged Text Deleted >

## B.2.4	Measurement receiver: uncertainty of the absolute level

The receiving device is used to measure the received signal level in TRP tests either as an absolute level or as a relative level. Receiving device used is typically a communication tester, spectrum analyser (SA), or power meter (PM). Generally, there occurs an uncertainty contribution from limited absolute level accuracy and non-linearity.

This uncertainty will be determined from the manufacturer’s datasheet and the distribution used (see clause 5.1.2 in [11]) shall match that provided in the datasheet which are commonly quoting MUs/accuracies with a “95% confidence level” and/or a “coverage factor of 2”. In the absence of a declared distribution in the datasheet, the rectangular distribution should be used. Given the wide bandwidths of the modulated signal transmitted during the testing, the level flatness and not just the CW level accuracy shall be considered.

If a spectrum analyser is used for TRP measurement as specified in Clause 7, the detailed parameters in Table B.2.4-1 should be used.

Table B.2.4-1: Detailed parameters of spectrum analyser for NR FR1 TRP measurement

|  |  |  |  |  |  | fspan | fGaussian | fflat top |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

< End of Changes >