3GPP TSG-RAN4 Meeting #114	R4-2502367

# Athens, Greece, 17th - 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.161 | CR | 0022 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for split measurement grids method for TRP/TRS measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of test methodology to allow XR devices to be tested without changing/charging the battery and multiple device re-positionings |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split the measurement grid up into multiple coarser grids, perform each TRP/TRS measurement on the coarse grid without the need for a battery change/charge, and then average the different results. This approach will allow TRP/TRS to be measured with the original, classical measurement grid. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | XR devices may need to change the battery during one long time OTA test, which yields higher uncertainty of the measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper was previously submitted in R4-2501806 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Add editor’s note related to MU and MTSU |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

## A.3.3.4 TRP/TRS OTA test based on averaging multiple split measurement grids

Editor’s note: The positioning uncertainty from using phantoms for limited battery capacity device is FFS. The final MTSU limit may not be relaxed to accommodate the worst-case test method due to the impact on test tolerance. This decision will need to be made after the MU analysis.

The classical TRP measurement requires 266 (for the Clenshaw-Curtis quadrature) or 264 (for the sin(theta) quadrature) unique grid points to properly assess the radiated TX performance as outlined in Clause A.3.5. Similarly, the classical TRS measurement requires 64 (Clenshaw-Curtis) or 62 (sin(theta)) unique grid points to properly assess the radiated RX performance. Some devices/device types with limited battery capacity might not support TRP/TRS testing performed using the full set of grid points without changing/charging the battery and multiple device re-positionings. Splitting the grid up into multiple coarser grids, performing each TRP/TRS measurement on the coarse grid without the need for a battery change/charge, and then averaging the different results would allow an accurate assessment of TRP/TRS performance without the need for device re-positionings during the TRP/TRS procedures.

A test grid with 266 test points, considered as the fine measurement grid (fine grid), can be split into four 64-grid points measurement grids (for example), considered here as split coarse measurement grid (split grid). The measurement grid points in each of split grid is a subset of that of the fine grid. Other than the grid points at the poles, the measurement grid points of one split grid are different from any measurement grid points in another split grid. However, the combination of the measurement points of all the split grid is still the same as the number of measurement points in the fine grid.

For such approximation, it is possible to split the one fine grid with 266 points into 4 coarse grids in different ways. The results of split grid should be that the measurement points are equally spreading around the sphere in the favour of the test procedure can be easily design for facilitate the OTA test.

One example of splitting 266 fine grids into 4x64 interpolated grids is illustrated in Figures A.3.3.4-1 and A.3.3.4-2.

Figure A.3.3.4-1: 266 measurement grid point for TRP.

Figure A.3.3.4-2: Split 4x64 measurement grids

To achieve the same accuracy of TRP measurement results based on the 4x64-points split grids compared with using 266-points fine grids, we first use the each of 62-points split grids in the OTA testing of TRP according to the procedure in Clause A.3.3. Repeat the OTA test four times to get the TRP1, TRP2, TRP3, and TRP4.

Then, the TRP can be calculated as in the equation below:

TRP=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104)

The calculation of the TRP1/TRP2/TRP3/TRP4 based on EIRP measurements follows Clause A.3.5.1 for TRP.

Similarly, using the same method to measure TRS by using four split grids in the OTA test which follows Clause A.3.5.2 for TRS1 through TRS4. Then the TRS can be calculated as in the equation below:

TRS=10log4/(110TRS110+110TRS210+110TRS310+110TRS410

<<< END OF CHANGES >>>