3GPP TSG-RAN WG4 Meeting #114												 	   		R4-2502586

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.4.5

Source:	Intel Corporation

Title:	WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2

Document for:	Approval

# Topic #1: Core RRM requirements

## Agreements summary for topic #1

Issue 1-1-1: Removal of brackets around N2 = [6] in Table 4.4.2.2-1 Note 2

- Proposals

  - Option 1: Remove brackets so N2 = 6.

  - Option 2: N2 equals other values.

- Recommended WF

  - Agree on option 1.

  - Big CR should be revised to reflect the change.

Agreement:

  - Remove brackets so N2 = 6.

Issue 1-1-2: Conditional handover including target MCG and candidate SCG delay requirements

- Proposals

  - Option 1: Update conditional handover including target MCG and candidate SCG delay requirements in 6.1.7.1.2 the same way as in other PSCell requirements: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

- Recommended WF

  - Agree on option 1.

  - Big CR should be revised to reflect the change.

Agreement:

  - Update conditional handover including target MCG and candidate SCG delay requirements in 6.1.7.1.2 the same way as in other PSCell requirements: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

# Topic #2: Performance RRM requirements

Sub-topic 2-1 Test cases list

Issue 2-1-1: Discuss based on the below table of test cases list and endorse in this meeting the list and work split among companies

Agreement on the Test case list:

Table 1 Test cases list for less than 5MHz CA/DC

| No. | Item of core requirements | Type of test cases | FR | Subclause | Note | Company |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | SCell activation delay | SCell activation and deactivation of SCell is with 12 PRB SSB bandwidth | FR1 | A.6.5.3.X | Discuss test case setup based on draftCR in the next meeting. ((un)known, DRX cycle, measCycle) | Ericsson |
| 2-1 | ((Subsequent) Conditional) PSCell addition/change delay | Addition and release delay of PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.11.X | The UE is required to pass only one set of test cases among 2-1, 2-2, 2-3 and 2-4. | Ericsson |
| 2-2 |  | Inter-frequency subsequent CPA to PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.12.X1 |  | ZTE |
| 2-3 |  | Intra-frequency subsequent CPC to PSCell with 12 PRB SSB bandwidth | FR1 | A.6.5.12.X2 |  | ZTE |
| 2-4 |  | Handover with PSCell delay | FR1 | A.6.5.12.X3 |  | Intel |
| 3-1 | Deactivated SCell/PSCell measurements | SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB | FR1 | Modify A.6.6.1.9 | Modify existing test cases. Tested together with neighbour cell measurements. | Nokia |
| 3-2 |  | SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used | FR1 | Modify A.6.6.2.13 |  | Huawei |
| 3-3 |  | Deactivated PSCell measurements with 12 PRB SSB on the target PSCell | FR1 | A.6.6.1.X | New test case is introduced. | Intel |
| 4-1 | IDLE mode CA/DC measurements | SA IDLE mode CA/DC measurements on target SSB with 12 PRB bandwidth | FR1 | A.6.6.9.X |  | Huawei |