3GPP TSG-RAN WG4 Meeting #114	R4-2502593

Athens, GR, February 17th ‒ 21st, 2024

Title: 	WF on RRM requirements for NR_RRM_Ph5_Part2

Agenda Item:	7.17.4

Source: 	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [114][218] RRM requirements for NR_RRM_Ph5_Part2.

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

Agreement in ad-hoc session:

- Discuss if additional conditions are needed to activate FBS-based L3 measurements.

  - Do not consider additional conditions beside the agreements from RAN4#113 meeting.

Agreement in RRM session (Monday):

- Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold.

  - Further discuss whether to limit the serving cell to FR2 serving cell.

  - Further discuss whether to add the hysteresis and timeToTrigger parameters.

  - Note: the scenario related to single carrier or CA/DC is discussed in another issue.

- The signaling design is up to RAN2.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement in ad-hoc session:

- UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4.

Further discuss this issue.

Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC

Agreement in RRM session (Monday):

- No need to add additional processing time in the delay requirement for scenario 5.

Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Further discuss this issue.

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Further discuss this issue.

Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement

Agreement:

- Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

Sub-topic 1-3 CR split for FBS L3 measurement

| CR No. | Scenario | Clauses impacted | Volunteering company |
| --- | --- | --- | --- |
| 1 | 1. SSB based Intra-frequency measurement without MG | 9.2.5.1, 9.2.5.2 | CATT |
| 2 | 2. SSB based Intra-frequency measurement with MG | 9.2.6.2 | Apple |
| 3 | 2. SSB based Intra-frequency measurement with MG | 9.2.6.3 | MTK |
| 4 | 3. SSB based Inter-frequency measurement without MG | 9.3.9.1, 9.3.9.2 | Huawei |
| 5 | 4. SSB based Inter-frequency measurement with MG | 9.3.4, 9.3.5 | CMCC |
| 6 | 5. Handover | 6.1.1.4.2, 6.1.1.5.2 | Nokia |
| 7 | 7. RRC Re-establishment | 6.2.1.2.1 | ZTE |
| 8 | 7. RRC Connection Release with Redirection | 6.2.3.2.1 | OPPO |

Topic #2: Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: How to capture the agreements in spec

This issue has been resolved in CR discussion.

Issue 2-1-2: UE capability

Postpone this issue to the end of the core part.

Issue 2-1-3: Definition of Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP

This issue has been resolved in CR discussion.

References

- R4-2500532, Topic summary for [114][218] NR_RRM_Ph5_Part2, CATT, RAN4#114.

- R4-2502576, Ad-hoc minutes for NR_RRM_Ph5 WI, Apple, RAN4#114.