# 3GPP TSG-RAN WG4 Meeting #114	R4-2502598

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.25.3

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Introduction

Note: The detail open issue list is captured in the moderator summary R4-2502578, R4-2500537.

# Topic #1: On-demand SSB(OD-SSB) requirements

# OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

  - The agreements in last meeting are as follow.

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

- Sub-issue 1-1 – The length of FMW

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

Agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

- The activation and deactivation of OD-SSB is fully up to NW.

# OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

  - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

RAN4 already agreed to support scenario 1.

In last meeting, RAN1 sent the reply LS to RAN4 as follow.

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

Issue 2-3: Combine OD-SSB and AO-SSB in measurement

Agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

# OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Sub-issue 3-1: Lower bound for measurement in FMW

- Background

  - The agreement in last meeting is as follow.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

Wayforward:

- Option 1: RAN4 not to define lower bound to measurement in FMW.

- Option 2: Define a lower bound with OD-SSB periodicity as [10ms].

  - The lower bound is only applied to measurement in FMW.

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers

Agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

- Option 1: The CSSF only counts OD-SSB layers

- Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

Agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

- FFS multiple OD-SSB carriers

# OD-SSB general issues

Issue 4-1: L1 measurement before SCell activation

Agreement:

- RAN4 not to support OD-SSB based L1 measurements for deactivated Scell.

Issue 4-2: L1 measurement after SCell activation

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Wayforward:

- RAN4 to support OD-SSB L1 measurement after SCell activation.

  - FFS the requirement when DRX is configured.

    - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi)

    - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung)

    - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT)

    - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

Issue 4-3: OD-SSB accuracy

Agreement:

- The legacy measurement accuracy is applied for OD-SSB based measurement.

# OD-SSB based SCell activation

# OD-SSB SCell activation scenarios

Issue 6-1: OD-SSB based direct SCell activation

- Background

RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement:

- RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 6-6: Interruption

Open Issue:

- RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

# Other OD-SSB measurement issues

Issue 7-2: OD-SSB based Neighbour cell measurement

    - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell

Agreement:

  - For Scenario 1a: No RAN4 spec. change.

    - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell

Agreement:

  - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB.

    - The agreement can be revisited if not aligned with the further progress in other WGs.

  - Further discuss for FR2.

    - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell

Agreement:

  - For FR1, follow legacy requirement for neighbour cell measurement.

    - The agreement can be revisited if not aligned with the further progress in other WGs.

  - Further discuss for FR2.

Issue 7-3: OD-SSB based L3 reporting

Wayforward:

- RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.