3GPP TSG-RAN WG4 Meeting # 114				                               R4-2502600

# Athens, Greece, 17 – 21 Feb., 2025

Agenda item:			7.26.5

Source:	Moderator (vivo)

Title:	WF on RRM requirements for NR_LPWUS

Document for:	Approval

Topic #1: Agreement on RRM core requirements for LP-WUS/WUR

- Sub-topic 1-1 General aspects

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence

Agreement:

- In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not.

Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence

Agreement:

- Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS

  - Reuse the same requirement for LP-SS OOK based

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Agreement:

At legacy state,  when and how to turn on LR for serving cell measurement is up to UE implementation.

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Agreement:

When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately.

MR and LR evaluation requirements are defined separately.

Issue 1-2-4-2-2: On single or multiple LR evaluation requirements

Agreement:

Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases.

Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Agreement:

LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y.

Number x and y are defined as measurement requirement as  [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

- Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

- MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

- Sub-topic 1-4 LP-WUR CONNECTED mode

- Sub-topic 1-5 Others

Topic #2: Agreement on simulation assumptions and results

Sub-topic 2-1 On simulation assumptions and parameters

Issue 2-1-1: SINR setting

Agreement:

Include -6 dB SINR in simulation assumptions SINR and -6 dB SINR is low priority and depending on company input;

Treat -0.5dB and 2dB SINR as low priority cases;

Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation

Agreement:

Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

Issue 2-1-6: On SCS in simulation

Agreement:

Using SCS = 30KHz as the base in the simulation

Issue 2-1-7: On OOK interference

Agreement: Define a specific LP-SS binary sequences of the serving and  interference cell for simulation alignment.

Issue 2-1-4: On LP-SS sequence for simulation purpose

Agreement:

For RAN4 simulation alignment purpose:

For M=1, L=4 (if supported), the set of LP-SS binary sequences is:

Serving cell:    [0 1 0 1]

Interference cell: [0 1 1 0]

For M=1, L=6 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 1 0]

Interference cell: [0 1 0 1 0 1]

For M=1, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 1 0 0 1 0 1]

Interference cell: [1 0 1 0 1 0 0 1]

For M=2, L=8 (if supported), the set of LP-SS sequence is:

Serving cell:    [0 1 0 1 1 0 0 1]

Interference cell: [0 1 1 0 0 1 0 1]

For M=2, L=12 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 1 0 0 1 1 0 0 1]

Interference cell: [0 1 1 0 1 0 0 1 1 0 0 1]

For M=2, L=16 (if supported), the set of LP-SS sequence is:

Serving cell:    [1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

Interference cell: [1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0110100110101010]

Interference cell: [0110101010011010]

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

Serving cell:    [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Interference cell: [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

Sub-topic 2-2 Simulation results alignment and summary

Issue 2-2-1: On simulation campaign

Agreement:

delta RSRP and delta RSRQ (the delta RSRP is the largest value of the difference between real RSRP and ideal RSRP at 5% and 95% point)

For simulation alignment purpose, interested companies can also report the “mean” value.

For the number of samples:

It is up to companies whether to provide result for all or some of the numbers of samples.

Topic #3: Open issues

Details on all open issues can be found in [1] and [2].

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Recommendations:

Postpone the discussion until more RAN2 progress

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

Recommendations:

For both issue 1-1-5, 1-1-6 and 1-1-7, criteria are up to RAN2’s consideration and close these issues.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Recommendations:

Further discuss whether LR evaluation requirements apply when LR is “ON” at legacy state.

Issue 1-1-10: Considerations on higher priority frequency layer

Recommendations:

Continue discussion.

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

Recommendations:

Continue discussion on existing proposals.

Issue 1-2-5-2: On MR evaluation requirements

Recommendations:

Continue discussion.

Issue 2-2-1: On simulation campaign

Recommendations:

Using the following table as the base to report simulation results

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 2 |  | X = 3 |  | X = 4 |  | X = 5 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std | mean | std |  |  |  |  |

Reference

[1] R4-2500539, Topic summary for [114][225] NR_LPWUS, vivo, RAN4 114

[2] R4-2502577, 	Ad-hoc minutes for NR_LPWUS WI, vivo, RAN4 114