3GPP TSG-RAN WG4 Meeting # 114															R4-2502614

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.12.4

Source:	Moderator (CMCC)

Title:	Coffee break discussion minutes on NR_ATG_enh

Document for:	Information

Introduction

Topics for Adhoc discussion

Issue 1-1-1 UE antenna type for inter-band CA

Issue 1-2-1 MRTD for ATG inter-band CA

Issue 1-4-4 Deactivated SCell measurement

Issue 1-4-6 Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

Issue 1-4-7 Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

Issue 1-3-1-3 Is there any impact from UE antenna types for interruption requirements

Issue 1-3-1-5 Applicable bands of Interruption requirements

Issue 1-1-2 UE Antenna Type signaling

Issue 1-3-3 SCell Addition/Activation mechanism

Issue 1-4-9 Measurement enhancement for ATG UE with antenna array

Topic #1: RRM core requirements

Issue 1-1-1: UE antenna type for inter-band CA

- Background

Agreement from RAN4#114 RF session:

Agreement:

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

- Proposals

  - Proposal 1: RAN4 RRM to wait for RF conclusion for Case 4 antenna type discussion. (LGE, CMCC, HW, Ericsson)

  - Proposal 2: (ZTE)

    - Pay more attention to the Rx beam assumption instead of antenna assumption, not preclude any possible antenna implementation for inter-band CA. Refine the Case 4 and Case 5 as below and keep both of them:

      - Case 4. ATG UE with an antenna array(s) on the bands of PCC and SCC, and one common Rx beam assumed.

      - Case 5. ATG UE with an antenna array(s) on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

  - Proposal 3: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receiving performance degradation, e.g., conditions for intra-band NCCA operation. (LGE)

- Recommended WF

    - Consider RRM requirements for Case#4 UE.

    - Further refine the description of Case#4 UE and Case#5 UE in the UE capability if needed.

LGE: What to refine?

ZTE: propose to refine to address the concern from RF. RF says based on implementation, open to do the refine, just not to restrict the UE implementation

E///: There are two Options in RF. Case 4 is corner case as stated in Option 2.

ZTE: Don’t decide corner or not. Just follow RF.

HW: Share similar concern from E///. Further check whether is sufficient.

LGE: Not the corner case. Proposed CA combination.

CMCC/CATT: example band n3 and n39

HW: possible is not strong word, is the time sufficient for core requirement definition

LGE: RF has agreed CASE4 is possible.

E///: we are talking about whether RRM requirement will be defined for Case#4 or not

Issue 1-2-1: MRTD for ATG inter-band CA

- Proposals

- Proposals for UE except Case#4:

  - Proposal 1: 33µs (CMCC, CATT, HW, Ericsson)

  - Proposal 2: Not larger than 3µs (ZTE)

- Proposals for UE Case#4

  - Proposal 1: 33µs (CMCC, Ericsson)

  - Proposal 2: at least 3µs (LGE)

  - Proposal 3: Not larger than 3µs (ZTE)

  - Proposal 4: FFS (CATT)

- Recommended WF

  - For the MRTD except Case#4

    - 33µs

  - FFS for the MRTD for Case#4

ZTE/LGE: For case#4, 3µs

E///: 3µs is not enough.

CMCC/HW/E///: Case#4 have 2 Rx chains, 33 is possible

ZTE: actual TAE is much less than 3. To move forward, can compromise to 33.

LGE: the principle of MRTD is TAE and propagation delay, no or minur propagation delay here.

CMCC: 33 also good for non-co-located scenario

LGE: To move forward, can compromise to 33.

Agreement: 33µs for the MRTD of ATG.

Issue 1-4-4: Deactivated SCell measurement

- Background

- Proposals

  - Proposal 1: RAN4 not to consider L3/L1 measurement sharing factor Klayer1_measurement for deactivated SCell measurement (LGE, ZTE)

  - Proposal 2: (CMCC, CATT, [HW])

    - For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

    - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

- Recommended WF

  - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

  - For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

- CATT: agree with Recommended WF, no L1 measurement on deactivated Scell, so no needed for UE except Case4, but for Case 4, UE also need to measure the L1 from PCell.

- E///: Support Recommended WF

- ZTE: For Case 4, will deactivated SCell measurement be impacted by co-located pCELL L1 measurement

- CMCC: deactivated SCell measurement also have neighbour cell measurement. Use measCycleScell to measure the neighbour.

- HW: e.g., when measure the neighbour cell on deactivated serving SCell MO.

- ZTE: FFS the Case#4

- Agreement:

  - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

  - FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

- Proposals

  - Proposal 1: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements: (CMCC, ZTE, CATT)

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

    - When inter-band carrier aggregation is performed:

      - For ATG UE antenna type Case  2, 3 and 5, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

      - For ATG UE capable of [antenna array capability] on either PCC or SCC, and support [new UE capability of Case 4]antenna type Case 4, i.e., ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

      - For ATG UE capable of [antenna array capability] on either PCC or SCC, [but don’t support [new UE capability of Case 4]], the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

- Recommended WF

  - Check whether the principle of Proposal 1 can be agreed, the specific wording can be refined in CR

Tentative agreement:

Principle of Proposal 1 can be agreed, the specific wording can be refined in CR

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

- Proposals

  - Proposal 1: (CMCC, CATT(except for Case#4))

    - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

      - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

      - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

  - Proposal 2: (Ericsson)

    - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

      - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

      - When inter-band carrier aggregation in FR1 is configured,

        - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

        - For ATG UE with the antenna array on PCell and omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

        - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

        - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

        - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

- Recommended WF

  - For ATG UE antenna types except Case 4

    - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

      - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

      - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

  - FFS for ATG Case#4 UE

Tentative agreement:

- For ATG UE antenna types except Case 4

  - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

    - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- FFS for ATG Case#4 UE

Issue 1-3-1-3: Is there any impact from UE antenna types for interruption requirements

- Proposals

  - Proposal 1: Reuse the current interruption requirements, no impact from UE antenna types. (CMCC)

  - Proposal 2: (Ericsson)

    - RAN4 should discuss and capture FR1 antenna array impact on the following Interruptions requirements.

      - Interruptions at SCell addition/release

      - Interruptions at SCell activation/deactivation

      - Interruptions at direct SCell activation

      - Interruptions at fast SCell activation

      - Interruptions during measurements on deactivated SCC

      - Interruptions due to SCell dormancy

      - Interruptions due to UE-specific CBW change

      - Interruptions due to Active BWP switching Requirement

- Recommended WF

  - Recommend the proponents of Proposal 2 to provide more details about antenna array impact

Tentative agreement:

Reuse the current interruption requirements, no impact from UE antenna types.

If any technical issues identified, can be further discussed.

Issue 1-3-1-5: Applicable bands of Interruption requirements

- Proposals

  - Proposal 1: The interruption requirements defined are applicable to the bands: n3, n39, and n79. If new bands have been introduced the interruption requirements need to be redefined. (HW)

- Recommended WF

  - To be discussed

Tentative agreement:

The issue is closed

Issue 1-1-2: UE Antenna Type signaling

- Proposals

  - Proposal 1: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side. (LGE)

  - Proposal 2: RAN4 should wait for the RF session’s decision on Case 4 and send LS to RAN2 to define the capability to differentiate Case 4, and Case 5. (Ericsson, CMCC)

  - Proposal 3: To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed with per BC type for the bands in which UE supporting phase antenna array.  (ZTE)

- Recommended WF

  - Further discuss how to define the UE capability to indicate common or separate Rx beam (i.e., Case#4 or Case#5).

Tentative agreement:

To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed

FFS for the details.

Issue 1-3-3: SCell Addition/Activation mechanism

- Proposals

  - Proposal 1: Consider distance triggered SCell addition (CMCC, HW)

    - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.(CMCC)

    - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. (HW)

  - Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. (Ericsson)

  - Proposal 3: further optimization methods for SCell addition and activation can be considered (CATT)

    - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

- Recommended WF

  - To be discussed

E///: Distance based for mobility in history, but now discussed for SCell activation. However, SCell activation should be fully decided by network, with considering the traffic and Cell quality. SCell will be activated all the time after UE do the SCell activation using the distance based way.

HW: Such approach can only used for Case 4.

Issue 1-4-9: Measurement enhancement for ATG UE with antenna array

- Proposals

  - Proposal 1: (CMCC)

      - If network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement. From requirement point of view, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed.

- Recommended WF

  - To be discussed