3GPP TSG-RAN WG4 Meeting #114	R4-2502619

Athens, Greece, 17 – 21 February, 2025

Title:	LS on UE capability for L1-RSRP measurement in LTM

Response to:

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Yanliang SUN

E-mail Address:	yanliang.sun@vivo.com

Attachments: -

# 1	Overall description

RAN4 has discussed the RAN4 FG 39-3-1, FG 39-3-3, FG 39-3-5 and FG 39-3-6 and achieved the following agreements:

RAN4 FG 39-3-1 <Agreement in RAN4 #114>: - The 1st component of RAN4 FG 39-3-1 does not include the frequency layers which SSB L1-RSRP measurement is not configured on any neighbor cell on. RAN4 FG 39-3-3 <Agreement in RAN4 #114> - RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, ‘total cells of serving cells and neighbouring cells’ counts both LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement, no matter SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. RAN4 FG 39-3-5 <Agreement in RAN4 #114>: - The first component of RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18 counts both LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement, no matter SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. - For RAN4 FG 39-3-5, include the candidate values of 12, 16, 20, 24 to existing candidate values. RAN4 FG 39-3-6 <Agreement in RAN4 #113>: - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

The updated RAN4 UE feature list for above FGs is as follows with revisions marked in red

| Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 39-3-1 | Number of frequency layers for L1-RSRP measurement | - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement. Only frequency layers which are configured with SSB-based L1-RSRP measurement on neighbor cell(s) by LTM-CSI-ResourceConfig-r18 are counted. - The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list and/or 39-2 2. Component 2: 39-2a | 1. Candidate values Component 1: {1,2,3,4,5,6,7,8} 2. Candidate values Component 2: {1,2,3,4,5,6,7,8} | Mandatory with capability signaling if UE supports 45-1, 39-2 and/or 39-2a |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement are counted, regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | 45-1 from RAN1 Rel-18 feature list or 39-2 | candidate values: {1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,22,23,24} Note: when same PCI is present in serving and candidates, one PCI is counted only once in total cells | Optional with capability signaling |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps. LTM candidate cells and serving cells are counted regardless of whether SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list or 39-2 2. Component 2: 39-2a | Candidate value of Component 1: {1,2,3,4,5,6,7,8,12,16,20, 24} Candidate value of Component 2: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. Both LTM candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are counted, regardless of whether SSB-based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | 45-1 from RAN1 Rel-18 feature list or 39-2 | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |

# 2 Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take above information into account and revise the TS 38.306 accordingly.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #114bis	7 – 11 April 2025	Wuhan, China

TSG RAN WG4 Meeting #115	            19 – 23 May 2025	Malta