3GPP TSG-RAN WG4 Meeting # 114			    								               R4-2502825

Athens, Greece, 17th – 21st February 2025

Agenda item:			6.1

Source:	CMCC

Title:	WF on HPUE for single NR bands

Document for:	Approval

Backgroud

In Rel-18, companies conducted research on n26 support for PC2 and the relevant outcomes have all been captured in TR 38.896. Detailed analysis could be found in [1][2][3].

Agreement online

Issue 2-2-2: PC2 REFERENCE for n5

- Proposals

  - Option 1: The sensitivity degradation between PC3 and PC2 should be similar as for n26.  (Ericsson)

  - Option 2: Others.

Agreement:

- The sensitivity degradation between PC3 and PC2, which is RSD and should be similar as for n26.

Way forword

Issue 1: n26 PC2

There are still A-MPR requirements that need to be defined for n26 PC2, specifically NS-12, NS-13, NS-14, NS-15 and RSD.

- Set the analysis (including 3MHz) in the draft CR R4-2503019 as baseline.

- These NS and RSD values need to be defined based on the analysis results from TR 38.896, and any additional relevant input.

Issue 2: A-MPR clean-up

For NS_48 and NS_49, and to account for PC2 2Tx rev-IMD, further study the approach below:

- Void Note 7 in Table 6.2.3.1-1,

- Replace the [2] dB PC2 2Tx A-MPR relaxation with a maximum of 0.5dB relaxation for cases where the specified PC2 (1Tx) A-MPR < PC3 A-MPR +3dB,

- To avoid tightening the Rel-17 and Rel-18 requirements, these changes are applicable to Rel-19 only.

Reference

[1] R4-2402214, PC2 A-MPR for band n26, Huawei, HiSilicon

[2] R4-2315374, A-MPR discussion for FDD HPUE n26, Apple

[3] R4-2407067, A-MPR discussion for FDD HPUE, Apple

[4] R4-2410561, “TP for TR 38.896 to add PC2 for n26”, China Unicom

[5] TR 38.896