3GPP TSG-RAN WG4 Meeting #114	R4-2502871

Athens, Greece, 17-21 February 2025

Title:	LS on Signalling for 7 MHz Channel Bandwidth

Response to:

Release:	Rel-19

Work Item:	NR_FR1_7MHz_BW-Core

Source:	RAN4

To:	RAN2

Cc:	RAN3

Contact Person:

#### Name:	Bill Shvodian

E-mail Address:	bill.shvodian@t-mobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 on the progress made on the topic of release independence for the 7 MHz channel bandwidth.

RAN4 has noted that TS 38.307 says that “NR channel bandwidths, all unless otherwise stated” are release independent from Release 15. RAN4 understands that there were some unique characteristics of 3 MHz channel bandwidths including new RAN1 requirements and signalling related to support for the punctured SSB that prevented the 3 MHz channel bandwidth from being release independent from Rel-15. RAN4 has discussed release independence of the 7 MHz channel bandwidth and have decided that there is no reason from a RAN4 perspective that 7 MHz cannot be release independent from Release 15.

RAN4 also notes that 35 and 45 MHz channel bandwidths were introduced in a Release 17 Work Item, but are release independent from Release 15, including the use of spare bits in 38.306 Release 15. RAN4 believes that adding 7 MHz should be similar to adding 35 and 45 MHz, and as such, can be release independent from Release 15.

For information, RAN4 notes that 7 MHz will neither be the widest channel bandwidth for a given band, not the narrowest channel bandwidth for a given band, since 5 MHz and 10 MHz are mandatory for most NR FDD bands, and all FDD bands for which 7 MHz has been requested or possibly may be added. And for backward compatibility with legacy UEs and networks using 5 MHz, 7 MHz is unlikely to be the minimum or maximum channel bandwidth that a UE supports for a band in a given band combination.

RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15, similar to how 35 and 45 MHz were added.

2. Actions:

To RAN2

ACTION: 	RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15, similar to how 35 and 45 MHz were added.

3. References:

3. Date of Next TSG-RAN4 Meetings:

3GPP TSG RAN WG4#114bis	7 - 11 Apr 2025			Wuhan, China

3GPP TSG RAN WG4#115	19 - 23 May 2025			St. Julian’s, Malta