3GPP TSG-RAN4 Meeting #114					R4-2502874

Athens, GR, 17th – 21st February, 2025

Source:	China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple

Title:	WF on NR PC2 RedCap UE in FDD bands

Agenda item:			7.7.2

Document for:	Approval

Background

This WF captures the agreements for RF requirements for PC2 RedCap UE in FDD bands.

Wayforward

Issue 1-1-1: MPR/A-MPR requirements

Online agreement:

- The 1 Tx MPR/AMPR requirements for FDD PC2 non-RedCap can be reused for the FDD PC2 RedCap UE. No need to define special MPR/A-MPR for FDD PC2 RedCap UE

  - Note: This agreement also applies to HD-FDD PC2 RedCap UE under the condition that, for NS cases where a filter is assumed, the assumptions of the PC3 HD-FDD filter are also valid for PC2 HD-FDD operation in the completed PC2 FDD bands.

Issue 1-2-1: REFSENS for HD-FDD

Online agreement:

- The existing reference sensitivity for RedCap PC3 for HD-FDD in sub-clause 7.3I of TS 38.101-1 can be applicable for RedCap PC2 for HD-FDD.

Issue 1-2-2: REFSENS for 2Rx in FD-FDD

Online agreement:

- 2Rx RSD for 1Tx PC2 is provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1, and this requirement could apply to RedCap UE for channel BWs up to 20MHz.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD

Agreement:

- Further discuss the following formats for further evaluations

  - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - Evaluate the values of ΔR band by band

  - Note: Other options are not precluded

Way forward: It is encouraged for companies to provide analysis for the above options and give the preference in next meeting.