3GPP TSG-RAN WG4 Meeting #114	R4-2502879

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.26.5

Source:	vivo

Title:	WF on UE RF requirements for LP-WUS

Document for:	Approval

Introduction

This is way forward for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General

Issue 1-1-1: How to define two LP-WUR types

Online Agreement:

- Define two set of RF requirements as the baseline, considering the following aspects including

  - Noise figures

  - Interference performance

  - Modulation

- Consider to merge two sets of requirements into one if the requirements are closed to each other

Issue 1-1-3: TP to TR 38.774 general aspects

- Proposals

  - Proposal 1: check and discuss the TPs in R4-2501067, R4-2500755 and R4-2501381, for some general aspects.

- Recommended WF

  - R4-2501067 can be merged into R4-2500755.

  - R4-2501381 for Architecture and testability is agreeable?

  - R4-2500755 for some general aspects and simulation assumption is agreeable?

  - With the understanding that potential update and improvement next meetings is still needed.

Sub-topic 1-2 System parameters

Issue 1-2-1: Whether operation band clause for LP-WUS/WUR is needed (5.2M	Operating bands for LP-WUS)?

Online Agreement:

- Operation band clause for LP-WUS/WUR is needed in Rel-19

  - Keep the band with DL operating band defined, and SDL band is precluded

Issue 1-2-2: Whether Channel bandwidth clause for LP-WUS/WUR is needed (5.3M Channel Bandwidth for LP-WUS)?

Online Agreement:

- [Transmission bandwidth configuration] for LP-WUS/WUR signal is needed to specify.

Issue 1-2-3: Whether Channel raster clause for LP-WUS/WUR is needed (5.4M Channel raster for LP-WUS)?

- Proposals

  - Option 1: Yes.

  - Option 2: No.

- Recommended WF

  - If Yes, how to update the spec

  - If no, it is the common understanding that the channel spacing requirements in clause 5.4.1 apply for each operating band for LP-WUS operation?

  - Collect views and confirm. Check the draft CR R4-2501070

5.4M	Channel arrangement for LP-WUS

5.4M.1	reserved

5.4M.2	Channel raster for LP-WUS

The PRB grid between LR and MR is aligned and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. This alignment ensures that the center frequency of LP-WUS is always a multiple of 10 kHz for NR bands with 100kHz channel raster. How to make sure the RB alignment between LP-WUS RB and MR RB grid is aligned is UE implementation specific.

Issue 1-2-4: Whether a LS on Channel raster related issues to RAN2 is needed?

- Proposals

  - Option 1: Send an LS to RAN2 inquiring how a UE in RRC_IDLE mode obtains the information on the centre frequency of the LP-WUS assigned to this UE. (CATT)

  - Option 2: No LS to RAN2 for now.

- Recommended WF

  - Check and discuss whether the LS is needed, align RAN4 understanding on this issue

Moderator:

In RAN2#126 meeting, it was agreed:

  - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

-	LP-SS configuration

-	LP-WUS configuration

-	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Issue 1-2-5: TPs to TR 38.774 on system parameters

- Proposals

  - Proposal 1: check and discuss proposed TPs.

- Recommended WF

  - TP R4-2501069 from CATT is agreeable?

Topic #2: REFSENS, ASCS and ACS requirements

Sub-topic 2-1 FR1 SNR, NF and IM for REFSENS requirements

Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR

Online Agreement:

- For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NR values are

    - Set 1: [17.5~18dB]

    - Set 2: [13.5dB]

Ad-hoc discussions:

Nokia: we suggest to update agreed IM+NF as set 1: [17]dB

Huawei: this is decided in online session.

Moderator: can we go with the following SNR value?

Set 1: SNR is about -2.5dB.

Set 1: SNR ~ 3.5dB.

QC: suggest to send an LS to RAN1 on RAN4 outcome of IM+NF. The information is just for checking by RAN1.

E///: this information is important for RAN1. Not so clear how to design test configuration in RAN4 for now. SNR in RAN4 RF is mainly decided by test configuration, maybe related to RAN1 design but not too much.

Huawei: not everything should be fully decided by RAN1. Undetermined part has less impacts on RF performance. We need tentative agreements in RAN4. Can be revisited if RAN1 has some agreements.

QC: Some details of waveform design may have impact on RF SNR.

Nokia: For OFDM-based LP-WUR the SNR has a bit large gap among companies.

Sony: Our simulation show that the results is related to waveform design, several dBs difference

CATT: If RAN1 need to check coverage, they can ask us.

E///: RAN2 also need to check the behavior of LP-WUS.

Agreements in ad-hoc:

- Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB)

- Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS.

- LS should also cc RAN2

Issue 2-1-3: Timing error impacts for OOK-based and OFMD-based SNR

- Proposals

  - Proposal 1: RAN4 consider timing error impact (e.g., 1dB) on LP-WUR performance degradation. (vivo)

- Recommended WF

  - Discus and confirm timing error impacts on LP-WUR

  - Check whether the timing error impacts should be considered in IM, or NF?

Ad-hoc discussions:

E///: maybe this is not related to RF test. From test perspective, timing error is presented or not?

Huawei: we also see the timing error impacts on RF performance. We suggest increasing the agreed IM+NF value. RAN4 should consider it. No need to ask RAN1 feedback.

Nokia: Channel design in RAN1 already consider this impact. No need in RAN4.

Tentative agreements in ad-hoc:

- FFS whether ask timing error value in the LS to RAN1.

- Whether consider timing error in RAN4 RF simulation can be contribution driven.

Sub-topic 2-2 FR2 SNR, NF and IM for REFSENS requirements

Issue 2-2-1: Combining strategy for FR2 LP-WUR

Ad-hoc discussions:

E///: RAN4 should assume the gain is 0 as combination gain in the REFSENS. We can consider it in future release.

Samsung: OK with WF. Gain should be -3dB. Compare with MR 0dB agreements.

QC: also think -3dB. Worst case of the linear antenna at different polarization.

- Agreements in ad-hoc:

  - For rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation.

  - Confirm 1Rx at baseband for FR2 LP-WUR.

Issue 2-2-2: NF for FR2 LP-WUR

Recommended Way forward for checking:

  - Adopt similar approach to FR1 LP-WUR, encourage companies to input analysis on IM, NF and SNR together.

Issue 2-2-3: Spherical coverage for FR2 LP-WUR

Recommended Way forward for checking:

  - RAN4 further discuss how to specify spherical coverage requirements for FR2 LP-WUR.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: Required number of guard RB for ASCS

Ad-hoc discussions:

QC: maybe we can consider two sets of requirements for this.

ZTE: this has impacts on Tx , for BS, we think one set guard RB should be specified.

Nokia: Two requirements can be based on UE capability.

Huawei: Two sets requirements is not required for ASCS. Single value should be specified.

CATT: one set should be final target.

Apple: 1RB as guard RB should be specified for both types.

Moderator: last meeting agreed [0] guard RB as starting point, this can be final confirmed in issue 2-3-2. With consideration of de-sense of wanted signal, check whether 0 RB can be confirmed in issue2-3-2.

Issue 2-3-2: in-band power level of ASCS Test case

- Proposals

  - Option 1: The inband WUS and NR are same power level as LP-WUS REFSENS.

  - Option 2: The inband WUS and NR are [x]dB higher that LP-WUS REFSENS, consider an implementation margin.

    - REFSENSLP_WUR+0.5dB (vivo)

    - REFSENSLP_WUR + 14 dBm (nokia)

  - Option 3: NR signal power level (Ericsson),

    - REFSENS + 10log (# RB of NR signal / # of RB of LP-WUS) + [x dB]

    - ASCS can be tested with both small adjacent interferer and big interferer case with selectivity of 0 dB.

- Recommended WF

  - Check whether 0Db/0.5dB/14dB higher power is sufficient. Also related to issue 2-3-1

Ad-hoc discussions:

Huawei: REFSENS+14dB is far away from the wanted signal condition of this requirement case.

Nokia: we can also consider 3dB.

Agreements in ad-hoc:

- For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB].

- 0 guard RB is the baseline

Issue 2-3-3: channel bandwidth of ASCS Test case

- Proposals

  - Option 1: all CBW of a NR band.

  - Option 2: single configuration: 11RBs for WUS and 11 RBs for NR. (Nokia)

  - Option 3: other

- Recommended WF

  - Discuss whether a typical CBW (10MHz/20MHz) is sufficient to verify ASCS performance

Ad-hoc discussions:

Huawei: as core requirements, ASCS should support all CBW. For test purpose, low/mid/high cases can be considered.

QC: agree with Huawei, core requirements apply to all CBW.

Nokia: no need to test all CBW, the number of LP-WUS RB is fixed.

Agreements in ad-hoc:

- For ASCS core requirements, it should be applied to all NR CBW.

- For conformance test, for NR CBW can be further discussed and down-selected.

- For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any).

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for OOK-based LP-WUR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for OOK-based LR.  (Moderator)

|  | vivo | CMCC | ZTE | Huawei | Nokia | E/// | Sony |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACS (dB) | 20-23 | 22-24 | 22 | Note1 | 22 | Note 2 | Note3 |
| Guard RB | 0-1RB | 1RB | 0RB |  | 2RBs |  |  |

Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Note 2: The ACS interferer shall be set the same with MR and wanted signal is set as WUR REFSENS plus 14 dB for ACS test requirement. The ACS depends on LR REFSENS difference compared with MR.

Note 3: The ACS interferer shall be set the same with MR and wanted signal is set as WUR REFSENS plus higher than 14dB for ACS test requirement. The ACS depends on LR REFSENS difference compared with MR, and relaxed wanted signal.

Ad-hoc discussions:

Huawei: for one set requirement, it would be same as MR interference. But for the other set, this could be different level.

QC: we think same interference level, but the guard RB can be different.

Sony: ACS is linked with the REFSENS level

E///: same interference is assumed, we can discuss different guard RBs and wanted signal level.

Tentative agreements in Ad-hoc

- Agree same interference signal level as MR for LP-WUR ACS requirements

  - [FFS at least for one set of Rx requirements.]

- The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point. Further discuss whether to relax the wanted signal of LP-WUS REFSENS + [>14]dB;

  - FFS whether two sets of LP-WUS wanted signal level can be defined.

- Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation

- FFS number of guard RBs.

Issue 2-4-2: Baseline ACS requirements (same interference) and required guard RB for OFDM-based LP-WUR

- Proposals

  - Option 1: same approach as OOK-based, based on conclusion in issue 2-4-1;

  - Option 2: other?

- Recommended WF

    - Option 1

Recommended Way forward for checking:

- For the same interference level condition, the approach on ACS is same as OOK-based.

Issue 2-4-3: Optional ACS requirements (lower interference level) and required guard RB for OOK-based LP-WUR

- Proposals

  - Option 1: 10dB ACS, and only consider case 1 ACS test parameter for LP-WUR type (aimed at ultra-low power consumption). (Huawei)

  - Option 2: other

- Recommended WF

    - TBD

Recommended Way forward for checking:

- FFS on how to specify set2 ACS.

Issue 2-4-3: Case 1 ACS test parameter

- Proposals

  - Proposal 1: BW of the interferer i.e. BWinterferer as 5 MHz.. (Nokia)

  - Proposal 2: only case 1 ACS is required. (Huawei)

- Recommended WF

    - Discuss whether a single test case, e.g., CBW, is sufficient to verify LP-WUS ACS requirements

    - Discuss whether only case 1 ACS is sufficient.

Recommended Way forward for checking:

- Confirm BW of the interferer i.e. BWinterferer as 5 MHz for LP-WUS ACS test case.

- Only case 1 ACS is sufficient for LP-WUR.

Issue 2-4-4: Case 2 ACS test parameter

- Proposals

  - Proposal 1: whether case 2 ACS is also required to LP-WUR.

- Recommended WF

  - TBD

Topic #3: Other RF requirements

Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF

Issue 3-1-1: IBB, OBB requirements

- Proposals

  - Option 1: the interference levels for IBB could be relaxed compared to the values defined for MR (at least for the LP-WUR type which targets ultra-low power consumption). OBB could be kept unchanged at least for LP-WUR type which targets moderate power consumption, 6dB relaxation of the interferer level for ultra-low power consumption LP-WUR. (Huawei)

  - Option 2: In-band blocking requirement for legacy UE could be reused. the OBB requirement for legacy UE in TS 38.101-1 could be reused. (CMCC, CATT, Nokia, ZTE, E///)

    - Agree to use larger signal power in blocking test cases compared to MR blocking test cases. (Nokia)

- Recommended WF

  - Discuss and conclude high-level common understanding on whether IBB and OBB can be relaxed. This also depends on how to define LP-WUR types.

Recommended Way forward for checking:

- The interference level is same as MR case, the wanted signal in blocking test case can be relaxed, FFS detailed value.

Issue 3-1-2: detailed table for test parameters of IBB and OBB test case

- Proposals

  - Proposal 1: further discuss detailed test parameters table and how to modify spec, after concluding IBB and OBB requirements value and guard RBs. (Moderator)

- Recommended WF

  - TBD

Recommended Way forward for checking:

- Encourage companies to provide input and analysis on IBB and OBB requirements and whether guard RB is needed.

Issue 3-1-3: NBB requirements

- Proposals

  - Proposal 1: Due to proximity of the NBB to LP-WUS signal and worse phase noise for LR, relaxation of the interferer level should be considered at least for the LP-WUR type aimed at ultra-low power consumption. (Huawei)

- Recommended WF

  - TBD

Issue 3-1-4: Intermodulation requirements

- Proposals

  - Proposal 1: specify two sets of requirements for OFDM signal and OOK signal. (CMCC)

- The requirements in TS 38.101-1 for legacy UE could be reused for OFDM signal.

- New intermodulation requirements need to be specified for the OOK signal.

  - Proposal 2: If the blocking interferer is relaxed to accommodate the low power design for LP-WUR type which targets ultra-low power consumption, intermodulation requirement could also be relaxed for LP-WUR. (Huawei)

- Recommended WF

  - Discuss and conclude first whether requirements can be reused for OFDM-based.

  - Further discuss how to handle OOK-based.

Issue 3-1-5: detailed table for test parameters of Intermodulation requirements test case

- Proposals

  - Proposal 1: further discuss detailed test parameters table and how to modify spec, after concluding Intermodulation requirements value and guard RBs. (Moderator)

- Recommended WF

  - TBA

Issue 3-1-6: FR2 OOB and requirements

- Proposals

  - Proposal 1: No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR. (Samsung)

- Recommended WF

  - TBA

Sub-topic 3-2 Other Rx requirements

Issue 3-2-1: spurious response requirements for FR1 LP-WUR

- Proposals

  - Option 1: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR, and it should be relaxed at least for the LP-WUR type which targets ultra-low power consumption. (Huawei)

  - Option 2: Reuse the spurious response requirements from TS 38.101-1 for LP-WUR. (ZTE, Nokia)

- Recommended WF

  - whether spurious response can be relaxed depends on how to define LP-WUR types

Issue 3-2-2: Maximum input level requirements for FR1 LP-WUR

- Proposals

  - Proposal 1: -25dBm. (ZTE, Nokia)

  - Proposal 2: -26.5dBm. (CATT)

- Recommended WF

    - TBD

Recommended Way forward for checking:

- Confirm -25dBm?

Issue 3-2-3: Maximum input level test case for FR1

- Proposals

  - Proposal 1: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs. (Qualcomm)

- Recommended WF

    - Confirm P1?

Issue 3-2-4: spurious response requirements for FR2

- Proposals

  - Proposal 1: No need to specify spurious response requirements for FR2 LP-WUR. (Samsung)

- Recommended WF

  - TBD

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework for LP-WUR testability

- Proposals

  - Proposal 1: confirm last meeting tentative agreements: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

    - MR could be (or should be?) switched to sleep mode and wakened up to send the test statistics to TE at the end of the test

- Recommended WF

  - Discuss and confirm P1

Issue 4-1-2: Separate RF test case for idle and connection mode

- Proposals

  - Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different and the LP-WUS signals could be different. (Huawei)

  - Proposal 2: LP-WUS is tested only in RRC_CONNECTED state. (CATT)

  - Proposal 3: If the new test mode that does not need to wake up MR is possible in terms of implementation, we also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode. (ZTE)

  - Proposal 4: RAN4 can wait for RAN1 progress on LP-WUS feature design and further decide which RRC state can be selected for RF performance verification. (vivo)

  - Proposal 5: RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing. (Samsung)

  - Proposal 6: Test under IDLE mode can be taken as a baseline since this is likely to be the most important use cases for Rel-19 LP-WUS. (Sony)

- Recommended WF

  - Depends on RAN1 UE feature design. whether all UE types support IDLE and/or CONNECTED mode?

Recommended Way forward for checking:

- RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?

Issue 4-1-3: UE test mode for LP-WUR

- Proposals

  - Option 1: Conclude in RAN4. (Samsung, vivo)

    - RAN4 to fully study and complete the testability aspects rather than to leave to RAN5.

  - Option 2: whether need UE test mode could be left to RAN5. (Huawei, Nokia, Ericsson)

    - Test mode shall support RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states. (Nokia)

- Recommended WF

  - Discuss whether Test mode is feasible to support all RRC state?

Issue 4-1-4: whether a LS to RAN5 on test issue

- Proposals

  - Option 1: Send a LS to RAN5 when the test framework is agreed and leave the test mode discussion to RAN5. (E///)

- Recommended WF

  - Can be on hold until RAN4 have clearer views on testability solutions and RAN1 design

Issue 4-1-5: Test configurations for all Rx requirements

  - Proposal 1: Test configurations in Table 1 could be considered as starting point for further discussion. (Huawei)

- Recommended WF

  - Collect feedback from companies.

Recommended Way forward for checking:

- Consider the test configuration in table 1 of R4-2500160 as a starting point for further discussion.

Issue 4-1-6: detailed Test procedure

  - Proposal 1: RAN5 defines the detailed test procedure for the conformance tests. A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested. (Nokia)

    - RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

  - Proposal 2: The detailed test implementation can also be left to RAN5. (ZTE)

  - Proposal 3: At least 1000 samples are needed to ensure the statistics of 1% MDR as RF performance verification. (vivo)

- Recommended WF

  - TBA