# 3GPP TSG-RAN WG4 Meeting #114		R4-2502884

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.2.4

Source:	Ericsson

Title:	TP to TR 38.922 Editorial corrections for Additional Information on AAS

Document for:	Approval

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. RAN WG4 has already replied for the 4400-4800 MHz frequency range [2], for the 7125-8400 MHz frequency range [3] and for the 14800-15350 MHz frequency range [4] with the recommended IMT technology related parameters.

This text proposal provides editorial updates to clauses of Additional Information on AAS.

# 2	TP

<Start of TP>

# 2	References

[24]	3GPP TR 38.817-1: “General aspects for User Equipment (UE) Radio Frequency (RF) for NR” V16.4.0, September 2022.

[25]	3GPP TR 38.820: “Study on NR: 7-24 GHz frequency range” V16.1.0, March 2021.

[26]	3GPP TR 37.840: “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station” V12.1.0

[27]	3GPP TR 36.897: “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE. Status: Under change control” V13.0.0.

[28]	ITU-R Recommendation M.2101: " Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies"

[29]	3GPP TR 38.803: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[30]	R4-2418397: “TP to TR 38.922: Addition of MIMO modelling simulation results in subclause 7.3”, 3GPP RAN4#113

[31]	R4-2418608: ” Views on Additional AAS aspects related to spatial beamforming”, 3GPP RAN4#113

[32]	R4-2419238: ” Simulation methodologies and results of AAS Modelling”, 3GPP RAN4#113

[33]	R4-2417545: ” TP for other issues (MIMO Beamforming) in TR 38 922”, 3GPP RAN4#113

# 7	Additional information on AAS

## 7.1	Array antenna model

### 7.1.1	Overview

A parameterized array antenna model has been developed over time in 3GPP. The technical background and modelling aspects relevant for AAS base stations was originally described in TR 37.840, subclause 5.4.4 [26]. The model has been used for numerous studies in RAN4, including AAS, NR, HST, IAB, etc. The model has been adopted in other forums outside RAN4 and is also described in RAN1 in e.g., TR 36.897 [27] and in ITU-R in recommendation M.2101 [28]. The extended model including sub-array structures is documented in TR 38.803, subclause 5.2.3.2.4. [29].

The model is defined around a set of equations, which rely on a set of input parameters to describe the array antenna. At a high level the model can be described as shown in Figure 7.1.1-1.

Figure 7.1.1-1: Array antenna model overview

Parameters can be divided into different categories:

-	General parameters, which are parameters that will be required for the simulator (spatial angles, considered wavelength).

-	Element parameters used to model the radiating elements.

-	Sub-array parameters used to model the sub-array.

-	Array parameters used to model the array.

The model will produce gain normalized radiation pattern for given q, j angles defined in the range 0≤θ≤180 degrees and -180≤φ≤180 degrees.

The wavelength, ld is related to the design of the array and is fixed by design, while the excitation wavelength, le may vary as function of considered frequency within a specific operating band.

### 7.1.2	Parameters

The input parameters required to describe the antenna is summarized and described in Table 7.1.2-1.

Table 7.1.2-1: Model input parameters

| Category | Parameter | Description | Note |
| --- | --- | --- | --- |
| General | ld | Design wavelength for array antenna in meters | The wavelength is fixed and selected for a given design frequency. The wavelength will not vary within a given operating band for a given design. |
|  | le | Array excitation wavelength in meters | This wavelength varies within the considered band. |
|  |  | Vertical angle in degrees |  |
|  |  | Horizontal angle in degrees |  |
|  | Ptx | Total conducted power in dBm | For a dual polarized antenna, the total conducted power is calculated over MxNx2 ports |
|  | mech | BS mechanical down-tilt angle in degrees | The mechanical down-tilt angle is a deployment parameter selected to maximize coverage within a given coverage area. The mechanical tilt can be applied as a coordinate transformation as described in TR 36.814, subclause A.2.1.6.2. |
| Element | Am | Element front-to-back ratio in dB |  |
|  | SLAv | Element side-lobe suppression in dB |  |
|  | 3dB | Element horizontal half power beamwidth in degrees |  |
|  | 3dB | Element vertical half power beamwidth in degrees |  |
|  | GE,max | Element peak gain in dBi | This parameter is related to the selection of 3dB, 3dB and LE, where GE,max=DE,max-LE and DE,max is related to 3dB, 3dB and array lattice unit area for the element. |
|  | LE | Element loss in dB |  |
| Sub-array | Msub | Number of element rows in sub-array |  |
|  | dv,sub | Vertical element separation in sub-array in meters |  |
|  | subtilt | Electrical pre-set sub-array down-tilt angle in degrees | The pre-set sub array down-tilt is a fixed design parameter for a base station. It is envisaged as a passive fixed (non-varying) electrical tilt within the sub-array elements. The deployment configuration (including mechanical tilt) for a base station is dependent on the environment. Thus, a same base station with fixed pre-set subarray tilt could also be used across different environments. The value of the pre-set tilt is determined based on the intended deployment scenarios and targeted user distribution and coverage range. |
| Array | M | Number of sub-array rows in array |  |
|  | N | Number of sub-array columns in array |  |
|  | dh | Horizontal sub-array separation in array in meters |  |
|  | dv | Vertical sub-array separation in array in meters | This parameter is related to Msub and dv,sub. |
|  | etilt | Electrical beam direction down-tilt angle in degrees |  |
|  | escan | Electrical beam direction scan angle in degrees |  |
|  | r | Array excitation correlation factor | For wanted signal ρ=1, while for unwanted emission the correlation falls of as function of frequency offset from carrier within the interval 0≤ρ<1. |

The total conducted power, Ptx at the Transceiver Array Boundary (TAB) is related to Total Radiated Power (TRP) in logarithmical scale as:

Ptx=10log102MN+Pn

TRP=Ptx-LE

Where, Pn is the power per transmitter branch in dBm. Total conducted power is defined as the total power for all ports, including two orthogonal polarizations.

The equivalent isotropic radiated power (EIRP) is calculated in dBm using the model as:

EIRPθ,φ=Ptx+AAθ,φ

The peak EIRP can also be derived from parameters as:

EIRP0=Ptx+GE,max+10log10Msub+10log10MN

When co-existence is evaluated in RAN4, the focus is on the main beam pointing towards the UE, but for other compatibility scenarios, e.g., with other services, the focus is not necessarily just the main beam. Spatial regions outside the main beam may also be highly relevant. Since the sub-array topology will affect the radiating characteristics in the sidelobe region, the model needs to be extended to provide the ability to model the sidelobe region characteristics correctly with reasonable complexity.

For the case where single element array geometries is considered (Msub=1), the extended array model collapses to the original model described in TR 37.840 [26].

The antenna element separation is an important parameter that must be selected with care. Obviously, the value is static for a specific base station design, whereas the compatibility analysis may need to cover an entire frequency band. Typically, the antenna is designed to support given element separations for the highest frequency to avoid grating lobes for lower frequencies, but other design principles can be used for base stations supporting multiple bands or very wide operating bands.

Some parameters, such as the element beamwidths are related to the peak element gain via the element loss. This means that these parameters must be selected carefully.

Considering base stations are optimized for various factors including performance, cost, and coverage, it is expected that sub array configurations are relevant for these bands as well as a set of physical antenna elements are combined to form a logical element. The model comprises of a basic element pattern which is then combined appropriately based on the equations to form the sub array pattern and the composite pattern. Since dual polarized elements are used in typical base stations, it is sufficient to model each polarization separately as considered in the specific model. The models are selected so that they are simple and representative to model base station performance with sufficient confidence. The element pattern is based on a simple gaussian beam which has a flat sidelobe level. The Gaussian pattern is sufficiently wide and cover most of the regions of interest, especially in the elevation domain. At high elevation angles, the flat sidelobe level is sufficient to model the side lobes of the antenna element which are significantly lower than the main beam. Thus, the proposed extended antenna model to model sub arrays is sufficient to model the beamforming capability of IMT base stations in considered frequency ranges.

Another aspect also to consider is the performance and coexistence simulator complexity. The antenna model provides is reasonable complex and produce a gain normalized radiation pattern. If parameters are selected properly, no additional directory normalization is required which will reduce complexity and save processing capacity and simulation time.

The antenna model has support to model the array response outside the wanted carrier bandwidth, within adjacent channels using the correlation factor, r. When the wanted signal is considered r is equal to 1. For unwanted emissions outside the wanted signal bandwidth values within the range 0≤ρ<1can be considered. Further details on the frequency response of r require further investigations.

### 7.1.3	Model equations

The array antenna model is built around pattern multiplication between the element radiation pattern, sub-array array factor radiation pattern and array factor radiation pattern, as described by equations listed in Table 7.1.3-1.

Table 7.1.3-1: Array antenna model equations

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλdsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλdcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλesinθetilt-n-1dhλecosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1 , where vm,n=expj2πm-1dvλdcosθ+n-1dhλdsinθsinφ |

## 7.2	MIMO modelling

### 7.2.1	Simulation methodologies

#### 7.2.1.1	Methodology 1

The following simulation methodology was adopted to produce simulation results:

1.	For the k-th UE, generate array coefficients, Ukn,m, to reflect the UE angles from the BS antenna array in both azimuth and elevation (NOTE 1): 

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians. The size of the matrix Ukn,m is N×M, where N is the number of rows and M is the number of columns of the antenna array.

2.	Assess the angular separation between the K UEs in the cell (NOTE 2).

3.	For each UE (NOTE 3), if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated K×MN channel matrix, Hk (NOTE 4):

Hk=hkT…hpTT

where T is the transpose operator and hk corresponds to the vectorization of Uk resulting in a vector of size MN×1 (NOTE 5).

4.	For each UE, the zero-forcing beamforming (precoding) matrix Wk (MNxK) is:

Wk=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of RX antennas (i.e., UEs) and its number of rows corresponding to the number of TX antennas (BS antenna ports), and H is the conjugate transpose operator.

1.	The entire Wk matrix can be normalized by diving it by its Frobenius norm, or, doing a column wise normalization by diving each column with its norm (NOTE 6).

2.	The actual precoding weights for each UE (wk) are obtained by selecting the first column of Wk. The size of wk is MN×1.

3.	Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in:

Akθ,φ=AEθ,φ+10log10m=1Mn=1Nwkn,m∙vkn,m2

where wk refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector. Note that for AAS with sub-arrays, AE(θ,ϕ) is updated by Asub(θ,ϕ):

Akθ,φ=Asubθ,φ+10log10m=1Mn=1Nwkn,m∙vkn,m2

where Asub(θ,ϕ) calculation follows the method defined in the extended AAS modelling supporting vertical sub-array configurations.

1.	The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,φ=10log10k=1K10Akθ,φ10

where AZF(θ,ϕ) refers to the composite ZF beam pattern and Ak(θ,ϕ) refers to the k-th individual zero forcing beam pattern.

NOTE 1: For the computation of Uk, θk and φk are in Local Coordinate System (LCS) which accounts for the mechanical down-tilt.

NOTE 2: The total 3D angular separation should be considered to determine the fulfillment of the minimum angular separation requirement. Additionally, only the azimuth angular separation may be considered in cases where the elevation angle does not significantly contribute to the total 3D angular separation.

NOTE 3: Hk must be constructed for each UE separately, since in some instances, one UE can be paired with all other UEs while another UE could be paired with only a single other UE. The next figure depicts this situation: UE1 can be paired with UE2 and UE3 since the min. angular separation is fulfilled (as shown in (a)), UE2 can only be paired with UE1 (as shown in (b)), and UE3 can only be paired with UE1 (as shown in (c)).

NOTE 4: Hk only includes the UEs that fulfill the minimum angular separation requirement depending on the size of the BS antenna, so Hk may have two or three columns (K=2 or 3) in each Monte Carlo snapshot depending on the geometry of the BS and the UEs. For the UEs that cannot be paired with any other UEs (K=1), Hk is not constructed, and the current beamforming model is used.

NOTE 5: The vectorization must ensure that the vector of size 1×MN correspond to the concatenation of the rows of Uk:

u11⋯u1m⋮⋱⋮un1⋯unm→concatenation→u11…u1mu21…u2m…un1…unm

NOTE 6: The total power constraint simplifies the design problem and leads to simple and efficient precoders. However, the magnitude of some elements of wk may be larger than 1/(MN), which can introduce per-antenna port transmit power larger than 1/(MN). In practice, many systems are subject to individual per-antenna port power constraints.

#### 7.2.1.2	Methodology 2

##### 7.2.1.2.1	Beamforming equations

An alternative to a single-user MIMO (a.k.a. point-to-point MIMO) system is MU-MIMO in which an antenna array at the mobile base station simultaneously serves a multiplicity of autonomous user equipments (UEs). These UEs can be single-antenna devices, in which case the multiplexing throughput gains of the MU-MIMO system are shared among the different UEs. A MU-MIMO system is more tolerant of the propagation environments than a single-user MIMO system. For instance, under line-of-sight dominant propagation conditions (implying strong levels of correlation across user channels), the multiplexing gains can disappear for a single-user MIMO system but are retained in the MU-MIMO case provided that the angular separation of the user equipments exceeds the Rayleigh resolution of the array.  The co scheduled UEs should lie in uncorrelated or weakly correlated positions. Considering the propagation conditions in dominant non-line-of-sight conditions, the multiplexing gains are naturally also retained for the MU-MIMO system. Keeping this in mind, it is worth noting that irrespective of the propagation conditions, the conclusions from standard MU-MIMO techniques like ZF and MMSE to serve multiple user equipments remain consistent. To this end, where necessary, we will assume the existence of a given propagation environment coupled with the appropriate model for generating the propagation channel impulse responses, without specifying or recommending a particular propagation model.

When the mobile base station employs multiuser spatial beamforming techniques like ZF, the antenna array produces spatial nulls in the direction of undesired users. Hence, assuming rank-1 transmission (in 3GPP terminology), if there are K users, then the array forms K analog beams towards the K users where retrieval of the single data streams will occur. Each of the K beams is given by the composite array radiation pattern, as given in Table 7.2.3-1. Yet the separation of users via analog beamforming (BF) alone is not adequate to sufficiently separate the K users signals. To do this, it is preferable to have the users adequately separated say in the azimuth. Spatial beamforming techniques are used in combination with analog beamforming. This will result in nulls in the undesired direction(s).

Consider the simple example below:

Suppose there are say three users communicating with a base station. The three users can be assumed to be located at \phi_{1} = -10 degrees, \phi_{2} = 0 degrees and \phi_{3} = +10 degrees in the azimuthal plane. One can also assume that the base station array (e.g., uniform planar configuration say of size (32x8) is able to steer three independent beams to the three user equipments via the composite radiation patterns generated via the array factor in Table 7.1.3-1 above, with the azimuth main lobe of each beam pointing towards the azimuth angles of -10, 0, and+10 degrees respectively, observed at a fixed elevation angle cut of, say, 90 degrees. Furthermore, one can assume that the base station can implement a multiuser beamforming technique which allows the base station to form nulls in certain undesired directions relative to a desired direction. Then, when serving the user at \phi_{2} = 0 degrees, the base station will form nulls in the direction of the other users (signifying the undesired directions), at \phi_{1}=-10 degrees and \phi_{3} = +10 degrees respectively.

When extrapolating this logic to the case when the base station is serving a higher number of users, e.g., eight users, located in eight different azimuth directions, allowing the base station to generate seven nulls in the direction of other users relative to the desired user in the azimuthal plane.

##### 7.2.1.2.2	ZF and MMSE-based beamforming for Rank-1 MU-MIMO Transmission

When spatial beamforming is used in conjunction with analogue beamforming, the beamforming weights may be estimated as below:

7.2.1.2.2.1	ZF based beamforming

Here, we consider ZF-based beamforming for two users, denoted by UE 1 and UE 2 served by a single base station (BS) within the same time-frequency resource. Large numbers of antennas enable the focusing and steering of energy in desired directions [3]. The BS is assumed to have MN antenna elements in total configured in a uniform planar array in the x-z plane in the standard three-dimensional (3D) local coordinate system. For the sake of simplicity, perfect channel estimation at both the BS and UEs are assumed. Both UEs are assumed to have a single antenna. UE 1 and UE 2 is assumed to be located at two discrete azimuth angles, ϕe-scan1 and ϕe-scan2, respectively, with say the same elevation angle, θ1=θ2. Considering this, the composite antenna array radiation pattern towards UE 1 and UE 2 can be obtained by following the mathematical expressions given in the Table 7.1.3-1 above, where the azimuth angles, ϕe-scan1 and ϕe-scan2, are used  Two RF matrices corresponding to UE 1 and 2 as FRF1 and FRF2.respectively are derived. The size of the matrices FRF1 and FRF2is equal to MN x NtRF. where NtRF is the number of  RF chains  at the base station. If every antenna has a transmitter then the size of the beamforming matrices FRF1 and FRF2 is MN x MN respectively.

Even though the UEs may be lying say in one dimension say the azimuth only their impulse response has both an azimuth and elevation component see the impulse response channel matrix in [2].

Note that the exact structure of h1and h2 depends on the radio propagation conditions at the time of transmission from the BS to both UEs. However, this notation is agnostic to the type of conditions and propagation mechanisms present in the channel responses and hence is preferred for use in this document.

Concatenating the equivalent channel responses into a total equivalent channel matrix of appropriate size by stacking the two equivalent user channels, one can write:

H= h1T   h2T T					(1)

Where    h1  and    h2  are the two equivalent channels and are in turn equal  h1 FRF1 and h2 FRF2  respectively.

Now the ZF-based beamforming to null the inter-user interference can be used on H. To this end, the ZF beamforming matrix, W, can be calculated as:

W=HHHHH-1.                                                              (2)

This will ensure that if the BS is transmitting the desired signal in the direction of UE 1, i.e., towards (ϕe-scan1,θ1), a null in the radiation pattern of the BS can be formed towards the unintended direction of UE 2, i.e., towards ϕe-scan2,θ2. We note that the ZF beamformer should be normalized to satisfy a total power constraint. There are two different ways to normalize the ZF beamformer, namely via a vector normalization or via a matrix normalization. Using the example of a vector normalization, the per-user digital beamforming weight vector specific to UE 1 and UE 2 can be written as w1and w2 forming the two columns of W. Normalizing these using the vector normalization method yields

w1normalized=w1FRF1 w1,                                                             (3)

and

w2normalized=w2FRF2 w2.                                                             (4)

This implies that for UE 1, multiplying the h1w1normalized=1,  while  h1w2normalized=0 and vice-versa for UE 2. Therefore, a null in the azimuth direction can be formed towards UE 2 while serving UE 1.

The array response vmn  ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of  the angles in wmn which can be viewed as additional phase terms in the array response.  It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on  wmn alone is flawed and will not provide an exact impact of ZF on the complete array response.

7.2.1.2.2.2	MMSE based beamforming

In the case of MMSE beamforming, the MMSE beamforming matrix, W, is given by

W=HHHHH+βI-1,                                                      (5)

where β>0 is the regularisation constant/factor and I is the identity matrix of the same dimension as HHH. By maximizing the SINR at the UEs, the optimal regularisation constant/factor was derived in [6] as β=K/ρ, where ρ is the operating signal-to-noise ratio (SNR) typically computed by taking the ratio of the EIRP with the noise variance (power spectral density) of the UE. Note that optimal regulariser was only obtained for the case of equal conducted transmit power being split across the K user equipments and only applies to rank-1 transmission.

It is noteworthy that when ρ →∞ (holding the value in K as constant), reflecting high SNR scenarios, the MMSE beamforming matrix in (5) reduces to the ZF beamforming matrix in (2) as β→0.

The normalization of the beamforming vectors of the MMSE approach is identical to the ZF and hence the expressions in (3) and (4) apply with w1 and w2 now derived from the MMSE expression in (5).

### 7.2.2	Simulation results

#### 7.2.2.1	Methodology 1

##### 7.2.2.1.1	Ericsson [30]

The parameters values listed in Table 7.2.2.1.1-1 and 7.2.2.1.1-2 is used for the simulation.

Table 7.2.2.1.1-1: IMT deployment-related parameters

| Parameter | Macro Urban |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz |
| Channel bandwidth | 100 MHz |
| BS antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector 1 |
| Frequency reuse | 1 |
| User Equipment |  |
| UE height | 1.5 m |
| UE density for terminal that are transmitting simultaneously | 3 UEs per sector |
| NOTE 1: This study considers only a single-entry scenario. |  |

Table 7.2.2.1.1-2: IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 1 |
| Mechanical down-tilt (degrees) | 10 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

For the sake of completeness, additional results are provided with an AAS with a subarray configuration. The parameters for this AAS are taken from the WRC-23 studies 5D/1235 and 5D/1461 and are presented in the Table 7.2.2.1.1-3.

Table 7.2.2.1.1-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 1 |
| Mechanical down-tilt (degrees) | 6 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam. These beams can be generated by both the traditional antenna model and as described in subclause 7.2.1.1, assuming a UE minimum angular separation of 10 degrees.

To assess the interference to possible interfered-with receivers, two scenarios are considered, where these receivers are represented by sample points:

1.	Equidistant sample points towards the horizon around the IMT BS (Figure 7.2.2.1.1-1a)

2.	Equidistant sample points on a hemisphere above the IMT BS excluding the sample points towards the horizon (Figure 7.2.2.1.1-1b)

It is noted that the angular separation between sample points is approximately 15.5 degrees, with scenario 1 having 23 sample points and scenario 2 having 62 sample points.

Figure 7.2.2.1.1-1: Equidistant sample points (a) towards the horizon around the BS, and (b) on a hemisphere above the BS

In this section the Monte Carlo simulations results are presented. For each sample point in the considered scenarios, 10.000 snapshots are simulated. The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 7.2.2.1.1-2 to 7.2.2.1.1-5. The CDF results for scenario 1 using a BS without and with subarrays are shown in Figures 7.2.2.1.1-2 and 7.2.2.1.1-3 respectively. The CDF results for scenario 2 using a BS without and with subarrays are shown in Figures 7.2.2.1.1-4 and 7.2.2.1.1-5 respectively. For instance, since scenario 1 has 23 sample points, Figures 7.2.2.1.1-2a and 7.2.2.1.1-3a show 23 pairs of CDFs.

Figure 7.2.2.1.1-2: Scenario 1 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

Figure 7.2.2.1.1-3: Scenario 1 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

Figure 7.2.2.1.1-4: Scenario 2 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

Figure 7.2.2.1.1-5: Scenario 2 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 7.2.2.1.1-2a, 7.2.2.1.1-3a, 7.2.2.1.1-4a, and 7.2.2.1.1-5a, Table 7.2.2.1.1-1 contains the summary of the average BS antenna gain difference between traditional beamforming model and ZF beamforming model.

Table 7.2.2.1.1-1: Average BS antenna gain difference between traditional beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.39 dB | 0.14 dB |
| Scenario 1 (with subarrays) | 0.05 dB | ~0 dB |
| Scenario 2 (without subarrays) | 0.48 dB | 0.06 dB |
| Scenario 2 (with subarrays) | 0.08 dB | ~0 dB |

Furthermore, the BS antenna gain integrated over parts of or the entire angular sphere around the antenna, i.e., the Total Integrated Gain (TIG), is also evaluated to provide a set of more consistent results by considering all angular directions. The TIG is calculated as follows:

TIG≈π2NθNφφ=0Nφ-1θ=0Nθ-1Gθ,φsinθ 		(Eq.7.3.2.1.1-1)

where θ and φ represent the elevation and azimuth angles, respectively, Nθ and Nφ are the number of samples on elevation and azimuth, respectively, and G is the antenna gain in linear units at the spherical coordinates (θ, φ).

For this assessment, the BS antenna gain is integrated over the whole sphere and over only the upper hemisphere by limiting the range of θ. The integrated gain is calculated for each individual beam for 10000 snapshots, resulting in three values per snapshot. Note that the integrated gain of the upper hemisphere of the AAS, where θ = 90 degrees is the AAS boresight direction including down-tilt, is not exactly the integrated above the horizon but includes most of it. This is depicted in Figure 7.2.2.1.1-6.

Figure 7.2.2.1.1-6: Upper hemisphere: (a) upper hemisphere above horizon, and (b) upper hemisphere considered in this study

Figures 7.2.2.1.1-7 and Figure 7.2.2.1.1-8 show the BS antenna gain integrated over the whole sphere and over only the upper hemisphere (Figure 7.2.2.1.1-6b), including the horizon, for the BS without and with subarrays, respectively. For simplicity, it is noted that for all the presented results, the integrated gain is always normalized by π2NθNφ, as indicated in Eq.7.2.2.1.1-1, regardless of the number of samples considered.

Figure 7.2.2.1.1-7: CDFs of the BS antenna gain without subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

Figure 7.2.2.1.1-8: CDFs of the BS antenna gain with subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

As can be seen, in all cases considered, the TIG difference between the beamforming schemes is negligible.

##### 7.2.2.1.2	Qualcomm [31]

BS AAS parameters and configurations that are considered in our Monte-Carlo simulation campaign are provided in Table 7.2.2.1.2-1, Table 7.2.2.1.2-2, and Table 7.2.2.1.2-3, respectively. In terms of scenario/deployment, we consider a single sector where within each snapshot, 3 UEs are randomly deployed within the coverage region of that sector with a given minimum angular/spatial separation of 10 degrees. The location of those UEs is assumed to be known at the BS and a dedicated beam is generated to serve each UE. In order to investigate the impact of BS emissions above the horizon, probing points in the upper hemisphere have been placed, as visualized in Figure 7.2.2.1.2-1. Black points represent the total probing points, while blue points represent the 3 UEs dropped in each snapshot, taking into account the minimum angular separation among those UEs (i.e., 10 degrees as assumed in this contribution). The probing points are equidistantly distributed on a hemisphere above the horizon with angular separation between them approximately 18 degrees. It is worth mentioning that the objective of our study is to investigate the experienced emissions at the different probing points, since they represent possible incumbent services that will suffer from to the IMT BS unwanted emissions above the horizon.

Table 7.2.2.1.2-1 IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz-band |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |

Table 7.2.2.1.2-2: IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 (Note 1) |
| Mechanical downtilt (degrees) | 10 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

Table7.2.2.1.2-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array downtilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 (Note 1) |
| Mechanical downtilt (degrees) | 6 |
| Note 1: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

.

Figure 7.2.2.1.2-1 System layout for a given sector

The CDF curves of the IMT BS gain towards the black probing points are shown in Figure 7.2.2.1.2-2, considering the AAS without subarray structure (left figure) and with subarray structure (right figure). It can be observed the performance degradation observed when ZF is applied is negligible. Such behaviour shows that when considering large number of snapshots in a Monte-Carlo framework, the impact of the spatial beamforming above the horizon has minimal impact on the emissions above the horizon.

Figure 7.2.2.1.2-2 Composite AAS gain from IMT BS towards probing points considering AAS with (left) and without subarrays (right)

##### 7.2.2.1.3	Huawei [32]

In order to assess the necessity and accuracy of interference comes from IMT systems for ITU-R sharing and compatibility studies, the IMT BS gain towards a number of potential interfered-with receivers located in the upper hemisphere are simulated. The angular separation between sample points is assumed 10 degrees.

Figure 7.2.2.1.3-1: 10-degree sample points in the upper hemisphere above the BS

Deployment related parameters of BSs and UEs are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC-23), and beamforming antenna characteristics are referred to 3GPP liaison statement on parameters for 4400 to 4800 MHZ of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, see 5D/136.

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 7.2.2.1.3-2 with additional individual per-antenna power constraint. It can be seen that the difference between ITU-R M.2101 and ZF gains are minor.

Figure 7.2.2.1.3-2: BS gain towards all sample points (composite) with individual per-antenna power constraint

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. Table 7.2.2.1.3-1 contains the summary of the BS antenna gain difference between ITU-R M.2101 model and ZF beamforming model with & without additional individual per-antenna power constraint. Additional individual per-antenna power constraint influences the simulation results very little. In addition, it shows that ZF beamforming get slightly less antenna gain than M.2101 towards to the sample points with individual per-antenna power constraint:

Table 7.2.2.1.3-1: BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming (Method 1)

| C band Urban (with subarrays) Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| M.2101 | -13.720 | 20.861 |
| ZF without individual per-antenna power constraint | -13.587 | 20.855 |
| ZF with individual per-antenna power constraint | -13.779 | 20.719 |
| M.2101- ZF difference (dB) (without individual per-antenna power constraint) | -0.133 | 0.006 |
| M.2101- ZF difference (dB) (with individual per-antenna power constraint) | 0.059 | 0.142 |

#### 7.2.2.2	Methodology 2

##### 7.2.2.2.1	Spark [33]

We provide below example simulation results for using ZF with two UEs, both are in the azimuth at 0 and 10 degrees respectively and are at the same elevation with a 5 degrees down tilt (angle from the Z axis is 95 degrees). The antenna and other parameters are:

Table 7.2.2.2.1-1 IMT deployment-related parameters

| Parameter | Value |
| --- | --- |
| Environment | Macro Urban |
| Carrier frequency | 6 GHz |
| Sectorization | 1 sector |
| Frequency reuse | 1 |

Table 7.2.2.2.1-2 IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Section 5 of Recommendation ITU-R M.2101 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 |
| Mechanical down-tilt (degrees) | 10 |

The cdf of the array gain of the elevation pattern above the horizon for methodology 2 using ZF MU MIMO  is shown below for 1000 simulation runs.

Figure 7.2.2.2.1-1 Aggregate Antenna Array Gain Above Horizon [dBi]

The following observations may be made:

The median gain difference is about 3 dB.

When more UEs are added to the ZF set, the placement of nulls in the azimuth pattern will have a consequential impact on the elevation pattern and will further increase this difference.

The example is for two UEs with a given azimuth placement. The median gain difference will also be sensitive to the actual azimuth angles at which the UEs are placed. The point of this exercise is just to demonstrate the impact of ZF beamforming using the correct formulation of pseudo inverse to accurately assess the aggregate antenna array gain above the horizon. The conclusions presented here can be replicated for a larger number of users in a given sector with the same methodology.

##### 7.2.2.2.2	Huawei [32]

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the target UE are shown in Figure 7.2.2.2.2-1. It can be seen that this method can also get reasonable target antenna gain compared to M.2101.

Figure 7.2.2.2.2-1: BS gain towards to target UEs

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 7.2.2.2.2-2 without additional individual per-antenna power constraint. It can be seen that the difference between ITU-R M.2101 and ZF gains are minor.

Figure 7.2.2.2.2-2: BS gain towards all sample points (composite) without individual per-antenna power constraint (Method 2)

Table 7.2.2.2.2-1: BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming (Method 2)

| C band Urban (with subarrays) Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| M.2101 | -20.810 | 19.527 |
| ZF (Method 2) without individual per-antenna power constraint | -20.992 | 19.197 |
| M.2101- ZF (Method 2) difference (dB) (without individual per-antenna power constraint) | 0.182 | 0.33 |

### 7.2.3	Summary and Conclusion of MIMO modelling

The simulation results in clause 7.2.2 using the simulation methodologies in clause 7.2.1 have indicated that MU MIMO based ZF BF does not need to be considered for the pure LOS case, and Rec. ITU-R M.2101 methodology remains valid for performing sharing and compatibility studies in WP5D.

## 7.3	Modelling array antenna gain outside the carrier

### 7.3.1	General

#### 7.3.1.1	Purpose

This clause aims to provide IMT base station and IMT user equipment technical parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations covering aspects related to modelling array antenna gain outside the wanted carrier relevant for modelling spatial characteristics for unwanted emissions.

#### 7.3.1.2	ITU radio regulations on adjacent terrestrial IMT systems with other co-primary services

There are adjacent services in the bands being considered for IMT 2030.

For example: for the case of 4400-4800 MHz, the adjacent band services are:

On the lower edge below 4400 MHz:

-	Aeronautical radionavigation

-	Aeronautical mobile

-	Additional possible allocation of EESS (passive) as per Agenda Item 1.19 of WRC-27;

On the upper edge above 4800 MHz:

-	Fixed service

-	Radio Astronomy

-	Radio navigation satellite

For the case of 7125-8400 MHz, the adjacent band services are:

On the lower edge below 7125 MHz:

-	Fixed

-	Fixed Service Satellite (Earth-to-space)

On the upper edge above 8400 MHz:

-	Space Research Service (space-to-Earth)

-	Additional possible allocations of EESS (passive) as per Agenda Item 1.19 of WRC-27

-	Earth Exploration Satellite

-	Radiolocation

-	Space Research (active)

For the case of 14.8-15.35 GHz, the adjacent band services are:

On the lower edge below 14.8 GHz:

-	Fixed Service

-	Fixed Satellite Service (Earth-to-space)

-	Radionavigation

On the upper edge above 15.35 GHz:

-	Space research (passive)

-	EESS (passive)

-	Radio Astronomy

In particular, the frequency range 15.35 – 15.4 GHz is marked as a footnote 5.340 band, which is stating “All Emissions are Prohibited” with the exception of those provided by RR 5.511.

It is therefore important that out of band emissions are correctly modelled so that appropriate guidance is provided for the above question to WP 5D.

### 7.3.2	Adjacent channel modelling

#### 7.3.2.1	Correlation roll-off based model

##### 7.3.2.1.1	Modelling overview

The array antenna model adopted by 3GPP in TR 38.803 [29] creates the composite pattern as a multiplication between the element factor (or sub-array element factor) and array factor. The array antenna model was first introduced in TR 37.840 [26] in the early days of AAS for BS. Based on the antenna model, the average radiation pattern produced by an array antenna was evaluated and an analytical expression for the average radiation pattern was established.

The composite radiation gain pattern for pure LOS channel can be expressed in logarithmical scale as:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1		(Eq. 7.4.1-1)

, where r is the correlation factor defined in the interval 0≤ρ≤1. Relevant parameter values for the frequency range 4400 to 4800 MHz can be found in subclause 4.4.1.2 and for 7125 to 8400 MHz in subclause 5.4.1.2, and for 14800 to 15350 MHz in subclause 6.5.1.2.

It can be noticed that the average antenna gain for the wanted signal where r is equal to 1, the composite array gain is produced (GE,max + 10log10(Msub) + 10log10(MN) dBi), while for unwanted emission away from the carrier where r is equal to 0 only the gain of the element/sub-array is produced (GE,max + 10log10(Msub) dBi). For sharing studies, it would be relevant to consider r being represented as a function of frequency.

The impact of array signal correlation to directivity and gain response have been evaluated for different array antenna structures considered for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz considering 10 degrees down-tilt beam steering direction. The results are plotted in Figure 7.3.2.1.1-1.

Figure 7.3.2.1.1-1: Vertical radiation pattern considering different array geometries and correlation factor.

The array factor impact due decorrelation can be analysed for all antenna geometries separately. The array factor drops gradually for low correlation as shown in Figure 7.3.2.1.1-2.

Figure 7.3.2.1.1-2: Array factor decorrelation characteristics plotted as function of correlation.

It can be noticed that the average gain drops gradually when the array correlation reduces. At array correlation 0.5 the array factor has reduced approximately 3 dB.

The characteristic is visualized in Figure 7.3.2.1.1-2 can be used to translate measured directivity drop to correlation factors to use for modelling purposes.

##### 7.3.2.1.2	Correlation roll-off model

Since the array factor gain relies on coherent combining of signals it is not expected that the array factor will operate outside the carrier with full strength in situations outside the carrier where the emission is created as the sum of correlated intermodulation distortion and uncorrelated wide band noise. The Inter-Modulation Distortion (IMD) is coming from non-linear transmitter. The characteristics is typically categorized as distortions due third order (IM3), fifth order (IM5), seven order (IM7), etc. IMD. It is known that the order of IMD will determine the spectral distribution of the distortion. The IMD will also depend on the wanted signal carrier bandwidth, B. The IMD response typically reduces moving away from the edge of the wanted signal.

For the wanted carrier (within the interval -0.5B≤fc≤0.5B, fc is the carrier centre frequency) full array factor is expected which means that the correlation factor is equal to 1. Moving away from the carrier edge (fc>0.5B) the correlation will drop gradually due to the composition of the emission outside the carrier. The emission will be created as the sum of correlated noise due to intermodulation products and non-correlated noise due to wideband transmitter noise. The relation to the induvial noise levels will determine the out-of-carrier correlation properties in the adjacent emission regions. At a certain frequency offset to the carrier edge the emission noise will be dominated by wideband noise which means that the correlation is equal to 0.

Therefore, it would be appropriate to define a model for the array correlation factor to account for carrier bandwidth and decaying correlation moving away from the carrier edge.

To properly model the AAS base station array antenna gain pattern outside the carrier the correlation factor needs to be modelled as a function of frequency offset to the carrier centre frequency. To capture fundamental aspects of the array antenna gain drop outside the wanted signal a parameterized piece-wise linear roll-off model can be adopted for the correlation factor, as visualized in Figure 7.3.2.1.2-1.

Figure 7.3.2.1.2-1: Array correlation factor roll-off model.

The roll-off is modelled to reduce stepwise moving away from the centre frequency, also the characteristics is assumed to be symmetrical around the wanted carrier. Close to the carrier the correlation is expected to be close to 1, while moving towards +/-1.5B reducing to r2. At f1 the correlation have reduced even more to r1. At f0 the correlation has reduced to 0, which corresponds to that the array factor gain in 0 dB.

For this model to be use-full parameter values for B, f0, f1, r1 and r2 needs to be determined.

Using the approach above the antenna model can be extended to capture array antenna gain outside the wanted carrier as:

AAθ,φ,f=Asubθ,φ+10log101+ρfm=1Mn=1Nwm,nvm,n2-1		(Eq. 7.3.2.1.2-1)

Observe that the extended array antenna model only captures effect due to array signal correlation and not effects due to antenna element detuning effects of mismatch effects in transmission lines and Radio Distribution Network (RDN).

##### 7.3.2.1.3	EIRP model

Observe that the array correlation roll-off model presented above only captures decorrelation effects related to the array factor. Keep in mind that even though the array factor totally collapses the sub-array still produce gain far outside the carrier. Typically, the sub-array starts to collapse outside the operating band until the antenna is completely detuned far from the centre of the operating band.

Another aspect to consider when modelling spatial unwanted emission outside the carrier is the suppression of unwanted emission outside the carrier in the adjacent channel regions as well as in the spurious domain. In these regions the emission is suppressed by the ACLR and RF filter suppression.

The radiated emission from a base station to be studied in a sharing situation can be calculated as:

EIRPθ,φ,f=Ptxf+AAθ,φ,f

, where Ptx is the power fed to the antenna, f is the frequency offset between the carrier centre frequency and the considered frequency. The frequency dependence for the gain pattern AA(q,j,f) as described in Eq. 7.3.2.1.2-1.

The power fed to the antenna can be expressed as a function of frequency offset as:

Ptxf=P-S(f)

, where is P is the carrier power, S(f) is the suppression achieved by ACLR1 (First adjacent channel), ACLR2 (Second adjacent channel), ACLR3 (Third adjacent channel) and additional RF filter suppression (if frequencies outside the operating band is considered) as function frequency offset.

It is evident that the radiated power in the adjacent channel regions depends on the drop of antenna gain and the power fed to the antenna. The combined effect should be considered to properly model sharing situations.

#### 7.3.2.2	Array ACLR based model

##### 7.3.2.2.1	Overview

Out of band emissions are well understood but often for a single transmitter.

It is necessary to distinguish between a single element ACLR, and an array ACLR. In a non AAS system, where the radiation pattern is “constant” over a sector, the single element ACLR makes sense, since the received power ratio at any point is the same as the transmitted. However, with AAS, the desired signal experiences also an array/beamforming gain that might be different from the array/beamforming gain of the interfering signal received in the adjacent band; this later observation is also dependent if SU or MU MIMO is used (i.e. the array may send distortions in unintended directions).

When an array in AAS base stations is used, the 3GPP specifications provide a method to compute the array ACLR (albeit in a TRP mode) [9]:

“The ACLR (CACLR) absolute basic limits in table 6.6.3.2-2 + X, 6.6.3.2-2a + X (where X = 9 dB) or the ACLR (CACLR) basic limit in table 6.6.3.2-1, 6.6.3.2-2a or 6.6.3.2-3, whichever is less stringent, shall apply.”

However, the above method is array pattern agnostic and does not account for beamforming variations due to MU MIMO.

Recent published results, discussed/summarized in this report, show that that array ACLR is spatially sensitive and is also dependent if SU or MU MIMO functionality is utilized. There is thus a need to accurately model the ACLR when AAS is used.

As part of additional information requested by ITU-R WP 5D establish a model for array correlation factor (r) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

To address the SI, the questions posed may be devolved into topics below:

1.	Establish a model for array correlation factor-ie the correlation of the in-band signal with the out of band signal.

There is no definition of an array correlation factor. Is it the correlation of the in-band signal with out of band signal. But the in-band signal, Pinband (θ,φ),   is spatially sensitive and as discussed below the out of band signal , P oob (θ,φ),  may also be spatially sensitive.  Then the array correlation  is also spatially sensitive may be defined as:

r (θ, φ) = E (Pinband (θ,φ) PT oob (θ,φ) ) ………………………….(6)

2.	How does the array factor collapse when moving away from the centre frequency?

This also means that r (θ, φ) defined above is a function of frequency separation from the band centre. In this case best to denote this as:

r f(θ, φ)………………………………………………………………..(7)

Where f is the frequency separation from band centre

3.	If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined.

Therefore we would need to characterize and investigate the constituent components of P oob (θ,φ) as a function of f

The above topics are interrelated but need to be separately discussed as well. The discussion below addresses topics 2 and 3 respectively.

Array performance in adjacent bands is governed by:

-	Array performance at different frequencies arising due to adjacent bands.

-	Modelling of the impact of PA non linearities

-	Modelling of band pass filters

-	Putting all of the above together to obtain an end-to-end model

Additionally, one must also consider if the array is supporting a single user or multi-user systems.

##### 7.3.2.2.2	Array performance at adjacent band frequencies

Assume that the lower and upper adjacent bands have a bandwidth of 100 MHz each respectively.  Arrays in a band are designed at the centre frequency of a given band ie vertical and horizontal spacing is determined by the centre frequency of the band which for example in band 1 is 4600 MHz shown in Table 7.3.2.2.2-1 below. Likewise for other bands. The lower and upper adjacent bands can be simulated via a proxy array with different spacings e.g for band 1, the lower adjacent band is 4300-4400 MHz with centre frequency 4350 MHz. Therefore, for band 1, changing the vertical spacing to 0.74 lambda and horizontal spacing to 0.53 lambda will effectively simulate the lower adjacent band. Likewise for the upper adjacent band (0.66, 0.474) will simulate the upper adjacent band.

Table 7.3.2.2.2-1: Array performance in adjacent bands

| Band | Centre Frequency | 0.7 Lambda, 0.5 Lambda (cm) | Centre freq of Lower adj band | Centre freq of Upper adj band | 0.7 Lambda, 0.5 Lambda for lower adj band | 0.7 Lambda, 0.5 Lambda for upper adj band |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4600 MHz | 4.565/3.26 | 4350 MHz | 4850 MHz | 4.827/3.45 (0.74, 0.53) | 4.33/3.09 (0.66, 0.474) |
| 2 | 7762.5 MHz | 2.7/1.93 | 7075 MHz | 8450 MHz | 2.96/2.118 (0.767, 0.55) | 2.48/1.773 (0.64, 0.46) |
| 3 | 15.075 GHz | 1.39/0.99 | 14.75 GHz | 15.40 GHz | 1.42/1.012 (0.715, 0.511) | 1.36/0.97 (0.68, 0.49) |

The following parameter/metrics are impacted [15] when the inter element spacing is changed:

-	Nulls

-	Maxima

-	Half power points

-	Minor lobe maxima

-	First null beamwidth

-	Half power beamwidth

-	First side lobe beamwidth

Exact expressions for the above are given below from [15] for a ULA along the z axis and for omni directional antennas. Considering the exact formulas below in Table 7.3.2.2.2-2 and Table 7.3.2.2.2-3 the changes due to inter element spacing in the proxy array will result in some changes to the values of the above metrics. See Figure 7.3.2.2.2-1, Figure 7.3.2.2.2-2 and Figure 7.3.2.2.2-3 for an eight element ULA for 0.5 lambda, 0.7 lambda and 0.8 lambda inter element spacing respectively. However these frequency/wavelength adjustments are no different to what is present today in systems that have a wide operating bandwidth and antennas are designed at the centre frequency of the band). Note that the results below are not for an array of sub arrays and 3GPP antenna element patterns, but the trends will be similar for the array of sub array case as well.

Table 7.3.2.2.2-2: Broadside array antenna pattern equations

| Nulls, Maxima, Half-Power points and Minor Lobe maxima for Broadside Arrays |  |
| --- | --- |
| Nulls | θn = cos-1 (±nN λd), n = 1,2,3…., n≠N, 2N,3N.. |
| Maxima | θm = cos-1 (±m λd), m = 0,1,2,3…., |
| Half Power Points | θh = cos-1 (±1.391λπ Nd), πdλ ≪ 1 |
| Minor Lobe Maxima | θs= cos-1 (±λ2d 2s+1N), s = 1,2,3, πλd ≪ 1 |

Table 7.3.2.2.2-3: Broadside array beamwidth equations

| Beamwidths for Uniform Amplitude Broadside Arrays |  |
| --- | --- |
| First Null beamwidth (FNBW) | θn=π2-cos-1λNd |
| Half Power beamwidth (HPBW) | θh≅2π2-cos-11.391λπNd πdλ ≪ 1 |
| First sidelobe beamwidth (FSLBW) | θs≅2π2-cos-13λ2Nd πdλ ≪ 1 |

Figure 7.3.2.2.2-1:  Eight element ULA response with 0.5 lambda spacing

Figure 7.3.2.2.2-2:  Eight element ULA response with 0.7 lambda spacing

Figure 7.3.2.2.2-3:  Eight element ULA response with 0.8 lambda spacing

It is also to be noted that in case of band 2 where the band is very wide (operating bandwidth 1275 MHz) proxy antenna inter element spacings for a carrier at the lower end of the band will almost have a similar impact as the lower adjacent band. Likewise for the upper end of the band.

The above inferences will also remain valid for the second adjacent band.

The above observations will remain valid even if the array is made of sub arrays and the notion that an array may drop down to a sub array in adjacent bands is not valid.

##### 7.3.2.2.3	Modelling of PA non-linearities

Non linearities in hardware in the RF path cause signal distortions- both in band and out of band-(adjacent bands). The nonlinear components in the RF path consist of power amplifiers that are the main cause of nonlinear distortions but other components such as D/A converters, mixers, beam forming circuitry may also contribute to the nonlinear distortions. When antenna arrays are deployed, multiple users may be served by beam forming either in the analog domain or both in the analog and digital domain.  Digital beamforming is commonly employed in today’s systems- such as zero forcing, MMSE [16]. This results in the placing of nulls in the path of all users except the desired user, but this operation may also cause out of band distortions to fall in unintended spatial directions.

The modelling of PA non linearities has received a lot of interest in the published literature [see 17,18-22 and references therein]. PA behavioural models can be classified as:

-	Volterra series based,

-	Artificial neural networks,

-	Table look up methods,

-	Models with linear and nonlinear memory, with and without memory etc.

Ref [17] gives a very good comparison of PA behavioural models in terms of metrics like identification complexity, adaptation complexity, running complexity etc. Measurements on some PAs and the accuracy of the models are presented in [20]. Some commonly used models use polynomials or memory polynomials [18,22], here PA units are modelled via 9th order polynomials whose coefficients are obtained from RF measurements. These papers conclude that the received signal with massive MIMO consists of a desired term with linear gain of the array and also a nonlinear distortion term. Additionally, if the linear response coefficients of the different parallel PAs (in the massive MIMO unit) are different (and they do have small differences), then even the linear signal terms are not fully coherently combining at the receiver. In case of the nonlinear distortion terms, this effect is even more evident as here the phase characteristics of the constituent terms representing the component PAs are different. These two effects manifest in the decrease of the desired instantaneous SINR. When pre-coded multiuser MIMO is used, [21,22] derive analytical models for three different precoder types and for each case derives the received signal. The later now consists of a linear useful signal part and several distortion terms arising from multiuser interference and nonlinear distortion. The conclusion of this paper is:

The choice of a PA behavioural model validated by measurements is needed. Ref [18,19] provide a good framework to model these non-linearities via polynomial models.

The differences in the linear responses of the parallel PAs have a large impact on the system performance. The nonlinear terms do not necessarily adopt the digital predistortion as easily and efficiently. Therefore, the impact of nonlinear power amplifiers is nontrivial both in band and out of band. However, to do this we would need a model for the differences in the PA models in an AAS.

Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used.  The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs.

We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS.

Adjacent band impacts is however not the main focus of [18], but they are considered in [17,18,21]. These papers give the spatial distribution of out of band radiation in the presence of non-linear amplifiers that are modelled by orthogonal polynomials- a special case of the more general Volterra series as described in [17]. Refs [8,21] derive an expression for array ACLR when linear precoding is used with MIMO arrays and a spatial distribution of the ACLR is derived for 1, 2 and 10 users respectively. In particular, [21] develops a framework for rigorous analysis of the spatial characteristics of nonlinear distortion from arrays both in band and out of band. The framework is validated via measurements on a GaN class AB amplifier [23]. It is shown that in the immediate adjacent bands, the third order distortion terms are significant. For a two user MU MIMO case it is shown that the desired signal is beamformed in two directions but the adjacent band distortion signal (arising from PA non linearities) is not beamformed in the same direction as the desired signal. Consideration of this will be important in evaluating the impact of adjacent band victims due to PA non linearities in the operating band.

##### 7.3.2.2.4	Modelling of band pass filters

The filters modelled in some references [15,] use a root raised cosine filter with an excess bandwidth of 0.22.  Also, the adjacent channel has the same bandwidth as the operating bandwidth. This is clearly not the case, for all the bands under consideration the operating bandwidths are much larger than the carrier bandwidth. The adjacent channel bandwidth may be saying 100 MHz. However, the framework in [23] can be used to change the bandwidth of the BPF to reflect the operating bandwidth. However, we must specify the bandwidth of the BPF and a model for the BPF.

<End of TP>

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2410576: “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#111, LSout

- R4-2414449: “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#112, LSout

- R4-2420385: “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#112, LSout

It should be note that the array response is both in the azimuth and elevation.

We assume a broadside array. An elevation angle of 90 degrees is when users are in the elevation boresight.

Whilst antenna numbers can increase due to advancements in semiconductor technology [3], the numbers of users to be nulled must be less than or equal to the number of RF chains

In practice, this is not the case, as Sounding Reference Signals (SRS) are used in time-division duplex (TDD) systems, where the UEs would send an uplink SRS signal based on which the BS will estimate the uplink channel and using channel reciprocity, will estimate the downlink channel. In frequency-division duplex (FDD) systems, Channel State Information-Reference Signal (CSI-RS) is transmitted from the BS periodically and the UEs will estimate the channel and feedback the channel response via a codebook to the BS [5].

Mathematical notation: We note that h1 denotes a vector, while H denotes a matrix. Furthermore, h is denoted as a real scalar value, where as ⋅Tand ⋅H are used to denote a transpose and a Hermitian transpose operation. Finally, H-1 is used to denote the matrix inverse of a square matrix (same number of rows and columns), H.