3GPP TSG RAN WG4 Meeting #114		 R4-2502949

Athens, GR, 17th – 21nd February, 2025

Agenda Item:	7.6.3

Source: 	MediaTek Inc.

Title: 	Impacts on UE RF requirements and DL performance impacts

Document for:	Approval

1. Introduction

In this meeting, we provide our RF performance evaluation on UE RF requirements and DL performance impacts based on agreed assumptions by RAN4.

TP for TR38.755

<Start of proposed TP>

7	Study on RF requirements for the inter-operator co-located BS scenario

It has been agreed baseline UE architecture is one Rx RF chain from single antenna to the end of ADC with a single down-converter. The evaluation assumptions were also agreed as summarized and re-organized bellow[4][5]:

- Example bands and CBW/BCS assumptions:

  - n2/n25, n3, n7, n66, n41, n26

  - Prioritize the frequency bands of ≤2700MHz

  - The example band(s) should already have NC DLCA specified

  - Don’t consider 3MHz channel bandwidth for fragmented carriers

  - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators

- Assumptions for collocated adjacent channel inter-operator:

  - To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared.

    - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

    - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

- Assumptions for Tx:

- Re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

- One UL carrier is prioritized

- UL power class PC3 is prioritized. Both PC3 and PC2 should be considered

- No relaxation on any Tx RF requirements should be assumed in this study item

- Assumptions for Rx:

  - One Rx RF chain (fully shared Rx) is assumed

  - Reuse the Rx chain image rejection, which is 25dB, as the starting point

- Baseband assumptions:

  - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI

    - 15KHz SCS is used as the starting point

  - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR

- Assumptions for test conditions:

  - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used

  - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc

  - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet

    - Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

    - It is the condition to apply the requirements

- [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

RAN4 also agreed a common table format to demonstrate evaluation parameters and results. Company A provide evaluation results for example bands in bellow:

7.1 RF requirements evaluation from company A

7.1.1 Analysis on Band n25

If the Rx chain is shared by the two non-contiguous CCs, (fully-shared Rx chain), the channel selection filter would need to be adjusted to wider bandwidth to accommodate the two DL carriers including Wgap between the two CCs. The Tx rejection of shared Rx chain would be much worse. Not only SCC would still impacted by self-band UL Tx, PCC may also be impacted due to worse filter rejection ratio toward UL Tx transmission. It is foreseen the DL AGC would need to be adjusted to tolerate larger Tx leakage portion thus the gain setting would become lower that brings noise level raised up for both PCC and SCC. Another potential degradation may happen on in-gap ACS/IBB/NBB performance, these requirements would also need to be evaluated.

Figure 7.1.1-1: Rx requirements to be evaluated

Figure 7.1.1-2: Illustration on the two BCS for DL NCCA test

The existing requirements of n25(2A) is copied below:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

Case 1, 5 + 5 MHz

Based on the agreed assumptions and reference table format, we put our evaluation parameters in the table 1 with proper change. We think assuming 0dBFS for the ADC input target level may not be a proper setting. In most cases, there would be PAPR, radio signal fluctuation and fading effect for the input signal. So we would suggest not to consider this case. The table is modified as below:

Table 7.1.1-1: Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -91.5 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -101.5 | -96.5 | / | / |
| LPF attenuation | 6.1 | 1 | 1 | / | / |
| LPF output (dBm) | -29.1 | -102.5 | -97.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.1 | / |
| Noise floor | / | / | / | -88.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -96.5 | / | / |
| ADC output (dBm) | / | / | / | / | -96.2 |
| SNR after ADC (dB) | / | / | / | / | -13.8(PCC) |
|  |  |  |  |  | -14.3(SCC) |

Case 2, 40 + 5 MHz

The REFSENS of 40MHz is not linear scaling from 5MHz in existing specs. So we had calculated excessive Tx leakage level fall in the DL receiving channel in. The evaluation parameters and results are demonstrated in below table.

Table 7.1.1-2: Link budget of Self-interference for 40+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -79.5 | -71.9 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -84.5 | -76.9 | / | / | / |
| LPF attenuation | 6.1 | 1 | 1 | / | / | / |
| LPF output (dBm) | -29.1 | -85.5 | -77.9 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -92.5 | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.1 | 13.1 | / |
| Noise floor | / | / | / | -79.4 | -88.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -76.9 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -87.2 |
| SNR after ADC (dB) | / | / | / | / | / | -6.5(PCC) |
|  |  |  |  |  | / | -24.9(SCC) |

7.1.2 Analysis on Band n2

The frequency span of band n2 is 5MHz smaller than band n25. So the gap between Tx and Rx can be slightly larger so that the evaluated result may be a bit better (less ΔRIBNC). Following common assumptions, the evaluation result are demonstrated in below tables.

Table 7.1.2-1: Link budget of Self-interference for 5+5 MHz on band n2

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -98 | -93 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -103 | -98 | / | / |
| LPF attenuation | 11.8 | 1 | 1 | / | / |
| LPF output (dBm) | -34.8 | -104 | -99 | / | / |
| Noise floor for basic REFSENS | / | / | / | -103 | / |
| NF increase due to AGC adjustment | / | / | / | 11.6 | / |
| Noise floor | / | / | / | -91.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -98 | / | / |
| ADC output (dBm) | / | / | / | / | -101.6 |
| SNR after ADC (dB) | / | / | / | / | -12(PCC) |
|  |  |  |  |  | -12.8(SCC) |

7.1.3 Analysis on Band n3

Band n3 is a similar case with band n2, 5+5 MHz configuration. The evaluated results are demonstrated in below tables.

Table 7.1.3-1: Link budget of Self-interference for 5+5 MHz on band n3

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -97 | -92.3 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -102 | -97.3 | / | / |
| LPF attenuation | 9 | 1 | 1 | / | / |
| LPF output (dBm) | -32 | -103 | -98.3 | / | / |
| Noise floor for basic REFSENS | / | / | / | -102 | / |
| NF increase due to AGC adjustment | / | / | / | 11.7 | / |
| Noise floor | / | / | / | -90.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -97.3 | / | / |
| ADC output (dBm) | / | / | / | / | -99.8 |
| SNR after ADC (dB) | / | / | / | / | -12.2(PCC) |
|  |  |  |  |  | -12.9(SCC) |

7.1.4 Analysis on Band n7

The band n7 evaluated results are demonstrated in below tables.

Table 7.1.4-1: Link budget of Self-interference for 10+5 MHz on band n7

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -94.8 | -98 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -99.8 | -103 | / | / | / |
| LPF attenuation | 17.2 | 1 | 1 | / | / | / |
| LPF output (dBm) | -40.2 | -100.8 | -104 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -99.8 | -103 | / |
| NF increase due to AGC adjustment | / | / | / | 6.1 | 6.1 | / |
| Noise floor | / | / | / | -93.7 | -96.9 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -103 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -104.7 |
| SNR after ADC (dB) | / | / | / | / | / | -6.4(PCC) |
|  |  |  |  |  | / | -6.4(SCC) |

7.1.5 Analysis on Band n26

The band n26 evaluated results are demonstrated in below tables.

Table 7.1.5-1: Link budget of Self-interference for 15+10 MHz on band n26

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -92.7 | -69.3 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -97.7 | -74.3 | / | / | / |
| LPF attenuation | 9.7 | 1 | 1 | / | / | / |
| LPF output (dBm) | -32.7 | -98.7 | -75.3 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -97.7 | -93.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.3 | 13.3 | / |
| Noise floor | / | / | / | -84.4 | -80.2 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -74.3 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -92.4 |
| SNR after ADC (dB) | / | / | / | / | / | -13.9(PCC) |
|  |  |  |  |  | / | -26.2(SCC) |

7.1.6 Analysis on Band n41

The main difference of evaluation results between TDD band and FDD band is due to there’s no Tx leakage fall into DL receiving channel since there’s no UL transmission during receiving mode.

Case 1, 10 + 10 MHz

Table 7.1.6-1 Link budget of Self-interference for 10+10 MHz on band n41

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | NA | / | / | / | / |
| Rx Antenna input | / | -94.8 | -94.8 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | NA | -99.8 | -99.8 | / | / |
| LPF attenuation | 0 | 1 | 1 | / | / |
| LPF output (dBm) | NA | -100.8 | -100.8 | / | / |
| Noise floor for basic REFSENS | / | / | / | -99.8 | / |
| NF increase due to AGC adjustment | / | / | / | 0 | / |
| Noise floor | / | / | / | -99.8 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -99.8 | / | / |
| ADC output (dBm) | / | / | / | / | NA |
| SNR after ADC (dB) | / | / | / | / | 0(PCC) |
|  |  |  |  |  | 0(SCC) |

Case 2, 40 + 40 MHz

Table 7.1.6-2 Link budget of Self-interference for 40+40 MHz on band n41

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | NA | / | / | / | / |
| Rx Antenna input | / | -88.6 | -88.6 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | NA | -93.6 | -93.6 | / | / |
| LPF attenuation | 0 | 1 | 1 | / | / |
| LPF output (dBm) | NA | -94.6 | -94.6 | / | / |
| Noise floor for basic REFSENS | / | / | / | -93.6 | / |
| NF increase due to AGC adjustment | / | / | / | 0 | / |
| Noise floor | / | / | / | -93.6 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -93.6 | / | / |
| ADC output (dBm) | / | / | / | / | NA |
| SNR after ADC (dB) | / | / | / | / | 0(PCC) |
|  |  |  |  |  | 0(SCC) |

7.1.7 Summary results for example bands

Evaluated results are summarized in the table below:

Table 7.1.7-1: Proposed ΔRIBNC for example bands on Rx RF chain mode

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 10 | 12 | 12.8 | FDD |
|  |  |  | Wgap = [10] | 25 | 0 | 0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 12 | 12.2 | 12.9 | FDD |
|  |  |  | Wgap = [20] | 25 | 0 | 0 |  |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55.0 | 32 | 6.4 | 6.4 | FDD |
|  |  |  | Wgap = [25] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.8 | 14.3 | FDD |
|  |  |  | Wgap = [10] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 | 24.9 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 13.9 | 26.2 | FDD |
| CA_n41(2A) | 15/15 | 10MHz + 10MHz | Wgap = 80.0 | N/A | 0 | 0 | TDD |
| CA_n41(2A) | 15/15 | 40MHz + 40MHz | Wgap = 20.0 | N/A | 0 | 0 | TDD |

7.1.8 Discussion on in-gap requirements

In last RAN4 meeting, it was agreed evaluate performance impact on the cases:

- image of in-gap interference overlaps with wanted carrier

- For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC

- image of in-gap blocking interference does not overlap with wanted carrier

Let’s take a look on the potential performance impact with the agreed assumptions. It can be seen for example, a DL NCCA composed of unequal CBW, such as 40+5MHz. When testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap interference may also fall into co-channel of CC0 that result in performance degradation as illustrated in Figure 7.1.8-1. The agreed assumption of Rx chain image rejection is 25dB and general assumption of receiver SNR is -1dB.

For the cases image of in-gap ACS/IBB1/IBB2/NBB overlaps with wanted CC, the maximum blocker to wanted signal power ratio shall not be larger than 26dB.

Figure 7.1.8-1: Illustration on impacts of the unwanted/interference image fall into co-channel CC

Then we also calculate the ACS case1 blocker level and the results are summarized in the table below:

Table 7.1.8-1: ACS case 1 blocker level for example bands on Rx RF chain mode

| CA configuration | SCS | UL PCC allocation | PCC REFSENS | PCC | ACS case1 PCC | SCC REFSENS | SCC | ACS case1 SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (PCC/SCC) | (LCRB) | 15KHz SCS | ΔRIBNC (dB) |  | 15KHz SCS | ΔRIBNC (dB) |  |
|  | (kHz) |  |  |  |  |  |  |  |
| CA_n2(2A) | 15/15 | 10 | -98 | 12 | -40.5 | -98 | 12.8 | -39.7 |
| CA_n3(2A) | 15/15 | 12 | -97 | 12.2 | -39.3 | -97 | 12.9 | -38.6 |
| CA_n7(2A) | 15/15 | 32 | -94.8 | 6.4 | -42.9 | -98 | 6.4 | -46.1 |
| CA_n25(2A) | 15/15 | 10 | -96.5 | 13.8 | -37.2 | -96.5 | 14.3 | -36.7 |
| CA_n25(2A) | 15/15 | 40 (RBstart = 176) | -79.5 | 6.5 | -36.5 | -96.5 | 24.9 | -26.1 |
| CA_n26(2A) | 15/15 | 5 (RBstart = 74) | -92.7 | 13.9 | -36.3 | -94.5 | 26.2 | -22.8 |
| CA_n41(2A) | 15/15 | N/A | -94.8 | 0 | -49.3 | -94.8 | 0 | -49.3 |
| CA_n41(2A) | 15/15 | N/A | -88.6 | 0 | -52.1 | -88.6 | 0 | -52.1 |

From the calculation we can see the ACS case 1 blocker level are mostly less than that of Tx leakage except baand n26. In the current TS38.101-1, it has been specified “The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active”. The UE design shall at least fulfil above requirement so it would be able to pass the -25dBm maximum acceptable absolute power of in-gap and out-of-gap interferers.

For the cases image of ACS/IBB/IBB2/NBB does not overlap with wanted CC, no relaxation would be needed based on evaluated ΔRIBNC on both PCC and SCC. Referring to current specification, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm

On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of interference may fall into the co-channel range of CC1. Though this may not be an issue for the conformance test since the conformance test does measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, this may be an implementation issue in the field as illustrated in Figure 7.1.8-2.

Figure 7.1.8-2: Illustration on impacts of the unwanted/interference image fall into the other CC

<End of proposed TP>

Reference:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2417201 WF on fragmented DL carriers study, RAN4#112bis

- R4-2420519 WF on NR FR1 DL fragmented carriers study

- R4-2418862 Discussion of impacts on UE RF requirements and DL performance impacts, RAN4#113, MediaTek