3GPP TSG-RAN WG4 Meeting # 114															       R4-2502954

Athens, GR, Feb 17 – Feb 21, 2025

Agenda item:			7.10.5

Source:	Samsung

Title:						WF on HPUE requirments for NR-NTN

Document for:		Approval

- Co-existence study

Issue 1-1: Results handling with different isolation distances

Agreement:

- Agree to capture all submitted results in the TR, and to separate results with different isolation.

- Use 1.5km isolation results to draw ACLR conclusion from co-ex study perspective.

Issue 1-2: NR-NTN Co-ex study results discussions

Agreement:

- For NR-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results.

Agreement:

- For NR-NTN PC1.5, it is observed that the required ACLR is less than 20.7 dB with 1.5km isolation distance assumed based on the simulation results.

Agreement:

- For NR-NTN PC1, it is observed that the required ACLR is less than 21.6 dB with 1.5km isolation distance assumed based on the simulation results.

Issue 1-3: IoT-NTN Co-ex study results discussions

Agreement:

- For NB-IoT based IoT-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results.

Agreement:

- For NB-IoT based IoT-NTN PC1, it is observed that the required ACLR is less than 26.2 dB with 1.5km isolation distance assumed based on the simulation results.

Issue 1-4: IoT-NTN ACLR definition

Agreement:

- For PC1/PC2 NB-IoT based IoT NTN, using same ACLR definition as current PC3, i.e., adjacent channel center frequency offset is ±2.5MHz and adjacent channel measurement bandwidth is 3.84MHz

- NR-NTN HPUE Tx requirements

Issue 2-1: NR-NTN PC2

Agreement:

- Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible

  - 1Tx handheld

  - 2Tx handheld

  - 1Tx non-handheld

  - 2Tx non-handheld

- If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19

Issue 2-3: NR-NTN PC1.5

Agreement:

- Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19.

- FFS on other cases if time is allowed

Issue 2-3: NR-NTN PC1

Agreement:

- RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19.

Issue 2-4-1: PC2 ACLR

Agreement:

- PC2 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-2: PC1.5 ACLR

Agreement:

- PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE

Issue 2-4-3: PC1 ACLR

Agreement:

- PC1 ACLR is [31 or 37] dBc for NR-NTN UE

- Further study averaging value, i.e. 34.

Issue 2-5: SEM

Agreement:

- Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline.

- Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202

Issue 2-6: MPR/A-MPR

Agreement:

- For PC 2 and PC 1.5 MPR:

- Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc.

- Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc.

- For PC 1 MPR:

- Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc:

  - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies

  - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec.

- Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc

- Aim for MPR/A-MPR analysis in #114bis for different power classes

- For PA model calibration:

  - DFT-s-OFDM QPSK 20 MHz

  - 100RB0

  - 4dB post PA loss

  - 1dB MPR

  - Carrier leakage 28dBc

  - IQ image:28 dBc

  - CIM3: 60 dBc

  - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB

  - SEM: existing PC3 SEM

  - EVM: 17.5%

- NR-NTN HPUE Rx requirements

Issue 3-1-0: General

Agreement:

- For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5.

Issue 3-1-1: RSD requirements for PC2 on n256

Agreement:

- For PC2 on n256: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx.

Issue 3-1-2: RSD requirements for PC2 on n254

Agreement:

- For PC2 on n254: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx.

- Open issues

Issue 3-1-3: RSD requirements for PC2 on n255

- Proposals

  - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek)

| Operating Band n255 | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| 1Tx | [0.4] | [0.4] | [0.5] | [0.5] |
| 2Tx | [0.8] | [0.8] | [0.9] | [0.9] |

  - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei)

    - PA noise as -117 dBm/Hz.

  - Option 3: Assuming IIP2 54dBm (Qualcomm)

| Operating Band n255 | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| 1Tx | 1.3 | 1.3 | 1.3 | 1.3 |
| 2Tx | 2.4 | 2.4 | 2.4 | 2.4 |

Way forward:

- Continue align the assumptions and further discuss above options.

Issue 3-2: RSD requirements for PC1.5

- Proposals

  - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo)

  - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek)

Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [1.8] | [1.8] | [1.8] | [1.8] |
| n255 | [2.4] | [2.4] | [2.5] | [2.5] |
| n254 | [1.8] | [1.8] | [1.8] |  |

Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [2.2] | [2.2] | [2.2] | [2.2] |
| n255 | [3.2] | [3.2] | [3.3] | [3.3] |
| n254 | [2.2] | [2.2] | [2.2] |  |

  - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson)

Way forward:

- Further discuss above options.

Issue 3-3: RSD requirements for PC1

- Proposals

  - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung)

  - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256.

Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |

Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3.7] | [3.7] | [3.7] | [3.7] |

  - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson)

Way forward:

- Further discuss above options.