3GPP TSG-RAN WG4 Meeting #114										                    R4-2502956

Athens, Greece, 17th-21th Feb, 2025

Agenda item:			6.1

Source:	Skyworks

Title:	WF on MSD rules for near-miss Rx mixing

Document for:	Approval

# 1			Introduction

CA_n7-n26 is the only band combination where band n26 may be affected by near-miss harmonic mixing. The MSD test point selection and MSD requirements were agreed based on [1], but the proposed rules for MSD test point design were never officially captured.

Also, it was found recently that:

- Band n28 direct-hit harmonic mixing MSD for CA_n28-n78 is wrong since band n28 may only be affected by near-miss harmonic mixing [2],

- Band n14 may be affected near-miss harmonic mixing for CA_n14-n30. This is a new requirement [2].

This WF captures MSD rules on near-miss harmonic mixing and invites companies to verify the proposed changes to MSD requirements [2].

# 2			WF

#### 2.1		MSD rules on near-miss harmonic mixing

Near-miss harmonic mixing MSD requirements are specified only for the UL1/DLx cases, where x=2,3,4,5,

- A maximum of one near-miss MSD test point may be specified:

  - For the smallest DL affected band (lower band) CBW,

  - For the smallest UL (higher band) CBW that ensures Rx harmonic mixing of the UL1 ACLR 1 lower channel ("ACLR1_low") or Rx harmonic mixing of the UL1 ACLR 1 higher channel ("ACLR1_high"),

  - For full UL band RB allocations.

- The UL/DL carrier frequency test conditions for near-miss harmonic mixing are specified using a footnote:

  - For DLx Rx harmonic mixing with the UL1 ACLR1_high:

    - “Note X” (eg. Table 7.3A.4-4 Note 10) applies: "The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band"

  - For DLx Harmonic mixing with the UL1 ACLR1_low:

    - A new “Note Y” is needed: "The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band”.

#### 2.2		Near-miss harmonic mixing MSD requirements

Interested companies are invited to review the following changes.

#### 2.2.1		Corrections to CA_n7-n26

Adopt the following corrections to the CA_n7A-n26A near-miss UL1/DL3 MSD test point.

Table 11: Proposed corrections to Band 26 PC3 UL1/DL3 near-miss MSD requirements.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n263 | 25 | 15 | 128 | 5 | 2.0 | NOTE 10 | UL1/DL3 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. |  |  |  |  |  |  |  |  |

#### 2.2.2		Corrections to CA_n28-n78

For Rel-17: Adopt the following corrections to the CA_n28A-n78A near-miss UL1/DL5 MSD requirements.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 (RBstart=0) | 5 | 8.2 | NOTE 6 | UL1/DL5 near-miss |
| NOTE 6: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Rel-18 and Rel-19: Adopt the following corrections to the CA_n28A-n78A near-miss UL1/DL5 MSD requirements.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 8.2 | NOTE 15 | UL1/DL5 near-miss |
| NOTE 15: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 10 | 15 | 50 | 5 | 13.0 | NOTE 10 | UL1/DL5 near-miss |
| NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

#### 2.2.3		Introduction of new MSD requirement for CA_n14-n30

Rel-18 and Rel-19: Introduce the following Band n14 PC3 MSD test point for CA_n14A-n30A with new Note “Y”.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n30 | n14 | 10 | 15 | 50 | 5 | 1.6 | NOTE Y | UL1/DL3 near-miss |
| NOTE Y: The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

REFERENCES:

[1] 		R4-2215516 MSD for CA_n7A-n26A, 3GPP TSG-RAN WG4 Meeting #104-bis-e, electronic meeting, Skyworks Solutions, Inc.

[2]		R4-2501987 On near-miss for Rx harmonic mixing, Skyworks Solutions, Inc., Anritsu, Qualcomm France, Oppo.