3GPP TSG-RAN WG4 Meeting #114										                    R4-2502961

Athens, Greece, 17th-21th Feb, 2025

Agenda item:			6.2.2

Source:	Murata, Skyworks, […]

Title:	WF on DC_(n)3AA and DC_(n)3CA

Document for:	Approval

# 1			Introduction

This is a WF on DC(n)3AA and DC(n)3CA supporting the relevant TP for TR37.719-11-11.

# 2			WF

#### 2.1		Background

A TP for TR37.719-11-11 was introduced [1] for the following intra-band contiguous band combinations, DC_(n)3AA BCS1 and DC_(n)3CA. The TP addressed the need for MSD analysis for the larger channel BW. Candidate test points for MSD analysis was proposed for DC_(n)3AA [2] and DC_(n)3CA [3].

#### 2.1		DC_(n)3AA

For DC_(n)3AA, a larger channel bandwidth of 40MHz was added assuming single switched UL according to [1]. The following concern was raised:

•  No need to introduce the MSD test point for BCS1 because there is no added value in adding multiple test points because the existing MSD for BCS0 already addresses the same CIM5 interference mechanism. The only difference between the test points is a larger aggressor UL BW and a smaller victim DL BW.

WF on DC_(n)3AA

•	Option 1: Do not introduce MSD test point for BCS1. Re-use the BCS0 MSD requirements by adding a footnote indicating this test point applies to both BCS1 and BCS0.

•	Option 2: Introduce the following BCS1 test point with the higher MSD value and replace existing BCS0 test point according to the band combination guidelines.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)3AA | 3 | N/A | 5 | N/A | 1837.5 | [TBD] |  |
|  | n3 | 1765 | 40 | 10 (RBstart = 184) | 1860 | N/A |  |

•	Other options not precluded

#### 2.2		DC_(n)3CA

For DC_(n)3CA, the proponent wishes to use simultaneous UL in addition to single switched UL. Furthermore, there exists multiple DL SCCs. The following concerns were raised:

•	No need to define test points for “CA” beyond “AA” assuming single switched UL. In intra-band non-contiguous case, for example CA_n25(3A) the maximum Wgap was provided as the test point for CA_n25(2A) to avoid the need for adding a test point for the added DL SCC.

•	The IMD landscape is different for simultaneous UL Vs single switched UL.

•	The TP [1] proposes to restrict UL DC_3A_n3A and DC_(n)3AA to support only single switched UL, but the fallback DC_3A_n3A is currently specified with single switched allowed, meaning that dual UL operation may be supported on this UL configuration. MSD test point selection requires clarification on the valid DC_3A_n3A UL configuration for DL DC_(n)3CA.

WF on valid DC_3A_n3A UL Configuration for DL DC_(n)3CA.

Proponents are invited to confirm which of the following two options is the valid assumption:

•	Option 1: DC_3A_n3A is restricted to only single switched UL operation for DL DC_(n)3CA.

•	Option 2: Single switched UL operation is allowed for UL DC_3A_n3A in DL DC_(n)3CA.

Interested companies are invited to propose MSD test points for either option if needed.

WF on MSD rules for DL SCCs for DC_(n)3CA

. •	Do not introduce new 2UL MSD test point for multiple DL SCCs as the existing 2UL MSD in TS38.101-3 addresses the self-IMD MSD introduced in the single DL CC. There is no new IMD information to be added.

•	Do not introduce new MSD test point for single switched uplink for multiple DL SCC provided the same interference mechanism occurs in the single DL SCC and the multiple DL SCC case.

•	Introduce MSD rules to address the above bullets.

•	Other options not precluded.

REFERENCES:

[1] 		R4-2500816 TP for TR 37.719-11-11 DC_(n)3, Samsung, KT, RAN4#114, Athens, Greece, February 17th-25th, 2025

[2] 		R4-2500820 MSD test point for DC_(n)3AA BCS1, Murata, RAN4#114, Athens, Greece, February 17th-25th, 2025

[3]		R4-25001087 MSD test point for DC_(n)3CA, Murata, RAN4#114, Athens, Greece, February 17th-25th, 2025