3GPP TSG-RAN WG4 Meeting #114										                     R4-2500039

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			6.6.2

Source:	Rogers, CableLabs

Title:	Comments on Fragmented Carrier Signalling

Document for:	Discussion

# Introduction

Fragmented carrier operation has been proposed with the objective of reducing the number of UE Rx chains required to handle intra-band non-contiguous CA when channel conditions permit. A study item was approved in RAN#104 and the objectives of the SID, as revised in RAN#106, are as follows [1]:

-	The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to one Rx RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx RF chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] ;

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. One Rx RF chain is not considered among inter-band DL carriers.

Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In the last meeting, companies were encouraged to begin discussions on fragmented carrier signalling, as captured below [2]:

#### Issue 2-4: Signaling related

WF:

Keep all options open. Companies are encouraged to provide feasibility analysis TPs with pros and cons on preferred signalling solutions study so that they can be discussed in next meeting.

The following sections will discuss some signalling issues arising from fragmented carrier discussions.

# Discussions

The use of fragmented carriers to increase the number of carriers a UE is able to aggregate will only be appropriate for certain operators under certain network conditions. In particular, given a UE, the operator must have access to:

- fragmented spectrum holdings in at least one band eligible for fragmented carrier operation,

- sufficient spectrum available in other bands to occupy more Rx chains than the UE has available,

- the flexibility to configure the UE with this spectrum in the context of channel state and competition for resources, and

- great enough DL demand from the UE to necessitate configuration in fragmented carrier mode.

Criteria 3 and 4 are dependent on the network conditions and traffic pattern at a given time and location. They constrain all networks to some degree. Criteria 1 and 2 pertain to the operator’s spectral resources and are far less likely to be satisfied by spectrum-poor operators. Since spectrum holdings change infrequently, an operator that does not satisfy these conditions in a certain licensing region is unlikely to do so for significant time thereafter.

Fragmented carrier usage will be limited to niche situations by some operators and restricted altogether by others.

The scope of the signalling discussions ought to include limiting any potential adverse effects when fragmented carrier is not in active use.

The focus of this document is on the effects of fragmented carrier on a UE’s CA combination capability reporting. In the discussion that follows, and in the context of a particular UE, consider the term “fragmented carrier combination” to mean a CA combination for which the UE configures at least one shared Rx chain in its implementation. Consider the term “legacy combination” to mean one for which the UE configures only partially shared or fully separate Rx chains. These terms must be defined in the context of a particular UE since distinct UEs may implement the same combination through different architectures.

A combination may be a “fragmented carrier combination” with respect to one UE and a “legacy combination” with respect to another.

As technology develops and the industry continues to identify occasions for aggregating spectrum, the combinations supported by UEs continue to proliferate. The opportunity for UEs to use fully shared Rx chains to support fragmented carrier operation would make feasible considerably more combinations. In theory, the number of combinations a UE can support is exponential in the number of carriers it can simultaneously support.

The process of a network selecting the most appropriate of a UE’s supported combinations for a given situation is becoming more complex.

A further challenge would arise if fragmented carrier combinations need not be distinguished from legacy combinations in a UE’s report to the network. In this case, since the network may not know the UE’s implementation of certain combinations, it would be subject to ambiguity.

Unless a UE must explicitly identify fragmented carrier combinations in its report to the network, the network may be unable to differentiate them from legacy combinations.

It is possible that fragmented carrier operation will deviate from legacy operation with respect to any of selectivity performance, PCell selection, fallback procedures, treatment of 4 Rx mandatory bands, or various other issues. Any resulting performance impact and additional complexity must be accommodated by the network. This issue would be complicated if the network were unable to differentiate between fragmented carrier and legacy combinations.

The presence of fragmented carrier combinations in a UE’s capability report would create complications for networks not intending to use fragmented carriers at that time.

When a network requires (additional) UE radio access capability information from a UE in RRC_CONNECTED, it sends a UECapabilityEnquiry and receives a response according to the procedure specified in Section 5.6 of TS. 38.331 [3]. It is through this process that a UE provides the network with a set of supported CA combinations.

The network is able to prioritize which CA combinations are listed as “candidate band combinations” by setting fields in the UE capability container capabilityRequestFilterCommon within the UECapabilityEnquiry. The network can filter by frequency bands, capabilities, etc. However, the list may be truncated or become bloated if the list of candidates meeting the filter criteria is too large. In the case of truncation, which bands are truncated is determined by UE implementation.

The network may receive a strict subset of candidate combinations from the UE, and, from its perspective, the choice of that subset may be unpredictable and uncontrollable.

The above concerns can be addressed by introducing a new field in capabilityRequestFilterCommon, specified in [3]. This field would allow the network to filter from the UE’s capability report any fragmented carrier combinations. For comparison, this behaviour is not dissimilar to the UE including as many combinations supporting UL Tx Switching as possible on receipt of the field uplinkTxSwitchRequest.

In the UECapabilityEnquiry message, include a new field in capabilityRequestFilterCommon allowing the network to filter fragmented carrier combinations from the capability report sent by the UE.

# Conclusion

In the previous section, we made the following observations:

Observation 1: Fragmented carrier usage will be limited to niche situations by some operators and restricted altogether by others.

Observation 2: The scope of the signalling discussions ought to include limiting any potential adverse effects when fragmented carrier is not in active use..

Observation 3: A combination may be a “fragmented carrier combination” with respect to one UE and a “legacy combination” with respect to another.

Observation 4: The process of a network selecting the most appropriate of a UE’s supported combinations for a given situation is becoming more complex.

Observation 5: Unless a UE must explicitly identify fragmented carrier combinations in its report to the network, the network may be unable to differentiate them from legacy combinations.

Observation 6: The presence of fragmented carrier combinations in a UE’s capability report would create complications for networks not intending to use fragmented carriers at that time.

Observation 7: The network may receive a strict subset of candidate combinations from the UE, and, from its perspective, the choice of that subset may be unpredictable and uncontrollable.

Proposal 1: In the UECapabilityEnquiry message, include a new field in capabilityRequestFilterCommon allowing the network to filter fragmented carrier combinations from the capability report sent by the UE.

# References

- RP-243002, Revised SID: Study on NR FR1 DL fragmented carriers

- RP-243001, Status Report: Study on NR FR1 DL fragmented carriers

- TS. 38.331 v18.3.0