3GPP TSG-RAN WG4 #114	R4-2500055

Athens, Greece, February 17-21, 2025

Agenda item:	7.22.3.2		Other RRM Requirements

Source: 	Qualcomm Incorporated

Title: 						On additional RRM requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112 the work on Rel-19 was started. Several agreements were made to identify RRM requirements and additional open issues have been identified [1].

In this paper we provide our views for the open issues identified in the way forward as defined in [1] in RAN4 #113.

# Discussion

## UE Reporting Enhancements for CJT Deployments

In the previous RAN4 meetings first discussions about accuracy requirements for delay and frequency offset reporting were started. Background is whether one or multiple samples are needed to provide sufficient accuracy for the reporting since, depending on the number of required samples, core or performance requirements are impacted in TS 38.133. To investigate the measurement performance simulation assumptions for delay and frequency offset reporting have been agreed [5].

CJT reporting enhancements supports NTRP  {1, 2, 3, 4} TRPs. The reports are relative to a reference TRP nref that is selected by the UE. Overall, the UE is configured with NTRP NZP-CSI-RS resources/resource sets with NTRP  {1, 2, 3, 4}. Further it has been agreed in RAN1 that the delay offset is uniformly spaced between 0 and AD and the frequency offset is uniformly spaced between between 0 and AFO. Since the reported delay and frequency offsets cannot be lower than zero, the UE needs to select the TRP with lowest estimated delay and lowest estimated frequency offset relative to the carrier frequency as the reference TRP nref. The upper limits AD and AFO and the number of uniform steps MD and MFO can be configured by the network. RAN1 agreed on AD = {0.5CP, CP}, MD = {32, 64, 128, 256} and AFO = {0.1ppm, 0.2ppm}, MFO = {16, 32, 256}.

Before we start looking into simulation results, RAN4 should define what accuracy requirements should be achieved. Based on the simulation assumptions in [5], we could assume a TDD system with a carrier frequency of 3.5 GHz. Under these assumptions, the quantization step sizes are Δt = 0.5 CP / 255 = 0.5 * 2.3 ms / 255 = 4.5 ns and Δf = 0.1ppm of 3.5 GHz / 255 = 1.37 Hz for the most accurate case of MD = 256 and MFO = 256. In the simulation assumptions, SNR = -3 dB, 0 dB and 3 dB was agreed.

Proposal 1: RAN4 should investigate whether a measurement accuracy of Δt = 4.5 ns and Δf = 1.37 Hz for delay offset, and frequency offset reporting at SNR = -3 dB, 0 dB and 3 dB can be achieved.

## Asymmetric DL sTRP / UL mTRP

In this contribution we provide our view to the open issues identified in RAN4 #113 based on the latest discussions in RAN1. RAN4 sent an LS to RAN1 in RAN4 #112bis with the question whether UL SSBs can transmit SSBs [3]. In RAN1 #119, RAN1 agreed to the following answer [4]:

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

In addition, in RAN1 #118bis it has been agreed for the DL sTRP / UL mTRP scenario that only one downlink reference timing is supported and applied to both TAGs.

Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In our understanding, the Reply LS implies three different scenarios

- Scenario 1: if PL offset is configured, the UL TRP does not transmit SSB

- Scenario 2: if PL offset is not configured, the UL TRP transmits SSB

- Scenario 3: if PL offset is not configured, the UL TRP does not transmit SSB

Scenario 3 does not seem to be excluded from the RAN1 LS, but it is not clear how the UE can transmit a signal to the UL TRP if no pathloss offset is provided and the UL TRPs do not transmit any downlink signal. Hence, we propose to exclude scenario 3 from all further considerations

Proposal 2: RAN4 should not consider the scenario that the UE is not configured with pathloss offset and UL TRPs do not transmit SSB (scenario 3). RAN4 should just focus on scenario 1 and scenario 2 as defined above.

Our interpretation of the RAN1 #118bis agreement that there is only one downlink timing reference is specific to scenario 1 and has not taken the existence of scenario 2 into account. We assume that it may be possible that RAN1 comes up with an additional agreement that two downlink reference timings are supported for scenario 2 when the UL TRP transmits SSB.

Observation 1: The agreement in RAN1 #118bis that there is only one downlink reference timing implies the scenario that PL offset is configured, and the UL TRP does not transmit SSB (scenario 1)

Observation 2: For the scenario that the UL TRP transmits SSB, a second downlink timing reference could still be defined by RAN1 (scenario 2).

In RAN4 #113, RAN4 agreed on the following way forward for the definition of the RRM requirements for 2TA enhancement:

Issue 4-1: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Alt 1: Define requirement for 2TA following the legacy principle.

- Alt 2: Wait for RAN1.

At least for scenario 2, RAN4 needs to wait further whether RAN1 extends the existing agreement for the downlink timing reference. For scenario 1, however, RAN4 can continue the discussion based on existing RAN1 agreements. We repeat our proposal from RAN4 #113 for scenario 1

Proposal 3: For scenario 2, RAN4 should wait further whether RAN1 extends the existing agreement for the downlink timing reference and introduces a second downlink reference timing.

Proposal 4: For scenario 1 with two TA, one for each TAG, the uplink transmission(s) timing take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

The RAN1 #118bis agreement above mentions autonomous timing adjustments. This topic was further discussed in RAN1 #119 without reaching a clear agreement. However, it seems clear that there is no RAN1 spec impact. In our understanding the above-mentioned autonomous timing adjustment is the gradual timing adjustment defined in TS 38.133. RAN4 could discuss how gradual timing adjustment should be applied in scenario 1 when there is just one downlink reference signal but two uplink TAGs. Our view is that the same gradual timing adjustment should be applied to both TAGs

Proposal 5: For scenario 1 gradual timing adjustment should be applied with the same value to both TAGs.

Regarding the UL TCI switching requirements, there was a discussion in RAN4 #113 whether to define UL TCI state switching requirements based on SRS

Issue 4-2: Whether to define UL TCI state activation requirement based on SRS?

Way forward:

- Option 1: Not define requirement (Samsung, HW, vivo)

- Option 2: RAN4 to define UL TCI state activation requirement based on SRS. (Xiaomi, Ericsson, Nokia)

- Option 3: Further discuss based on the feasibility of defining requirement and testing (QC, Apple, MTK)

The purpose of uplink TCI state switching is to define whether additional time is needed in the UE to apply the new uplink TCI state. When a pathloss reference signal is present, 38.133 distinguishes between known and unknown uplink TCI state. For a known TCI state, the switching time is just given by the MAC CE timeline. Additional time is needed in case the TCI state is not known that is different for FR1 and FR2. In case of FR2, additional time for Rx beam refinement is required. In case that there is no downlink pathloss reference signal (scenario 1), the network tells the UE either what uplink TCI state to use or it tells the UE that SRS beam sweeping is needed for beam refinement. In both cases, the UE just follows the command by the network and timeline is fully specified by RAN1/RAN2. From an RRM point of view, no additional time is needed to execute the TCI state switch or to execute the SRS beam sweeping. Potentially, additional requirements may be needed for the RF and/or the network for explicit uplink beam adjustment since there is no downlink correspondence when UL TRP does not transmit a downlink reference signal. But these additional requirements are outside RRM. Hence, we do not see a need to define to define RRM requirements for uplink TCI switching based on SRS.

Observation 3: For SRS based TCI state activation, the UE follows the command by the network and applies the timelines being defined by RAN1/RAN2. There is no need for defining RRM tests for SRS based TCI switching since no additional time is needed by the UE.

Proposal 6: RAN4 should not define RRM TCI state switching requirements based on SRS since no additional RRM capability in the UE is tested.

In case that the UL TRP does not transmit SSB (scenario 1), for FR2 SRS beam sweeping is required in the UE to let the gNB find out the best uplink beam and switch the uplink beam accordingly. Although we see no need to define TCI state switching requirements based on SRS in RRM, there are several challenges for SRS beam sweeping for FR2 in RF. In addition, how to support testing SRS beam sweeping from a TE point of view, may require further discussions in RAN5.

Observation 4: Whether or whether not additional requirements for SRS beam sweeping are needed, should be discussed in RF session or in RAN5.

Generally, it is not clear how well the system would work in case that the UL TRP does not transmit SSB (scenario 1) and continuous SRS beam sweeping is required. Hence, we propose that RAN4 focuses for FR2 only on the case that UL TRP transmits SSB (scenario 2). This is in line with the discussions in the RF room.

Proposal 7: RAN4 should focus for FR2 only on the case that UL TRP transmits SSB (scenario 2).

In case that the UL TRP transmits SSB (scenario 2), there is a downlink pathloss reference signal. In this case existing requirements can be reused, and no additional requirements need to be defined.

Proposal 8: In case that the UL TRP transmits SSB (scenario 2), existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined.

Issue 4-3: RRM core impacts of active uplink TCI state switching delay for unified TCI?

Way forward:

- Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

- Option 2: RAN4 to specify UL TCI state switching requirements.

- Option 3: FFS.

For the case that the UL TRP transmits SSB (scenario 2), it should be noted that the UL TRP may not transmit SIB1. Hence, the SSB transmitted by the UL TRP should be an NCD-SSB but not a CD-SSB. RAN4 should only focus on this case that the UL TRPs transmit NCD-SSBs.

Observation 5: Since the UL TRP may only transmit a SSB but no SIB1, the SSB from the UL TRP should be an NCD-SSB.

Proposal 9: RAN4 should only focus on the case that the UL TRPs transmit NCD-SSBs.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

UE reporting enhancement for CJT deployments

Proposal 1: RAN4 should investigate whether a measurement accuracy of Δt = 4.5 ns and Δf = 1.37 Hz for delay offset, and frequency offset reporting at SNR = -3 dB, 0 dB and 3 dB can be achieved.

Asymmetric DL sTRP/UL mTRP

Proposal 2: RAN4 should not consider the scenario that the UE is not configured with pathloss offset and UL TRPs do not transmit SSB (scenario 3)

Observation 1: The agreement in RAN1 #118bis that there is only one downlink reference timing implies the scenario that PL offset is configured, and the UL TRP does not transmit SSB (scenario 1)

Observation 2: For the scenario that the UL TRP transmits SSB, a second downlink timing reference could still be defined by RAN1 (scenario 2).

Proposal 3: For scenario 2, RAN4 should wait further whether RAN1 extends the existing agreement for the downlink timing reference and introduces a second downlink reference timing.

Proposal 4: For scenario 1 with two TA, one for each TAG, the uplink transmission(s) timing take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of the corresponding downlink frame of the reference signal associated with UL/joint TCI state.

Proposal 5: For scenario 1 gradual timing adjustment should be applied with the same value to both TAGs.

Observation 3: For SRS based TCI state activation, the UE follows the command by the network and applies the timelines being defined by RAN1/RAN2. There is no need for defining RRM tests for SRS based TCI switching since no additional time is needed by the UE

Proposal 6: RAN4 should not define RRM TCI state switching requirements based on SRS since no additional RRM capability in the UE is tested.

Observation 4: Whether or whether not additional requirements for SRS beam sweeping are needed, should be discussed in RF session or in RAN5.

Proposal 7: RAN4 should focus for FR2 only on the case that UL TRP transmits SSB (scenario 2).

Proposal 8: In case that the UL TRP transmits SSB (scenario 2), existing requirements for uplink TCI state switching can be reused, and no additional requirements need to be defined.

Observation 5: Since the UL TRP may only transmit a SSB but no SIB1, the SSB from the UL TRP should be an NCD-SSB.

Proposal 9: RAN4 should only focus on the case that the UL TRPs transmit NCD-SSBs.

# References

- R4-2420108, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4 #113, November 2024

- R4-2420209, Simulation assumptions for CJT calibration, RAN4 #113, November 2024

- R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, RAN4 #112bis, October 2024

- R1-2410870, Reply to RAN4 LS on UE RF Issues for Rel-19 MIMO Enhancement, RAN1 #119, November 2024

- R4-2420209, Simulation assumptions for CJT calibration, RAN4 #113, November 2024