3GPP TSG-RAN WG4 #114	R4-2500056

Athens, Greece, February 17-21, 2025

Agenda item:	7.25.2.1		On-demand SSB SCell operation

Source: 	Qualcomm Incorporated

Title: 	Further considerations for on-demand SSB SCell operation

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started and the work plan has been agreed in [1]. According to the work plan, the first focus for on-demand SSB is on MAC CE based SCell activation. In a second step, RAN4 may discuss other SCell activation scenarios if needed.

Several agreements have been made for on-demand SSB according to [2] and additional open issues have been identified in [3]. In this paper we provide our views for the issues identified in the way forward as defined in [2] in RAN4 #113.

# Discussion

In our opinion, the following topics need to be addressed with high priority since they are key for OD-SSB framework.

- Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

- Issue 1-3-1: Scenario 1 – OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

Other issues are lower priority or are straight forward in our opinion as Issue 1-1-3: L1 measurement before SCell activation.

## L1 Measurement before SCell Activation

In RAN4 #113, it was left open whether L1 measurements should be defined before SCell activation

Issue 1-1-2: L1 measurement before SCell activation

- Proposals

  - Option 1: L1 measurements are not supported for deactivated SCell.

  - Option 2: Wait RAN1’s further clarification on the applicable scenario.

L1 measurement reports are part of the CSI reporting according to 38.214 and according to 38.321, CSI reporting is not supported for deactivated SCell. Hence, it is a major change in RAN 1/2/4 to extend the measurement and reporting framework to L1. Since there is currently no agreement in RAN1 and RAN2 to introduce L1 measurements for deactivated SCell, we do not see how this could be done in RAN4 in the remaining time of Rel-19 even if RAN1 and RAN4 would still introduce it. Hence, RAN4 should not spend time to discuss this topic further.

Proposal 1: L1 measurements are not supported for deactivated SCell.

## OD-SSB based deactivated SCell L3 Measurement

For case #1 (no always-on SSB), several agreements have been reached in RAN4 #113. For the case of CSSF = 1, it has been agreed that for the fast measurement mode that the measurement periodicity is the OD-SSB periodicity. Secondly, it was agreed that after the fast time window, the UE shall meet the legacy deactivated SCell measurement requirements as long as OD-SSB is activated from UE perspective. In our understanding this means that after the fast measurement mode, the configuration of measCycleSCell determines the measurement periodicity.

Observation 1:  For the fast measurement mode, the measurement periodicity is the OD-SSB periodicity. After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirements if OD-SSB is activate.

Observation 2: The previous observation implies that after the fast measurement mode, the configuration of measCycleSCell determines the measurement periodicity.

During the discussion of the OD-SSB based measurement procedure, the following figure and sub-issues have been identified in [2].

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1: How to perform measurement after time window within one time OD-SSB activation/deactivation?

Sub-issue 2: What’s the expected minimum interval (T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined?

Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2?

The goal of the fast measurement mode is to provide a valid measurement report to the network. If one valid measurement report is sent there is no need any more to continue measurements in the fast mode. Continuing doing measurements in fast mode would increase UE power consumption unnecessarily.

To address the questions raised in Issue 1-2-3, it needs to be considered how OD-SSB can be used. RAN1 has made the following agreement in RAN1 #119:

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

Option 1 introduces an explicit indication of deactivation for the OD-SSB. This implies that the OD-SSB can be always present once triggered by the corresponding MAC CE. As logn as the network does not send the deactivation MAC CE, the OD-SSB will be present. Option 2 introduces a finite set of N OD-SSB bursts. That means after the MAC CE for the second option is sent, the end point of the OD-SSB is also already clear. RAN4 needs to address solutions for Issue 1-2-3 for both options.

In our understanding the main purpose of OD-SSB based L3 measurements for deactivated SCell is to provide a measurement report to the network to ensure the SCell is known. For that the UE follows the OD-SSB periodicity in the measurement. However, once the report is sent to the network, there is no need anymore to continue to measure in fast measurement mode. Afterwards it should be possible for the UE to fall back to legacy measurement mode to avoid an increase in UE power consumption. This is in particular important for the RAN1 option 1 to avoid that the UE always stays in fast measurement mode.

Observation 3: If one valid measurement report in fast mode is sent to the network there is no need any more to continue measurements in the fast mode. This is important when the network activates the OD-SSB but does not deactivate it again.

Once the measurement report has been sent to the network, the SCell is known. Or equivalently, in case that the SCell is already known to the network, a valid L3 measurement report has already been sent. Then additional measurement reports with faster measurement period are not needed anymore and the legacy requirements based on measCycleScell can be reused.

Observation 4: For a known deactivated SCell, no faster measurements are required by the network.

Proposal 2: Once the UE has sent a valid measurement report, the UE should be allowed to ignore the OD-SSB periodicity and fall back to legacy measurement mode.

If the OD-SSB is not deactivated by the network again, it becomes an always-on OD-SSB. This configuration possibility is allowed based on the RAN1 agreement above. In this case there is no need any more to measure again in the fast mode and OD-SSB periodicity should not be followed for measurements. Of course, the network can also send a deactivation command. In this case the status of the SCell may become unknown after some time and, if the deactivation of OD-SSB remains, the UE may even lose synchronization to the SCell. However, it is not clear why the network should allow such a transition of the SCell status.

We now want to consider the time T2 in Issue 1-2-3 above and want to address first the minimum time of T2. Once the UE has sent a valid measurement report and the SCell becomes known, the UE should be allowed to ignore the OD-SSB periodicity and continue to measure with a periodicity proportional to measCycleSCell to meet the legacy requirement that the UE is able to send valid measurement reports each 5*max(measCycleSCell, DRX cycle).

If the SCell is known, the network should not be allowed to trigger another measurement report in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) after a measurement report has been sent. Figure 2-1 illustrates the triggering of the fast measurement mode for the case that the SCell is already detected. This is the minimum time that allows to keep the SCell known. There is no need to trigger the fast measurement mode even faster since the Scell is already known..

Figure 2-1: Illustration of Triggering Fast Measurement Mode – Minimum Time T2

Proposal 3: The network should not be allowed to trigger another measurement in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) if the SCell is known (minimum time of T2 in Issue 1-2-3).

If the network deactivates OD-SSB again (option 1 in the RAN1 agreement) or only configured an finite set of OD-SSB bursts (option 2 in the RAN1 agreement), the SCell becomes unknown again if the minimum time T2 is exceeded. In this case the network may trigger another fast measurement time window. If the network waits for too long, the UE may also lose synchronization to the SCell again. It is not clear why the network should allow such a behaviour. However, in order to allow the UE to synchronize to a SCell for case 1 when only OD-SSBs and no always-on SSBs are present, the fast measurement window should be long enough to allow the UE to synchronize. Hence, it does not seem to be required to specify a maximum time T2.

Observation 5: It does not seem to be required to specify a maximum time T2.

Finally, we address the length of the fast measurement window. The most likely configuration seems to be that the network configures a set of OD-SSB that allow to send a L3 measurement report, to do a detection of PSS/SSS and an identification of SSB index. RAN4 defines a maximum number of SSBs that is allowed for these tasks. If our counting is correct, this is 13*(max(measCycleSCell, DRX cycle). This means that up to 13 SSBs are allowed and, hence, the network may consequently schedule blocks of 13 OD-SSB bursts for the tasks above. However, not all UEs may require the same number of OD-SSBs for these tasks. Hence, it should be allowed to the UE to fall back to legacy measurement and cell identification mode once the task is fulfilled. Proposal 2 already addresses the fall back to the legacy measurement mode. A similar fall back should apply for the cell identification.

Proposal 4: If cell identification is required by the UE in fast measurement mode, the UE should follow OD-SSB periodicity.

Proposal 5: The UE should be allowed to fall back to legacy mode for PSS/SSS detection and SSB index identification after the task has been accomplished.

For case #2 (always-on SSB is present), also OD-SSB can be triggered. It is still open in RAN4 what measurement periodicity should be taken for the L3 measurements for the deactivated SCell and several options are still under discussion.

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 1-1: The measurement periodicity is OD-SSB periodicity

  - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB

- Option 2: UE always follow legacy requirements based on measCycleSCell

- Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

Proposal 6: UE follows legacy requirements based on measCycleSCell if the SCell is already known.

In case that the SCell is unknown, a similar approach as in case 1 can be reused and the measurement periodicity should follow the periodicity of OD-SSB.

In case that the measurement time window only consists of OD-SSB as it would the case for case #1, the OD-SSB are periodic across a time span. That simplifies potential combination of measurements. In case that the measurement time window includes also always-on SSB, both always-on SSB and OD-SSB should fit into the same periodic pattern as in illustrated Figure 2-2. If this is the case, the processing of always-on SSB and OD-SSB is easier, and implementation gets simplified. If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

Observation 6: SSB processing and implementation in the UE gets simplified if both always-on SSB and OD-SSB fit into the same periodic pattern.

Proposal 7: If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

Figure 2-2: SSB Structure for Case #2 (always-on SSB and OD-SSB)

## MAC CE Timeline

In [4], RAN4 replied to a RAN1 LS regarding the MAC CE timeline as follows:

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be ready to process the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

Figure 2-1 illustrates the MAC CE timeline. A MAC CE triggered OD-SSB is received in slot #n and the corresponding PDSCH is acknowledged in slot #(n+m). In this slot the MAC CE is processed and the information contained in the MAC CE is available in the UE. This delay is unrelated to the OD-SSB and applies to each MAC CE. In case MAC CE triggers OD-SSB, the delay of m slots till HARQ feedback is sent to the network can remain unchanged.  Then additional 3 ms are left to the UE to get ready for the reception of the OD-SSB. Hence, T_min is the minimal time when the UE needs to be ready to process the OD-SSB. These additional 3 ms apply so far for A-TRS and aperiodic CSI-RS. However, the RAN1 LS extends the time of 3 ms when the UE is ready to process A-TRS or aperiodic CS-RS to SSB. Since A-TRS and aperiodic CSI-RS on the one side and SSB on the other side have usually different purposes, it cannot easily be assumed that the same timeline can be applied for these signals.

Figure 2-3: MAC CE Timeline for OD-SSB

It is reflected in the Reply LS to RAN1 that the UE may need more time to be ready to process OD-SSB. RAN4 should discuss this additional processing/preparation time for OD-SSB reception further.

Observation 7: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB

Proposal 8: RAN4 should discuss this additional processing/preparation time for OD-SSB reception further.

The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB. For example, in the Reply LS by RAN1 [7] there is the following comment.

Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?):

- …….

- …….

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

That means potentially there are up to 64 different OD-SSBs with different beams within an OD-SSB burst. In this case the UE needs to prepare for up to 64 different Rx beams. This may take more time than preparing for the reception of an OD-SSB burst with only 4 different beams. This scenario with only four beams in FR2 may take again more preparation time than for FR1 when no analog Rx beam needs to be prepared. RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

Observation 8: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Proposal 9: RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

## OD-SSB based SCell Activation

In [2] the following additional scenarios have been listed for which RAN4 should discuss	whether requirements for OD-SSB shall be defined.

Issue 1-5-1: The type of OD-SSB based SCell activation

FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios.

- Option 1: Direct SCell activation at SCell addition

- Option 2: Multiple SCells activation

- Option 3: Deactivated PUCCH SCell activation

- Option 4: Rel-17 Fast SCell Activation

- Option 5: Rel-18 SCell Activation with the L3 reporting during activation

In our view, RAN4 should focus first on option 2 since multiple SCell activation is a mandatory feature. The other Scell activation approaches are optional features in Rel-16, Rel-17 and Rel-18 with less relevance in our view given current deployment status. RAN4 should also distinguish in those features whether OD-SSB is used for deactivated SCell measurement or acceleration of SCell activation. For Rel-17 Fast SCell activation, an aperiodic TRS is applied. For that feature, OD-SSB should only be used for deactivated SCell measurement.

Proposal 10: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

Proposal 11: OD-SSB based SCell activation delay requirements should not be defined for Rel-17 Fast SCell Activation

## Miscellaneous

In RAN4 #112bis two LSs have been sent to RAN2 and RAN1 in [5] and [6] to get further clarification whether there is impact on SMTC configuration and what the relation between OD-SSB and always-on SSB in time, frequency, spatial relation is. While the reply from RAN2 has not yet been received, RAN1 provided a Reply LS in [7]. RAN1 has agreed that the frequency location of OD-SSB and always-on SSB is the same at least for the case where always-on SSB is not CD-SSB. RAN1 has also identified that the main use case [for OD-SSB] is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB, though the periodicity of on-demand SSB can be configured separately from the periodicity of on-demand SSB.

This answer from RAN1 fits to the main use discussed in RAN4 so far that always-on SSB and OD-SSB are on the same frequency, have the same offset by different periodicities and OD-SSB periodicity is smaller than the periodicity of always-on SSB. RAN4 should focus on this scenario first.

Proposal 12: RAN4 to focus first on the case that always-on SSB and OD-SSB are in the same frequency, have the same offset but different frequencies.

Since no answer from RAN2 has yet been received, discussions related to SMTC in RAN4 should be delayed till further clarification from RAN2 is received.

Proposal 13: Discussions related to SMTC should be delayed in RAN4 till further clarification from RAN2 is received.

We also want to point out that the current discussions only consider measurements for a single deactivated NES SCell. Whether and/or how to extend this is multiple NES SCell should be subject to a separate discussion / decision.

Observation 9: It is observed that the current discussions only consider measurements for a single deactivated NES SCell.

The existing specification in 38.133 allows interruptions during measurements on deactivated SCell when the measCycleSCell is larger than or equal to 640 ms. There are additional conditions on band combination when interruptions on any active serving cell are allowed during SCell activation/deactivation. RAN4 should investigate whether there is impact if the measurement period for OD-SSB is changed.

Proposal 14: RAN4 should investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated SCell and SCell activation/deactivation.

Since only one carrier to be activated was considered so far, the implicit assumption is that CSSF = 1. In case of carrier aggregation, CSSF becomes larger than one due to the assumption how many entities for search and measurement are available and how many measurements can be executed in parallel. General rule for determining CSSF could be that the carrier does not count for CSSF calculation if OD-SSB is not activated by MAC CE for this carrier assuming there is no always-on SSB. However, it needs to be considered how quickly an entity for measurement can be released and reallocated to another carrier. Hence, there may be a need to leave some guard time in between OD-SSBs on different carriers for CSSF reduction.

Observation 10: General rule for determining CSSF could be that the carrier does not count for CSSF calculation if OD-SSB is not activated by MAC CE for this carrier assuming there is no always-on SSB.

Observation 11: There may be a need to leave some guard time in between OD-SSBs on different carriers for CSSF reduction.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #113. Our observations and proposals are summarized as follows:

Proposal 1: L1 measurements are not supported for deactivated SCell.

Observation 1:  For the fast measurement mode, the measurement periodicity is the OD-SSB periodicity. After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirements if OD-SSB is activate.

Observation 2: The previous observation implies that after the fast measurement mode, the configuration of measCycleSCell determines the measurement periodicity.

Observation 3: If one valid measurement report in fast mode is sent to the network there is no need any more to continue measurements in the fast mode. This is important when the network activates the OD-SSB but does not deactivate it again.

Observation 4: For a known deactivated SCell, no faster measurements are required by the network.

Proposal 2: Once the UE has sent a valid measurement report, the UE should be allowed to ignore the OD-SSB periodicity and fall back to legacy measurement mode.

Proposal 3: The network should not be allowed to trigger another measurement in fast measurement mode within 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) if the SCell is known (minimum time of T2 in Issue 1-2-3).

Observation 5: It does not seem to be required to specify a maximum time T2.

Proposal 4: If cell identification is required by the UE in fast measurement mode, the UE should follow OD-SSB periodicity.

Proposal 5: The UE should be allowed to fall back to legacy mode for PSS/SSS detection and SSB index identification after the task has been accomplished.

Proposal 6: UE follows legacy requirements based on measCycleSCell if the SCell is already known.

Observation 6: SSB processing and implementation in the UE gets simplified if both always-on SSB and OD-SSB fit into the same periodic pattern.

Proposal 7: If the measurement time window includes both OD-SSB and always-on SSB, requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

Observation 7: The UE may need more time than given by the regular MAC CE timeline for aperiodic signals to be ready to process OD-SSB

Proposal 8: RAN4 should discuss this additional processing/preparation time for OD-SSB reception further.

Observation 8: The additional processing time may depend on the spectrum (FR1 or FR2), the time domain and/or frequency domain location of OD-SSB and always-on SSB and other configuration parameters of OD-SSB.

Proposal 9: RAN4 should discuss whether a single margin is introduced that allows to cover the worst-case preparation time or whether multiple margins are introduced that reflect different configuration scenarios.

Proposal 10: RAN4 should focus next on OD-SSB based SCell activation delay requirements for multiple SCell activation since multiple SCell activation is a mandatory feature.

Proposal 11: OD-SSB based SCell activation delay requirements should not be defined for Rel-17 Fast SCell Activation

Proposal 12: RAN4 to focus first on the case that always-on SSB and OD-SSB are in the same frequency, have the same offset but different frequencies.

Proposal 13: Discussions related to SMTC should be delayed in RAN4 till further clarification from RAN2 is received.

Observation 9: It is observed that the current discussions only consider measurements for a single deactivated NES SCell.

Observation 9: It is observed that the current discussions only consider measurements for a single deactivated NES SCell.

Proposal 14: RAN4 should investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated SCell and SCell activation/deactivation.

Observation 10: General rule for determining CSSF could be that the carrier does not count for CSSF calculation if OD-SSB is not activated by MAC CE for this carrier assuming there is no always-on SSB.

Observation 11: There may be a need to leave some guard time in between OD-SSBs on different carriers for CSSF reduction.

# References

- R4-2112508, Work plan for R19 NES, RAN4 #112, August 2024

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 ##113, November 2024

- R4-2418282, Topic summary for [113][224], Netw_Energy_NR_enh_Part1, RAN4 #113, November 2024

- R4-2420192, Reply LS on timeline for On-demand SSB operation on SCell, RAN4 #113, November 2024

- R4-2416911, LS on SSB adaptation, RAN4 #112bis, October 2024

- R4-2416913, LS on SSB relation in on-demand SSB and SSB adaptation on SCell, RAN4 #112bis, October 2024

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN1 #119, November 2024

- R4-2420084, Ad-hoc minutes for Netw_Energy_NR_enh, RAN4 #113, November 2024