3GPP TSG-RAN WG4 Meeting #114	R4-2500061

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On CLTM RRM requirements

Agenda item:	7.27.2.3

Document for:	Discussion

Introduction

In this contribution we continue the discussion about RRM requirements for CLTM. In the discussion we take into account the agreements RAN2 has made in the Orlando meeting:

Agreements on C-LTM: - The triggering condition of conditional LTM can be based on L3 measurement. - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - The L1 execution condition of a candidate cell is associated to only one triggering event. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. - To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. - The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer. - For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Discussion

Scope of conditional LTM requirements

As there was no related RAN2 agreement before the last RAN4 meeting, it was left open whether to define requirements for L3-based CLTM:

Issue 1-1-2: Measurement requirements on conditional LTM < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting.

RAN2 agreed in the last meeting that condition for CLTM cell switch may also be L3-based. Therefore, it is logical for RAN4 to define cell switch delay and other relevant requirements for this case.

Define CLTM requirements for the case when condition to trigger cell switch is L3 based.

Regarding the detailed scenarios to be supported, it was agreed in RAN4#112bis to support the same scenarios as for Rel-18 LTM:

Issue 1-1-1: The applicable scenarios for conditional LTM RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.. < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

What remained open was whether to support conditional cell switch to a candidate cell that is configured as UE’s current deactivated SCell. Currently there are no L1 measurement requirements for deactivated SCells as there was no time to define such requirements within the Rel-18 mobility WI. Hence, if the CLTM condition is L1-based, it is not possible to define a condition evaluation delay for the UE due to the lack of related measurement requirements. Considering the already high RAN4 workload, we think defining new measurement requirements for deactivated SCells in Rel-19 CLTM objective is not feasible, unless specifically listed as an objective in the WID, which it currently isn’t. Hence, we propose to leave this scenario out of the RAN4 scope.

Defining new measurement requirements to support any missing scenarios within the CLTM objective is not mentioned in the WID, and hence should be avoided.

RAN4 not to define CLTM cell switch delay requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based.

However, as mentioned earlier in this paper, RAN2 agreed in the last meeting that the condition for cell switch may also be L3 based. SSB-based L3 measurement delay requirements are defined for deactivated SCells. Therefore, we think this scenario can be supported by the Rel-19 CLTM cell switch delay requirements.

CLTM requirements to cover the case of target cell being a deactivated SCell, when the condition to trigger cell switch is based on L3 SSB measurements.

Early synchronization and TCI state activation

Issue 1-2-1: Early UL Synchronization RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.. For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK) For PDCCH ordered RACH: < Agreement > - PDCCH ordered RACH is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new PDCCH ordered RACH procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

RAN2 has agreed that PDCCH-ordered early TA acquisition and UE-based TA estimation are supported for conditional LTM.

For UE-based TA measurement:

Since the issue of defining RRM requirements for UE-based TA measurement was already discussed for Rel-18 LTM, we repeat here our discussion and proposal from previous RAN4 meetings, which can apply for both LTM and CLTM.

We have no strong preference whether RAN4 should define requirements for UE based TA measurement in Rel-19. However, if the requirements for the feature are defined in Rel-19, there are multiple critical issues from the network point of view that need to be addressed. These are discussed in detail in our earlier contribution [1] and summarized below.

- No Interruptions due to UE-based TA measurement: SSB-based L1-RSRP measurements specified for LTM do not cause interruptions. Therefore, UE-based TA measurements shall not cause interruptions either.

- Existing transmit timing accuracy requirements apply: If UE is using UE-based TA measurements, it shall not deviate from the existing requirements for any uplink transmission.

- Reference signals to use: It is not visible to the network when, and based on which measurements, the UE performs UE based TA estimation. This needs to be clarified.

- TA acquisition time for UE-based TA measurements: Without the information on how long TA acquisition takes, the network will not know when it can send the cell switch command with the assumption that the UE has the TA.

- Accuracy of the UE-estimated TA: RAN4 should discuss timing requirements for the UE estimated TA and how to verify that the UE estimated TA is accurate enough.

- Applicability of the UE based TA estimation requirements: Since Rel-18 LTM also supports UE based TA estimation (in RAN2 spec), but RAN4 requirements do not exist, we would prefer the requirements for UE based TA estimation to be defined in a way that the requirements can apply both for CLTM and Rel-18 LTM.

If RAN4 agrees to define requirements for UE based TA estimation in Rel-19, at least the issues listed below need to be taken into account.

  - Any UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

  - Existing transmit timing accuracy requirements shall apply.

  - Which measurements are expected to be used when performing UE-based TA measurements.

  - How long the acquisition of the UE-based TA takes after candidate cell configuration, to the point where UE is expected to have estimated the TA.

  - How to ensure the accuracy of the UE-estimated TA.

  - Requirements to cover not only CLTM but ideally also Rel-18 LTM.

For PDCCH-ordered RACH:

The latest RAN2 agreements have introduced some updates to the PDCCH-ordered RACH procedure for CLTM. RAN2 has agreed that: (1) the Early TA is signalled to the UE from the source cell and not from the candidate cell, (2) the network can inform the candidate cell’s TA information to UE via new MAC CE, which includes the TA value to be used when UE switches to that candidate cell during CLTM, and (3) candidate cell TA is maintained by a new timer.

We believe that these updates have no impact on the existing PDCCH ordered RACH delay requirements, which end at preamble transmission. There is also no need to update the interruption requirements. However, it is not clear yet whether the introduction of the new MAC CE with the candidate cell’s TA information can impact the cell switch delay requirement for CLTM. Hence, we propose to wait for more RAN1/2 agreements before deciding about the need for new or edited requirements.

No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Issue 1-2-2: Early TCI state activation RAN2#127bis - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.. < Agreement > - Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

RAN2 agreed to reuse the Rel-18 TCI state activation procedure for the Rel-19 early TCI state activation in CLTM. Based on this agreement, we do not see a need to change the existing TCI state activation delay requirements for CLTM (section 8.25). However, we believe that RAN4 should follow any new agreements that can update the activation procedure and/or introduce more than one option for early TCI state activation and study their potential impact on the requirements.

No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Conditional LTM delay

The starting point of conditional LTM has remained open for the first two meetings of this WI:

Issue 1-3-2: The starting point of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. - Proposal 2: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. - Proposal 3: Similar as event-triggered L1 reporting. The CondEvent is met over the air. - Proposal 4: Wait for more RAN2 agreements.

After the last RAN2 meeting, the situation remains the same as before: There have been no new agreements made by RAN2 regarding the starting point of condition evaluation. Earlier RAN2 agreements stated that the source cell sends the conditional LTM configuration to the UE via RRCReconfiguration, which includes the LTM candidate configurations and the corresponding execution conditions. However, it is still unclear whether the condition evaluation begins at the time the UE receives the configuration. Therefore, we think that RAN4 cannot make an agreement about the starting point yet.

Wait for more RAN2 agreements regarding the starting point of the CLTM cell switch delay.

Issue 1-3-4: The components of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. - Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. - Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. - Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements. - Proposal 7: Wait for RAN2 progress.

The lack of clear starting point of course also leads to RAN4 not being able to agree on the details of the delay and the complete delay equation. However, we think discussion on some of the delay parameters is already possible, at least on high level. We have included such discussion under each related issue from the last meeting’s way forward.

Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay.

Issue 1-3-5: TRRC < Way forward > FFS the following proposal: - Proposal 1: TRRC is the RRC procedure delay.

The necessity of TRRC or similar component depends on the starting point of the delay. Hence, it is too early to agree whether this component is to be included.

Too early to agree whether TRRC is needed.

Issue 1-3-6: TEvent_DU < Way forward > FFS the following proposals: - Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - Proposal 2: Wait for more RAN2 progress. - Proposal 2a: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU. - Proposal 3: Wait for more RAN2 agreements.

The definition of Tevent_DU in CHO is from the point UE successfully decodes the CHO command until the condition becomes met over the air. Some companies have proposed to reuse this definition for CLTM. However, starting point of condition evaluation remaining open, it is not possible to define a starting point for Tevent_DU either. Furthermore, it is still not fully clear when and how early DL sync and UL sync happen in CLTM, and whether there are different options here. Hence, we think it would be difficult to come up with a definition for Tevent_DU that would not need any revision later. Hence, we propose to postpone this discussion until the whole CLTM procedure becomes clearer.

Wait for further RAN2 agreements before agreeing on the definition of Tevent_DU.

Issue 1-3-7: Tmeasure < Way forward > FFS the following proposals: - Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts. - Proposal 1a: - If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - Proposal 1b: If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. - Proposal 1c: FFS on L3 measurement period depending on the conclusion of whether L3 based measurement is considered in R19 C-LTM. - Proposal 2: Wait for more RAN2 progress.

The conditions to be supported as the baseline according to the current RAN2 agreements are CondEvent A3 and CondEvent A5. The definition of these conditions for CHO is:

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell

CondEvent A5: PCell becomes worse than absolute threshold1 AND conditional reconfiguration candidate becomes better than another absolute threshold2

As can be seen, both conditions take into account both the serving cell (or beam) quality and the candidate cell (beam) quality. Therefore, to be able to realize that the condition is met, UE may need to measure both the serving cell and the candidate cell after the condition becomes met over the air. Hence, Tmeasure needs to consider both candidate cells/beams. The existing L1 measurement delay can be used as the baseline, but the delay may be impacted depending on whether the UE is evaluating only the target cell or also other candidate cells when the condition becomes met. This may depend on further RAN2 agreements, which RAN4 of course needs to take into account in the definition of Tmeasure, if there is an impact.

When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure.

When the condition is L3-based, we think the duration of Tmeasure can be defined in the same way as for CHO. However, the definition may need to be edited if needed based on any new RAN2 agreements.

When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

Issue 1-3-8: TCLTM_execution / TLTM_execution < Way forward > FFS the following proposal: - Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

Regarding TCTM_execution, which is proposed to be based on the definition of TCHO_execution, the impact of this parameter is in Rel-18 covered through TLTM_RRC-processing. There is no need to duplicate this parameter. The definition of TLTM_RRC-processing can of course be edited to better fit the CLTM execution procedure, but there is no need to add an additional parameter to cover the delay for UE processing the LTM candidate cell configuration.

TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay.

Issue 1-3-9: Tinterrupt < Way forward > FFS the following proposals: - Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. - Proposal 2: It highly depends on further discussion on procedure design in RAN2 group. - Proposal 3: The term Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure. - Proposal 4: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Issue 1-3-10: The components of Tinterrupt < Way forward > FFS the following proposal: - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch.

The components that are included in Tinterrupt depend obviously on the components that will be included in the CLTM cell switch delay. Similar to cell switch delay, we do not think it is possible to agree on the detailed components before the design is more complete, but we would like to suggest that RAN4 discusses at least the need of including some of the Rel-18 components in the interruption, if included in the delay.

Such components are TLTM_RRC-processing, Tfirst-RS and TRS-proc. Independent of whether we will use these same components or somewhat edited components, we think it is reasonable to assume that these components do not need to be considered as part of the interruption. When the steps related to these components –ASN.1 decoding and fine DL synchronization – are done before the cell switch in LTM, the procedures do not cause an interruption. We proposed in Rel-18 that the components would not cause an interruption during the cell switch either, but at that time it was argued by some companies that since the network sends the cell switch command, the UE will not be scheduled after receiving the cell switch command anymore. This is not the case in CLTM, where the cell switch decision happens at the UE side. Hence, we think ASN.1 decoding and DL synchronization components, if included in the cell switch delay, should not be part of Tinterrupt.

ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Subsequent Conditional LTM

Regarding subsequent LTM, we think the situation is the same as for the initial cell switch delay. It is not possible to agree on the starting point based on the current RAN2 agreements, and the procedure as such is not so complete that defining requirements makes sense. Hence, for subsequent CLTM requirements, we propose to wait for more agreements for RAN2. We think defining the delay after the details are clear will be straightforward based on the discussion about the initial cell switch delay.

Wait for more RAN2 agreements before defining the requirements for subsequent CLTM.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: Define CLTM requirements for the case when condition to trigger cell switch is L3 based.

Observation 1: Defining new measurement requirements to support any missing scenarios within the CLTM objective is not mentioned in the WID, and hence should be avoided.

Proposal 2: RAN4 not to define CLTM cell switch delay requirements for a deactivated SCell, when the condition to trigger cell switch is L1-based.

Proposal 3: CLTM requirements to cover the case of target cell being a deactivated SCell, when the condition to trigger cell switch is based on L3 SSB measurements.

Proposal 4: If RAN4 agrees to define requirements for UE based TA estimation in Rel-19, at least the issues listed below need to be taken into account.

I.	Any UE-based TA measurements shall not cause any impact to the network scheduling, for example, interruptions.

II.	Existing transmit timing accuracy requirements shall apply.

III.	Which measurements are expected to be used when performing UE-based TA measurements.

IV.	How long the acquisition of the UE-based TA takes after candidate cell configuration, to the point where UE is expected to have estimated the TA.

V.	How to ensure the accuracy of the UE-estimated TA.

VI.	Requirements to cover not only CLTM but ideally also Rel-18 LTM.

Proposal 5: No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Proposal 6: No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Proposal 7: Wait for more RAN2 agreements regarding the starting point of the CLTM cell switch delay.

Proposal 8: Wait for more RAN2 agreements before agreeing on the exact components of the CLTM cell switch delay.

Proposal 9: Too early to agree whether TRRC is needed.

Proposal 10: Wait for further RAN2 agreements before agreeing on the definition of Tevent_DU.

Proposal 11: When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure.

Proposal 12: When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

Proposal 13: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay.

Proposal 14: ASN.1 decoding and validity check is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Proposal 15: Fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Proposal 16: Wait for more RAN2 agreements before defining the requirements for subsequent CLTM.

- References

[1] R4-2405533, On remaining aspects of LTM core requirements, Nokia, RAN4#110bis, Changsha, China, April 15-19, 2024