3GPP TSG-RAN WG4 Meeting # 114	R4-2500069

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.2.2

Source: 	Nokia

Title: 	TP to TR 38.922: Corrections and clarifications on IMT parameters for 7125 to 8400 MHz frequency range

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range.

The reply LS from RAN4 to ITU-R on Parameters for 7125 to 8400 MHz was approved at TSG RAN4#112 [2]. In [2], the AAS BS beamforming antenna characteristics for 4 deployment scenarios, including macro and small cell, are provided in Table 4 of [2], while the spectral masks for non-AAS and AAS BS are provided in Table 1 by referring to Table 1A, Table 1B, and the tables for Category B WA BS in clauses 6.6.4 and 9.7.4.2 of TS 38.104 [3].

Tables 1, 1A, 1B, and 4 in [2] are extracted below and the relevant parts are highlighted:

Table 1: IMT technology related parameters in 7125 – 8400 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | 100 MHz typical (Note 2) |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 Will be derived from Channel Bandwidth, see [1], § 5.3.2. |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB |  | 56 dB typical |
| 4.2 | Spectral mask (dB) | Category A: (Note 3) See table 1A (Wide Area BS) (ΔfOBUE = 40 MHz) Category B: (Note 3) See [1], § 6.6.4, Tables 6.6.4.2.2.1-2a and 6.6.4.2.2.1-2b (Wide Area BS) (ΔfOBUE = 40 MHz) | Category A: (Note 3) See table 1B (Wide Area BS) (ΔfOBUE = 100 MHz) Category B: (Note 3) See [1], § 9.7.4.2 and related Tables 6.6.4.2.2.1-2a and 6.6.4.2.2.1-2b (Wide Area BS) (ΔfOBUE = 100 MHz) | See [2], § 6.5.2.2 |
| 4.3 | ACLR | 38 dB |  | 26 dB |
| 4.4 | Spurious emissions | Category A: (Note 3) See [1], § 6.6.5, Table 6.6.5.2.1-1. Category B: (Note 3) See [1], § 6.6.5, Table 6.6.5.2.1-2. |  | See [2], § 6.5.3. |
| 4.5 | Maximum output power | See [1], § 6.2. | See Item No. 1.13 in Table 4 for typical values | 23 dBm typical (Note 2) |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 6 dB (Wide Area BS) 11 dB (Medium Range BS) 14 dB (Local Area BS) For BS class definitions, see [1], § 4.4 |  | 13 dB |
| 5.2 | Sensitivity (dBm) | To be specified | To be specified | To be specified |
| 5.3 | Blocking response | In-band blocking level: See [1], § 7.4.2, Table 7.4.2.2-1 Out-of-band blocking level: See [1], § 7.2.2, Table 7.5.2-1a (band n104) ΔfOOB = 60 MHz (Note 4) | In-band blocking level: See [1], § 10.5.2.2 Out-of-band blocking level: See [1], § 10.6.2.1 ΔfOOB = 100 MHz (Note 4) | See [2], § 7.6 Tables 7.6.2-3 and 7.6.2-4, 7.6.3-3 and 7.6.3-4 for blocking levels and § 7.7 Tables 7.7-1a and 7.7-2 for spurious response |
| 5.4 | ACS | 42 dB |  | 32 dB |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

Note 1:	 Non-AAS can be used for Urban small cell/Micro cell scenario and indoor scenario.

Note 2:	 Refer to [3] for more information on other values for channel bandwidth and maximum output power.

Note 3:	Base station Operating band unwanted emissions define all unwanted emissions in the supported downlink operating band plus the frequency ranges extending ΔfOBUE above and ΔfOBUE below each band. Base station Unwanted emissions outside of this frequency range are limited by the spurious emissions requirement.

Note 4:	Base Station In-band blocking applies in the supported uplink operating band plus the frequency ranges extending ΔfOOB above and ΔfOOB below each band, excluding the downlink frequency range in case of an FDD operating band. Out-of-band blocking applies from 1 MHz to 12.75 GHz, excluding the in-band blocking frequency range, but including the downlink frequency range in case of an FDD operating band. Requirements are defined assuming a receiver desensitization of 6 dB.

References used in the Table:

[1]	3GPP TS 38.104 v.18.6.0, “NR; Base Station (BS) radio transmission and reception”

TABLE 1A

Non-AAS BS Spectral mask (Operating band unwanted emissions limits) for 7125 - 8400 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 20MHz | -7dBm-720f_offsetMHz-0.05 | 100 kHz |
| 20 MHz £ Df < min(40 MHz, Dfmax) | -14 dBm | 100 kHz |
| 40 MHz £ Df £ Dfmax | -13 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 40 MHz outside the downlink operating band. |  |  |

TABLE 1B

AAS BS Spectral mask (Operating band unwanted emissions limits) for 7125 - 8400 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -5 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -4 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Table 4: Beamforming antenna characteristics for IMT in 7125 to 8400 MHz

|  |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  | Refer to Recommendation ITU-R M.2101 | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | N/A |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 16 | 8 × 16 | 8 × 8 | N/A |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |
| 1.7a | Number of element rows in sub-array | 3 | 3 | N/A | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | 0.7 of wavelength for V | N/A | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | N/A | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | N/A |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 22 | 22 | 16 | N/A |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | N/A |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 | N/A |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | N/A | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 78.3 | 78.3 | 61.5 | N/A |

Moreover, only the Category A WA BS spectral masks are included in clause 5.2.1.2 of TR 38.922 [4]. This would create ambiguity that the WA spectral masks are also applicable for the small cell BS which should not be the case. This contribution provides a text proposal to update clause 5.2.1.2 of TR 38.922 to avoid the ambiguity.

# 2.	Text proposal

<Start of text proposal>

#### 5.2.1.2	Spectral mask

Both Category A and B spectral mask are specified. Since the frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104, it is proposed that existing spectral mask (Category B) for band n104 in TS 38.104 subclause 6.6.4 for conducted requirements and subclause 9.7.4.2 for radiated requirements are applicable for the range. Category A limits for Wide Area BS are specified in Table 5.2.1.2-1 for non-AAS BS and in Table 5.2.1.2-2 for AAS BS in macro cell scenarios.

Table 5.2.1.2-1: Wide Area BS operating band unwanted emission limits for non-AAS BS (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 20MHz | -7dBm-720f_offsetMHz-0.05 | 100 kHz |
| 20 MHz  f < min(40 MHz, fmax) | -14 dBm | 100 kHz |
| 40 MHz  f  fmax | -13 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 40 MHz outside the downlink operating band. |  |  |

Table 5.2.1.2-2: Wide Area BS operating band unwanted emission limits for AAS BS (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -5 dBm | 100 kHz |
| 100 MHz  f  fmax | -4 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

<End of text proposal>

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[3]	3GPP TS 38.104 v18.8.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	3GPP TR 38.922 v1.0.1: “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.