3GPP TSG-RAN WG4 Meeting # 114	R4-2500070

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.2.3

Source: 	Nokia

Title: 	BS in-band blocking requirements for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1.	Introduction

The revised SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#106 [1]. The SI was extended to TSG RAN#107 to complete the open issues in [2].

#### 2.4.2 Remaining Open issues - Resolve BS blocking in TR according to R4-2420330 - Final corrections and alignments to the TR prior to RAN approval, as necessary.

The topic on BS blocking for 14800 to 15350 MHz frequency range was discussed at TSG RAN4#113 [3] and the WF was agreed [4].

This contribution further discusses the BS in-band blocking requirements based on the agreed WF and the simulation results provided in this contribution.

# 2.	Discussion

2.1		BS antenna parameters for Urban Macro

It is stated in the topic summary [3]:

Issue 3-2-4: Observed Blocking level (Urban macro) - Proposals - Option 1: 99.99th percentile is -48dBm for 1 user scheduled or -39dBm for 3 users scheduled for FR1 like UE (Nokia) - Option 2: 99th percentile is -63dBm (Ericsson)

The simulation results in [5] and [6] have shown that, using the agreed simulation assumptions as recorded in TR 38.922 [7], the 99.99%-tile received blocking signal power levels with sub-array and beamforming gains are less than -48 dBm and -39 dBm, respectively, with 1 and 3 UE scheduled per slot for FR1 like UE. On the other hand, the simulation results in [8] have shown the 99.99%-tile received blocking signal power levels with sub-array gain but without beamforming gain would be in the range of -62 dBm.

Currently, the in-band blocking requirements are specified in TS 38.104 [9] as conducted requirements applicable at the antenna connector or TAB connector, as well as radiated requirements applicable at the RIB. However, non-AAS BS architecture section has been removed as not applicable for 14800 to 15350 MHz frequency range [10]. Therefore, the received blocking signal power levels at the TAB connector (i.e., with sub-array gain but without beamforming gain) and RIB (i.e., without sub-array or beamforming gain) should be considered for the in-band blocking requirements for 14800 to 15350 MHz frequency range.

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e., with sub-array gain but without beamforming gain) with 1 UE scheduled per slot in un-coordinated and coordinated operations are provided, respectively, in Figure 1 below. The 99.99%-tile received blocking signal power levels are less than -55 dBm and -56 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

(a) 64x24 BS, FR1 like UE								(b) 64x24 BS, FR2 like UE

Figure 1: Urban Macro received blocking signal power at TAB with 1 UE scheduled per slot

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e., with sub-array gain but without beamforming gain) with 3 UE scheduled per slot in un-coordinated and coordinated operations are provided, respectively, in Figure 2 below. The 99.99%-tile received blocking signal power levels are less than -52 dBm and -60 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

(a) 64x24 BS, FR1 like UE								(b) 64x24 BS, FR2 like UE

Figure 2: Urban Macro received blocking signal power at TAB with 3 UE scheduled per slot

The urban macro simulation results of the victim BS received blocking signal power at the RIB (i.e., without sub-array or beamforming gain) with 1 UE scheduled per slot in un-coordinated and coordinated operations are provided, respectively, in Figure 3 below. The 99.99%-tile received blocking signal power levels are less than -62 dBm and -61 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

(a) 64x24 BS, FR1 like UE								(b) 64x24 BS, FR2 like UE

Figure 3: Urban Macro received blocking signal power at RIB with 1 UE scheduled per slot

The urban macro simulation results of the victim BS received blocking signal power at the RIB (i.e., without sub-array or beamforming gain) with 3 UE scheduled per slot in un-coordinated and coordinated operations are provided, respectively, in Figure 4 below. The 99.99%-tile received blocking signal power levels are less than -59 dBm and -51 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

(a) 64x24 BS, FR1 like UE								(b) 64x24 BS, FR2 like UE

Figure 4: Urban Macro received blocking signal power at RIB with 3 UE scheduled per slot

In all simulated scenarios, the currently specified -43 dBm in-band blocking requirement for Wide Area BS in FR1 can provide enough protection for the BS receiver against the UE transmission in adjacent channel. Therefore, it is proposed to adopt the same in-band blocking requirements for AAS BS in FR1 for 14800 to 15350 MHz frequency range. Note that this approach is also adopted for 7125 to 8400 MHz frequency range [11].

# 3.	Conclusion

This contribution further discusses the BS in-band blocking requirements s based on the agreed WF and the simulation results provided in this contribution.

Proposal 1: To adopt the same in-band blocking requirements for AAS BS in FR1 for 14800 to 15350 MHz frequency range.

| 5.3 | Blocking response | In-band blocking level: See [1], § 10.5.2.2 Out-of-band blocking level: See Table 3A for radiated requirements, (ΔfOOB = 100 MHz) (Note 5) | 200 MHz BW requirements the same as [2], § 7.6A Tables 7.6A.2.1-1 and 7.6A.3-1 Bandwidth Class C for blocking levels and § 7.7 Table 7.7A-1 Bandwidth Class C for spurious response |
| --- | --- | --- | --- |

# References

[1]	RP-242944, “Revised SID: Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	RP-242943, “Status report for SI Study on IMT parameters for NR for 4400 to 4800, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[3]	R4-2414449, “Topic summary for [113][119] FS_NR_IMT”, Moderator (Ericsson).

[4]	R4-2420330, “WF on 15GHz BS parameters”, ZTE.

[5]	R4-2417536, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 1 UE per slot”, Nokia.

[6]	R4-2417537, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 3 UE per slot”, Nokia.

[7]	3GPP TR 38.922 v1.0.1: “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.

[8]	R4-2418991, “On 15 GHz BS-UE radio and antenna parameters”, Ericsson.

[9]	3GPP TS 38.104 v18.8.0: “NR; Base Station (BS) radio transmission and reception”.

[10]	R4-2420385, “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[11]	R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.