3GPP TSG-RAN WG4 Meeting # 114	R4-2500075

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.26.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#105 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#113 and the WF was agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

2.1	Concept of LP-WUS power boosting

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: EPRE ratio can be used for the LP-WUS power boosting concept. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs) when both LP-WUS and NR are transmitted at the same time in the NR carrier. (CATT) - The case when there is no NR REs allocation should be covered. (Nokia) - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (ZTE, Ericsson, Nokia) - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Proposal 3: An NB-IoT-like approach to defining power boosting could serve as a baseline, offering flexibility in declaration. Additionally, the option for a single-value declaration should also be permitted, accommodating different scenarios. (Huawei) - The LP-WUS RB power dynamic range is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). If a single power booting value is to be declared irrespective of channel bandwidth, the difference in per-RE power ratios between the average power of LP-WUS REs (within the NR transmission bandwidth configuration) and the average power NR REs (excluding LP-WUS REs) could alternatively be used. - Proposal 4: RAN4 should use LP-WUS power boosting concept rather than “dynamic range” as definition for this requirement. The power boosting concept and RE dynamic range concept for LP-WUS be separated. (vivo) - Adhoc WF - “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition. - FFS whether other cases can be additionally considered. E.g., single value declaration.

As the concept of LP-WUS power boosting must be finalized first before other topics related to power boosting can progress, and this topic has been discussed since TSG RAN4#110-bis with all pros and cons considered. It is proposed to finalize the definition as in the Adhoc WF to avoid further delay in the progress on other related topics.

“The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)”

Note that single value declaration may still be possible with this definition, but this mean the declared value will be limited by the narrowest bandwidth supported by the BS.

2.2	How to comprehend the de-boost on the NR transmission power

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: LP-WUS power boosting does not change the total maximum transmit power of BS. (CATT) - There is no power boosting for the case where all RBs are used by LP-WUS and 0 RBs for NR, i.e., SCS 30kHz and CBW 5MHz. - For the cases where there would be RB(s) allocated to NR, x dB NR power degradation is allowed. - Proposal 2: The average power over all REs should be fixed and when the power boost of WUS is enabled with or w/o the NR signal at the same time: (Ericsson) - The WUS signal should be transmitted always with x dB above the average power. - There is no NR signal de-boosting in normal transmission. - The NR de-boosting transmission power is for the testing purpose only and whether to de-boost NR signal transmission power should be up to network implementation. - WF - Consider the de-boosting behaviour together with other relevant issues - Companies are encouraged to think about the draft spec language to reflect corresponding proposals

It is stated in the agreed WF at TSG RAN4#110-bis [3] as follows:

- With following assumption to guide future discussion, RAN4 will revisit the above concept after BS core requirements are settled. - Transmission power is shared between NR OFDM signal and NR LP-WUS for the same carrier. - The rated carrier output power and rated total output power are not changed with LP-WUS power boosting.

As the rated carrier output power and rated total output power are not changed with LP-WUS power boosting, the BS must attain the required power for LP-WUS power boosting from by reducing the transmission power for NR signal, either by de-boosting the NR signal (i.e., PSD reduction) or reducing the transmission bandwidth of the NR signal (i.e., RB blanking). However, this should be treated as implementation issue of the BS scheduler and thus there is no need to reflect this in the specifications.

2.3	Core requirement for LP-WUS power boosting

It is stated in the agreed WF as follows:

- WF - FFS the following options - Option 1: Leave the power boosting level to manufacturer declaration - Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW - Other options are not precluded

It is stated in the SI TR 38.869 [4] as follows:

#### 7.1.2.5 gNodeB RF impacts The power boosting of 3 dB or 6 dB of a WUS signal relative to the average power of a configured NR carrier can improve the WUS signal coverage. It is beneficial to enable the 3dB or 6 dB power boosting at the existing network. Therefore, RAN4 assumes to reuse the legacy RF specification as a starting point. Some company observed that 3 dB or 6 dB power boosting for a 24 RB WUS signal may not be feasible for some of deployed gNB when average power is kept the same before and after power boosting. It is feasible to configure different PSD carriers in a multiple carrier configuration where WUS signal can be placed in a higher PSD carrier among all configured carriers. The amount of boosting is up to manufacturer declaration following the legacy manufacturer declaration. BS manufacturer could declare if power boosting for WUS signal is supported and the boosting level in the range of [0] to [6] dB is considered. Final power boosting level and the condition of power boosting will be decided based on further analysis in WI phase

It is already considered during the SI that BS manufacturer could declare if power boosting for WUS signal is supported and the boosting level of 0 dB (no power boosting) could be declared.

Moreover, the RE power control dynamic range requirement for NR BS is specified in TS 38.104 [5] as follows:

### 6.3.2 RE power control dynamic range #### 6.3.2.1 General The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. #### 6.3.2.2 Minimum requirement for BS type 1-C and BS type 1-H RE power control dynamic range: Table 6.3.2.2-1: RE power control dynamic range Modulation scheme used; RE power control dynamic range (dB). on the RE; (down); (up). QPSK (PDCCH); -6; +4. QPSK (PDSCH); -6; +3. 16QAM (PDSCH); -3; +3. 64QAM (PDSCH); 0; 0. 256QAM (PDSCH); 0; 0. 1024QAM (PDSCH); 0; 0. NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station..

In table 6.3.2.2-1 above, the RE power control dynamic range requirement is specified depending on the modulation schemes and physical channels. The RE power up and down range for each modulation scheme was derived by considering the relation between RE power boosting/de-boosting and other RF requirements like UEM, ACLR and EVM, as explained in TR 38.817-02 [6].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the dynamic range for LP-WUS/LP-SS. In table 6.3.2.2-1 above, the RE power up and down range for modulation order higher than 16QAM is 0 dB considering the impact on PA with the higher PAPR of higher order modulation. It is shown in Figure 1 below that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM. Therefore, the core requirement for the dynamic range for LP-WUS/LP-SS should be, if specified, the same as 64QAM, i.e., 0 dB.

(a) LP-WUS (bit sequence 1101) using ZC sequence with M=1

(b) LP-WUS (bit sequence 1101) using ZC sequence with M=2

(c) LP-WUS (bit sequence 1101) using ZC sequence with M=4

(d) LP-WUS (bit sequence 1101) using M sequence with M=1

(e) LP-WUS (bit sequence 1101) using M sequence with M=2

(f) LP-WUS (bit sequence 1101) using M sequence with M=4

(g) Typical QAM signals

Figure 1: PAPR comparison of LP-WUS and typical QAM signals

On the other hand, it was agreed at RAN1#117 to support 11 PRBs for LP-WUS and LP-SS with SCS 30kHz [7]. The power degradation of RBs other than LP-WUS signal for this setting with different LP-WUS power boosting levels are provided in Table 2 below, which shows that the power reduction for other RBs because of LP-WUS power boosting varies between 0.1 dB to 1.1 dB depending on the power boosting level and channel bandwidth.

Table 2: Impact of LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 30 | 162 | 11 | 46 | 60 | -0.32687277 |
| 3 | 30 | 189 | 11 | 46 | 70 | -0.27568025 |
| 3 | 30 | 217 | 11 | 46 | 80 | -0.237165576 |
| 3 | 30 | 245 | 11 | 46 | 90 | -0.208095127 |
| 3 | 30 | 273 | 11 | 46 | 100 | -0.185374077 |
| 4 | 30 | 162 | 11 | 46 | 60 | -0.506770566 |
| 4 | 30 | 189 | 11 | 46 | 70 | -0.425992238 |
| 4 | 30 | 217 | 11 | 46 | 80 | -0.365577629 |
| 4 | 30 | 245 | 11 | 46 | 90 | -0.32017752 |
| 4 | 30 | 273 | 11 | 46 | 100 | -0.284811572 |
| 6 | 30 | 162 | 11 | 46 | 60 | -1.063293018 |
| 6 | 30 | 189 | 11 | 46 | 70 | -0.884288268 |
| 6 | 30 | 217 | 11 | 46 | 80 | -0.752987437 |
| 6 | 30 | 245 | 11 | 46 | 90 | -0.655701573 |
| 6 | 30 | 273 | 11 | 46 | 100 | -0.580711194 |

Moreover, support of higher levels of LP-WUS power boosting will increase BS implementation complexity. Therefore, it is proposed that, like NB-IoT power boosting in TS 36.141 [8] and TS 38.141-1 [9], BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, including 0 dB (no power boosting), in the manufacturer declarations for BS supporting LP-WUS operation to differentiate their product performance.

2.4	General consideration on other requirements

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: Define a set of the BS Tx requirements for LP-WUS. (CMCC) - Proposal 2: RAN4 to determine whether a full set, simplified set of BS Tx requirements or just dynamic range requirement should be specified for LP-WUS. (Huawei) - Proposal 3: First focus on the power boosting and transmit signal quality requirements that clearly need to be defined, then work on other Tx requirements when their need become clear. (Nokia) - Proposal 4: No new RF requirements is identified for OOK-1. No other new RF requirement than the dynamic range or potential EVM requirement for OOK-4 is identified for LP-WUS. (Ericsson) - Adhoc WF - Discuss on whether transmit signal quality should be defined - FFS whether equivalent EVM requirement could be considered, - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

Considering the time constraint, it is proposed to first focus on the power boosting and transmit signal quality requirements that clearly need to be defined, then work on other Tx requirements when their needs become clear.

Currently, the EVM requirement for NR BS is specified in TS 38.104 [5] as follows:

### 6.5.2 Modulation quality #### 6.5.2.1 General Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. Details about how the EVM is determined are specified in Annex B. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. #### 6.5.2.2 Minimum Requirement for BS type 1-C and BS type 1-H For BS type 1-C and 1-H, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met using the frame structure described in clause 6.5.2.3. Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for PDSCH; Required EVM. QPSK; 17.5 %. 16QAM; 12.5 %. 64QAM; 8 %. 256QAM; 3.5 %. 1024QAM; 2.5 %1 2.8 %2. Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz..

In table 6.5.2.2-1 above, the EVM requirement is specified depending on the modulation schemes and physical channels. The EVM for each modulation scheme was derived by considering the EVM impacts on overall link performance, as explained in TR 38.817-02 [6].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement would need to be added for the modulation quality for LP-WUS/LP-SS. The EVM requirement for LP-WUS/LP-SS should be derived by considering the EVM impacts on overall link performance according to RAN1 waveform design.

On the other hand, it was agreed at RAN1#117 as follows [7]:

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

On the other hand, it was agreed at RAN1#118-bis as follows [10]:

To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us].

Based on RAN1 assumptions for LP-WUS operation (in low SNR) and design, the EVM requirement do not seem relevant for LP-WUS. Therefore, it is proposed to consider the EVM requirement for LP-WUS/LP-SS only when its need become clear.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: Finalize the definition of LP-WUS power boosting as in the Adhoc WF to avoid further delay in the progress on other related topics.

“The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)”

Proposal 2: How to attain the required power for LP-WUS power boosting should be treated as implementation issue of the BS scheduler and thus there is no need to reflect this in the specifications.

Proposal 3: BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, including 0 dB (no power boosting), in the manufacturer declarations for BS supporting LP-WUS operation.

Proposal 4: First focus on the power boosting and transmit signal quality requirements that clearly need to be defined, then work on other Tx requirements when their need become clear.

Proposal 5: Consider the EVM requirement for LP-WUS/LP-SS only when its need become clear.

# References

[1]	RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2419802, “WF for [113][308] NR_LPWUS”, Huawei.

[3]	R4-2406140, “Way Forward for [110bis][314] NR_LPWUS”, Huawei.

[4]	3GPP TR 38.869 v18.0.0: “Study on low-power wake-up signal and receiver for NR”.

[5]	3GPP TS 38.104 v18.8.0: “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.817-02 v15.11.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[7]	RP-241039, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo.

[8]	3GPP TS 36.141 v18.5.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[9]	3GPP TS 38.141-1 v18.8.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[10]	RP-242619, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo.