3GPP TSG-RAN WG4 Meeting #114							         			R4-2500108

Athens, GR, 17th – 21st February 2025

Title: 	Candidate TN PA for IoT-NTN HPUE in smartphones

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.10.2.3.1 Tx requirements	 [NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#113, we discussed in [1] the options for NTN HPUE in smartphones and how to enable the optimum reuse of the smartphone TN hardware. In the meeting, some options were captured in a way forward [2] focusing on IoT-NTN and also in [3] in terms of potential ACLR relaxation. In this contribution, we further discuss which target TN PA should be used to develop the IoT-NTN HPUE requirement for smartphones and explore options for increased power, a similar contribution [4] focuses on NR-NTN HPUE.

# Discussion

## 2.1 WF inputs

The way forward [3] provides several inputs on relaxed ACLR and power class targets for IoT-NTN.

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study

Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective:

For IoT-NTN PC2:

- Less than 21.1 dB with 1.5km isolation distance assumed;

- Less than [20.3] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For IoT-NTN PC1:

- Less than 21.9 dB with 1.5km isolation distance assumed;

- Less than [24.2] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

More focussed on IoT-NTN the way forward [2] provides further details.

Issue 1-1-1: For NB-Iot Based Iot-NTN, whether and how to define GSM_ACLR: Remove GSM_ACLR for NB-IoT based NTN HPUE.

Issue 1-1-2: PC1/2 ACLR should not below PC3 requirements, based on the assumption that PC3 requirement can be relaxed if necessary: HPUE ACLR should not be relaxed than that for PC3 for NB-IoT NTN.

Issue 1-1-3: For NB-IoT based NTN, considering to extend the SEM for PC2 and PC1 as following: For NB-Iot based NTN, consider PC3 SEM as a starting point. More evaluation can be considered.

Issue 1-1-4: SEM relaxation for the case UE CBW is much narrower compared to BS CBW

- Proposals: When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements.

- WF : FFS whether MPR reduction technique currently discussed in Rel-19 can be considered here

Issue 1-1-5: NB-IoT based IoT-NTN, MPR=0 reference point: As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

Issue 1-1-6: NB-IoT based IoT NTN, P-MPR has not been defined: Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details are FFS. Other solutions for SAR are not precluded.

Issue 1-1-7: General guideline for eMTC based Iot-NTN: Proposals; As a general guideline, for eMTC based IoT-NTN, the HPUE requirements can using legacy LTE requirements as starting point [unless otherwise stated.]

Issue 1-1-8: The nominal MOP tolerance]

- Proposals: As for IoT-NTN eMTC and NB1/NB2, regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows.

- Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment.

- Option 2-1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 255 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 254 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 253 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| 252 | 26 | +22 | 23 | +22, -2 | 20 | +22, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. |  |  |  |  |  |  |

- Option 2-2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 255 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 254 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 253 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| 252 | 262 | +2 | 232 | +2, -2 | 202 | +2, -2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |  |  |

- Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD).

- Option 4: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE.

Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN:

- No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19;

- Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN

- FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

## 2.2 Target TN PA for smartphone and potential power increase and limitations

For IoT-NTN HPUE, the consensus is that only 1Tx is targeted in the scope of a smartphone, and reuse of the existing hardware is a must. For NB-based IoT-NTN, however, two candidates could be considered:

- A 1Tx PC2 FDD PA as suggested for NR-NTN in [4] and the main option for MTC-based IoT-NTN

- The mid-band GSM PA still supported in many phones.

Compare the two PAs in term of power capability:

- The 1Tx PC2 FDD mid-band PA is designed to deliver 26dBm average at the antenna, with waveforms that have >7dB PAPR and significant post-PA losses due to duplexers, switches and diplexers.

- The mid-band GSM PA sees less post-PA losses (simple LC LPF instead of a duplexer) and uses waveforms that are constant envelope.

- The result is that the NR 1Tx PC2 FDD PA saturated power is at least 1dB higher than the GSM PA, and if used for HD-FDD it can benefit from the same lower post-PA losses.

- In term of DC current, the GSM PA requires about 30% more current when transmitting GMSK at max power than the PC2 1Tx NR FDD PA transmitting DFT-s-OFDM QPSK at PC2 power.

Another comparison is the band coverage of the two types of PAs as described in Figure 1. The GSM mid-band PA only covers band 2 and band 3 while the 1Tx PC2 NR PA covers all mid bands from 1.6GHz to 2GHz. Table 1 show the NTN UL bands, the 2G and NTN bands and the associated worst-case distance.

Figure 1: band landscape around NTN bands.

Table 1: NTN, NR, LTE and 2G bands

| NTN bands |  | FLow | FHigh |  |  |
| --- | --- | --- | --- | --- | --- |
| NTN UL | 252 | 2000.0 | 2020.0 |  |  |
| NTN UL | 253 | 1668.0 | 1675.0 |  |  |
| NTN UL | 254 | 1610.0 | 1626.5 |  |  |
| NTN UL | 255 | 1626.5 | 1660.5 |  |  |
| NTN UL | 256 | 1980.0 | 2010.0 |  |  |
| 2G, LTE and NR bands |  | FLow | FHigh | delta NTN | overlap |
| 2G/NR UL | 2/n2 | 1850.0 | 1910.0 | 110.0 | 0.0 |
| 2G/NR UL | 3/n3 | 1710.0 | 1785.0 | 100.0 | 0.0 |
| NR UL | n24 | 1626.5 | 1660.5 | 16.5 | 34.0 |
| NR UL | n25 | 1850.0 | 1915.0 | 105.0 | 0.0 |
| NR UL | n1 | 1920.0 | 1980.0 | 40.0 | 0.0 |
| LTE UL | 23 | 2000.0 | 2020.0 | 20.0 | 20.0 |
| NR U/DL | n34 | 2010.0 | 2025.0 | 30.0 | 10.0 |
| NR UL | n65 | 1920.0 | 2010.0 | 10 | 30.0 |
| LTE U/DL | 54 | 1670.0 | 1675.0 | 60.0 | 5.0 |
| NR UL | n70 | 1695.0 | 1710.0 | 85.0 | 0.0 |
| NR DL | n70 | 1995.0 | 2020.0 | 15.0 | 25 |
| NR DL | n25 | 1930.0 | 1995.0 | 25.0 | 15 |

As evident in Figure 1, and detailed in Table 1, the NR PA already covers a significant frequency range as it covers all TDD and FDD bands. It needs significantly less re-tuning to cover the NTN bands than the 2G PA.

Beyond this, the 2G PA is not designed to handle wider CBW or the high PAPR needed for MTC based IoT-NTN. Furthermore, 2G networks are being tuned off with time in many regions and the remaining 2G support may converge with 1Tx PC2 FDD PA anyhow.

Observations: Compared to a mid-band 2G PA, the 1Tx PC2 FDD NR PA offers higher saturated power, wider CBW support, support for linear modulations, and operates closer to the NTN bands.

Proposal:

- For NB-IoT-based IoT-NTN HPUE support in smartphones, a single requirement is developed that covers implementations with either a mid-band 2G PA or a 1Tx PC2 FDD NR mid-band PA assuming lower post-PA losses.

- For MTC-based IoT-NTN HPUE support in smartphones, a single requirement is developed based on implementation with a 1Tx PC2 FDD NR mid-band PA.

## 2.3 Handling of power exceeding power class 2 for TS 36.764

The applicable emission requirements seem confusing for IoT-NTN HPUE for MTC and NB-IoT when comparing the results of the coexistence analysis in [3] and some of the agreements in [2]. Nevertheless, the PC2 power defined in LTE will be exceeded:

- to benefit from lower post PA losses in HD-FDD mode

- to benefit from the low PAPR of single subcarrier operation for NB-IoT especially if in the inner third sub-carriers.

- reduced PAPR 1RB allocation for NB-IoT.

Issue 1-1-8 in [2] collects several ways to account for the HD-FDD-related power increase from using notes to creating new power classes. It does not cover further power increases for lower PAPR NB-IoT waveforms or relaxed OOB requirements.

In our view, it is better to fully understand the scope of increased power for a PC2 NR PA in the different IoT-NTN modes to decide if it is power boosting over a PC2 power class (but then what to do if boosting if >3dB in some cases?). Given that 36.764 only has two power classes defined, RAN4 should not hesitate to create new power classes if needed. In any case, if boosting is possible it should apply to both the lower and upper bound of the power class and thus is better applied on the reference power rather than the tolerances.

Proposal on handling higher power in TS 36.764:

- Before adopting a solution to enable higher power than 26dBm for HD-FDD operation, the full scope for higher power should be understood, including lower PAPR waveform, boost for NB-IoT and power boosting for some allocations for MTC.

- Creating a new 1Tx power class shall not be precluded.

- If power boosting on top of PC2 26dBm is specified, the power increase applies to both the lower and the upper bound of the power class, ideally via a shift of the reference.

# Conclusion

In this contribution, we discuss the TN PAs of interest for IoT-NTN HPUE. This allowed us the following proposals.

Proposal:

- For NB-IoT-based IoT-NTN HPUE support in smartphones, a single requirement is developed that covers implementations with either a mid-band 2G PA or a 1Tx PC2 FDD NR mid-band PA assuming lower post-PA losses.

- For MTC-based IoT-NTN HPUE support in smartphones, a single requirement is developed based on implementation with a 1Tx PC2 FDD NR mid-band PA.

Proposal on handling higher power in TS 36.764:

- Before adopting a solution to enable higher power than 26dBm for HD-FDD operation, the full scope for higher power should be understood, including lower PAPR waveform, boost for NB-IoT and power boosting for some allocations for MTC.

- Creating a new 1Tx power class shall not be precluded.

- If power boosting on top of PC2 26dBm is specified, the power increase applies to both the lower and the upper bound of the power class, ideally via a shift of the reference.

# References

[1] R4-2419521 PA architectures for handheld NTN HPUE, Skyworks Solutions Inc., RAN4#113

[2] R4-2420404 WF on IoT NTN HPUE, vivo, RAN4#113

[3] R4-2420403 WF on NR NTN HPUE, Samsung, RAN4#113

[4] R4-2500090 Candidate TN PA reuse for NR-NTN HPUE in smartphones, Skyworks Solutions Inc., RAN4#114