3GPP TSG-RAN WG4 Meeting #114	R4-2500109

Athens, GR, Feb. 2025

Source	:	Qualcomm Incorporated

Title	:	On UE RF requirements for the LPWUR

Agenda item	:	7.23.2.2

Document for	:	Approval

# 1.	Introduction

In this contribution we first address FR2-specific aspects like radio architecture and spherical coverage. We also share our views on open items pertaining to the core requirements in general.

# 2. 	Discussion

## 2.1	FR2-specific details

### 2.1.1	Receiver topology

We have previously laid out the options for the FR2 LPWUR architecture assumption [2]:

- Legacy PDSCH-like: 2 Rx with MRC

- Pol. selection diversity (PSDiv): 2 Rx with antenna selection diversity

Note that here ‘2Rx’ here refers to the legacy FR2 UE (main) receiver assumption of comprising 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized.

For an FR2 LPWUR, the legacy main receiver serves as a natural precedent. It may however be beneficial to assume a slightly simpler receiver for LPWUR for cost and complexity reasons. The simplification here pertains to combining strategy, for example it would choose the Rx chain with the stronger signal (in principle this is antenna selection diversity). See figure below.

H antenna

V antenna

PSDiv scheme for FR2 LPWUR

Demod block

The standard should leave the selection algorithm to UE implementation, but for the sake of discussion, one example implementation is ‘pre-selection’ based on LPSS. Other approaches could depend on prior LPWUS for selection or real time energy estimation.

The worst-case situation for the PSDiv scheme is when the E-field associated with the LPWUS is 45 misaligned from either of the polarization directions of the LPWUR antennas. In this case, the signal on each branch is 3 dB lower than optimal, and after PSdiv, the SNR is degraded by 3 dB also [3] compared to an optimally aligned receiver. This type of scheme hence suffers a minor loss in sensitivity, but it does not lose any diversity in the polarization dimension, an acceptable compromise for LPWUR.  Further, this assumption allows FR2 to align with FR1 in terms of number of receivers used to determine SNR performance (1RX), along with simpler baseband processing.

The competing proposal is MRC. MRC is well understood in the frequency domain, but it is not clear what technique and assumptions are optimal for time-domain OOK receivers. Standardizing an MRC method for time domain processing will require significant study which is beyond the scope of the Rel-19 WI.

Proposal 1: MRC is not pursued for FR2 LPWUS due to lack of precedent for combining OOK signals, at least in Rel-19.

A comparison of the candidate architectures for FR2 LPWUR is below:

| LPWUR architecture candidates | #1: 2Rx with MRC (like legacy main radio PDSCH) | #2: PSDiv |
| --- | --- | --- |
| RF front end | 2 chains, orthogonal polarized antennas | (same as #1) |
| Combining strategy | MRC – NOT well understood for OOK signals | PSDiv – simple algorithm based on better SNR |
| Sensitivity to angular misalignment with polarization basis | Very low to none compared to optimally aligned receiver (TBD for OOK) | Degraded from optimally aligned receiver, degradation upper bound exists however (3 dB) |
| Simplicity, power consumption | Worse | Better |

To summarize, PSDIV is a good candidate for the nominal architecture assumption for the FR2 LPWUR.

Proposal 2: The FR2 LPWUR assumes antenna selection diversity among its 2 Rx chains (equivalent to PSDiv)

If PSDiv (prop2) can be agreed, then an automatic 3 dB degradation has to be expected compared to the MR. FR2 unlike FR1 does not lend itself to making a standalone ‘low power’ radio for LPWUS monitoring. Practical implementations will generally require significant front end sharing with the MR, and one of the benefits of that is good noise figure. To not preclude aggressive implementations that turn down LNA power for LPWUS duty, it may be reasonable to account for minor degradation beyond the 3 dB.

Proposal 3: For an FR2 LPWUR that uses PSDiv, NF is MR + 4 dB for both, OOK based and OFDM based topologies.

### 2.1.2	Spherical coverage of LPWUR sensitivity

It is already agreed that single element beams would be used for the LPWUR [2]. The figure below shows the normalized spherical coverage CCDFs of sensitivity of a UE using realistic modules and MRC based on size of beam forming antenna (i.e. number of elements).

The curves are normalized to the REFSENS value, so the focus can be on relative performance. A first observation is that the best sensitivity degrades by 5.7 dB when going from 4 elements to single element beam. Uniform linear array theory suggests this value should be 6.0 dB, but several aspects perturb the outcome from this ideal, for example non-identical elements.

Another observation is that in this example, the 50th%ile direction features 7.1 dB of sensitivity degradation in the 4x1 configuration, but only 6.3 dB for the single element case. This difference is expected because only discrete beam choices (limited ‘codebook’) are allowed for the 4x1 case, and the impact of non-identical element fields.

Overall, however, this result is largely as expected for the MRC case. For the LPWUR which uses pol. selection diversity (rather than MRC), there is a systematic loss of up to 3 dB, and a conservative (for the UE) assumption is that this applies uniformly in all directions. With this conservative and simplifying assumption it is readily seen that relative curves remain unchanged (due to normalization to a value that is affected by the same 3 dB).

Observation 1: The sensitivity degradation CCDFs are comparable between the 4x1 and single element module configurations.

Proposal 4: The spherical coverage requirement for LPWUS sensitivity reuses both, the percentile rank as well as dB of degradation as the connected mode sensitivity requirement.

For example, for PC3 in n258, 50 MHz channel:

|  | Connected mode requirement | LPWUR sensitivity |
| --- | --- | --- |
| REFSENS | -88.3 dBm | X dBm |
| Sensitivity at 50th% ile direction | -77.4 dBm = -88.3 + 10.9 | X + 10.9 |

## 2.2	SNR proposal

In previously shared some SNR results [3] based on OFDM-based receivers, we noted that FR1 and FR2 baseband sections perform similarly. We also reported strong dependence of SNR vs MDR curves on known sequence length and chip rate ‘M’ for realistic OFDM receivers as well as OOK receivers

Applicable to both LPWUR types then, SNR results are contingent on RAN1 specification of LPWUS design, including OOK type, chip rate and known sequence length pairings, as well as identification of sequences for receivers that are susceptible to cross-identification. It is therefore premature to conclude on SNRs for sensitivity requirements for the LPWUR.

Observation 2: It is premature for RAN4 to conclude on SNRs for sensitivity requirements for LPWUR, regardless of receiver architecture, due to lack of sufficient progress on LPWUS specification in RAN1.

## 2.4	Ultra-low power mode

For low rate DL signals like LPWUS, phase noise requirements are often driven by projected interferer size, rather than signal quality. While it is possible to have ‘ultra-low power’ LPWUR implementations, these do not make sense for eMBB due to other sources of consumption not related to the radio; for example, a typical smart phone has screen and user-apps that dominate consumption. Additionally, since system design cannot guarantee a no-interferer situation, an implementation with poor hardening against expected interferers cannot be standardized. Consequently, it should be left up to the UE to detect interferers and opportunistically engage ‘ultra-low power mode’.

Proposal 5: Ultra-low power mode is left to UE implementation, i.e. to be used opportunistically by the UE when no interferers are detected. Ultra-low power mode considerations are not suitable for standardization.

## 2.5	Max. input power

Legacy practice for this parameter is to define the requirement for all possible channel bandwidths. The challenge for the LPWUR is that input power can be differently distributed between LPWUS and legacy DL RBs, depending on channel bandwidth. Moreover, since competitive implementations are expected to use AGC, the additional challenge is to set the AGC correctly, i.e. not saturate components while leaving enough LPWUS power for detection. Finally, the core requirement should ensure all channel BWs have a specified max. power guarantee.

Proposal 6: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs.

It can be left up to RAN5 how to streamline test cases.

# 3.	Conclusions

Observation 1: The sensitivity degradation CCDFs are comparable between the 4x1 and single element module configurations.

Observation 2: It is premature for RAN4 to conclude on SNRs for sensitivity requirements for LPWUR, regardless of receiver architecture, due to lack of sufficient progress on LPWUS specification in RAN1.

Proposal 1: MRC is not pursued for FR2 LPWUS due to lack of precedent for combining OOK signals, at least in Rel-19.

Proposal 2: The FR2 LPWUR assumes antenna selection diversity among its 2 Rx chains (equivalent to PSDiv)

Proposal 3: For an FR2 LPWUR that uses PSDiv, NF is MR + 4 dB for both, OOK based and OFDM based topologies.

Proposal 4: The spherical coverage requirement for LPWUS sensitivity reuses both, the percentile rank as well as dB of degradation as the connected mode sensitivity requirement.

Proposal 5: Ultra-low power mode is left to UE implementation, i.e. to be used opportunistically by the UE when no interferers are detected. Ultra-low power mode considerations are not suitable for standardization.

Proposal 6: The max. input power level is specified for all channel bandwidths, in the presence of legacy RBs with the same PSD as the LPWUS RBs

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2420394, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#113

[3] 	R4-2417630, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#113