3GPP TSG-RAN WG4 Meeting #114	R4-250xxxx

Athens, GR, Feb. 2025

Source	:	Qualcomm Incorporated

Title	:	FR2 enhancement overview for Rel-19

Agenda item	:	7.1.1.2.3

Document for	:	Approval

# 1.	Introduction

In RAN#103 a new work item [1] was created to investigate if lower MPRs can be made applicable to FR2 UEs configured for UL CA without the need for a new simulation effort to simulate back off requirements. RAN4 has already agreed to an enhancement where the CA MPR available to the UE is derived from the UL CA configuration, rather than the UL+DL CA configuration [2]. Separately, it was also agreed [4] to specify MPR enhancement based on activation-status of the CCs. In this contribution we share our view on whether these enhancements interact, and also adress the question of applicable emissions requirements when MPR is derived based on the activated subset of all configured CCs.

# 2. 	Discussion

## 2.1	Overview of Rel-19 FR2 enhancements

For Rel-19, the FR2 enhancement discussion covers two types of enhancements. These enhancements are based on:

- CA configuration

- Activation status of the CCs in the CA configuration

In the sections below, each are explained further, with an aim to identifying any interdependence.

## 2.2	‘Configuration based’

The first enhancement is where the applicable CA MPR is derived from the UL configuration rather than UL + DL configuration. The principle here is that CA MPR generally decreases with narrower bandwidth, and the UL CA aggregated BW is usually narrower than the DL CA agg. BW. This is already agreed in draft CR form [2]. It is loosely referred to as ‘configuration based’ because the enhancement is derived from configured quantities alone.

Observation 1: The ‘configuration based’ Rel-19 FR2 CA MPR enhancement is based on the UE’s ability to optimize its UL LO location independently of the DL CA configuration.

## 2.3	‘Activation based’

The legacy CA MPR assumption was that the for an intra-band CA configuration, the UE parks a common LO for Rx and Tx based on the UL + DL CA configuration. Often, UEs get configured for many contiguous CCs in a band, even if traffic may not dictate simultaneous traffic on all of them. It is therefore feasible for the network to deactivate some of those configured CCs when traffic conditions allow in exchange for other benefits (reduced CA MPR for Rel-19).

For the Rel-19 ‘activation based’ enhancement, the assumption is that UE can park its LO based on the activated subset of configured CCs, rather than all the configured CCs. Here the principle is that the CA MPR generally decreases with narrower bandwidth, and the aggregated BW over just the activated subset of configured CCs can be narrower than the agg. BW of all configured CCs. The challenge for the UE for this enhancement is to execute its internal hardware reconfiguration inside the MAC-CE timeline, which is the vehicle of activation and deactivation commands

Observation 2: The ‘activation based’ Rel-19 FR2 CA MPR enhancement is based on the UE’s ability to execute LO location changes inside the MAC-CE timeline.

This enhancement has already been agreed in principle [4], but details about which emissions apply and interaction with ‘configuration based’ remain open

## 2.4	Interaction between the two enhancement schemes

From an implementation perspective, the two enhancements above pose different challenges for a UE. The ‘configuration based’ enhancement depends on independently located R/T LOs and the ‘activation based’ enhancement depends on executing LO location changes in a compressed timeline.

| Rel-19 FR2 MPR enhancement space |  | Basis for ‘Configuration based enhancement’ |  |
| --- | --- | --- | --- |
|  |  | Independent R/T LO | Common R/T LO |
| Basis for ‘Activation based enhancement’ | Hardware re-config inside MAC-CE timeline | (both enhancements supported - possible?) | Only ‘activation based’ enhancement supported |
|  | Longer duration hardware re-config | Only ‘configuration based’ enhancement supported | Legacy assumption |

The enabling skills are clearly unrelated, with support of one not impacting support of the other.

Observation 3: The Rel-19 FR2 ‘configuration based’ and ‘activation based’ enhancements depend on unrelated UE skills, consequently, a UE’s ability to support each is also independent of the other

A logical next step is to game out if it is feasible for a UE to support both types of enhancements. This is a trivial exercise, i.e. A UE whose execution timelines are sufficiently short, and one that can optimize LO location for UL and DL can support both enhancements. This type of UE needs no new physics and is feasible today with the appropriate design goals.

Observation 4: It is feasible for a UE to support both ‘configuration based’ and ‘activation based’ enhancements.

Proposal 1:  For Rel-19 FR2 UE MPR enhancement, the capabilities for ‘configuration-based’ enhancements and ‘activation-based’ enhancement are separate and independent.

## 2.5	Emissions requirement for ‘activation based’

The main goal of the enhancement is to reduce the basis BW for CA MPR determination relative to the UL + DL CABW, in effect reducing the allowed MPR. So, there are two options to specifying the emissions requirements for the Rel-19 ‘activation based’ enhancement:

- Change ‘configured CCs’ to ‘activated CCs’ in all emissions verbiage.

- Retain the legacy configuration-based emissions requirement.

These are compared in more detail in the subsections below.

### 2.5.1	Change ‘configured CCs’ to ‘activated CCs’ on existing dependencies

#### 2.5.1.1	SEM/ACLR/SE

In the legacy case, SEM, ACLR, SE depend on the UL aggregated BW. If the verbiage is changed to ‘activated CCs’, the BW-basis gets narrower and the spec. would tighten, which can be viewed as a significant UE burden with uncertain network benefit. It is also a RAN4 burden, because new behavior pertaining to UE awareness of de-activated CCs, and how the emissions can be partially relaxed there have to be studied in detail. Due to the general nature of the CC combinations possible, it is not clear what benefit can be derived compared to the alternative (not changing emissions requirements). It however does seem to not pose a problem from the perspective of ensuring regulatory compliance.

#### 2.5.1.2	IBE

In the legacy case, IBE applies over all configured CCs and UL is limited to one CC. If the verbiage is changed, IBE in the deactivated CCs would be replaced by SEM, ACLR and SE.  Typically, SEM, ACLR and SE represent stricter emissions requirements, but corner cases exist where this is not clear. Consider for example a narrow UL allocation in one of the CCs – the legacy CA IBE requirement in RBs sufficiently far away from the allocation, including those in a neighboring CC is 25 dBc. The ACLR requirement for the adjacent CC is therefore 25 dBc which is tighter than the OBW requirement of approximately 24 dBc. While minor, the bottom line is that a move to ‘activated CCs’ as basis cannot guarantee legacy emissions levels. For consistency RAN4 would have to ensure that IBE is separately checked based on CA configuration even if a general change to ‘activated CCs’ is agreed.

### 2.5.2 	Retain CA configuration as basis for emissions

This avenue represents no change to the emissions requirements, and ‘No change’ is a low-risk strategy for both system operation as well as regulatory compliance. Spec. implementation is a non-event.

### 2.5.3	Emissions verbiage proposal

A summary of the considerations in sections 2.5.1 and 2.5.2 is below:

- There is a spec tightening (UE burden) when the bandwidth basis for emissions requirements is changed to ‘activated CCs’ from ‘configured CCs’ with unclear network advantage. Furthermore, IBE testing must be retained based on ‘configured CCs’ to ensure consistent spec. requirement coverage.

- Spec. implementation is more straightforward with retention of ‘configured CCs’ as the basis for emissions requirements

Changing the emissions verbiage to depend on ‘activated CCs’ is therefore not a good option.

Proposal 2: For the agreed Rel-19 ‘activation based’ enhancements, the emissions requirements are unchanged from legacy CA, whether the feature is supported or not.

# 3. 	Conclusion

Observation 1: The ‘configuration based’ Rel-19 FR2 CA MPR enhancement is based on the UE’s ability to optimize its UL LO location independently of the DL CA configuration.

Observation 2: The ‘activation based’ Rel-19 FR2 CA MPR enhancement is based on the UE’s ability to execute LO location changes inside the MAC-CE timeline.

Observation 3: The Rel-19 FR2 ‘configuration based’ and ‘activation based’ enhancements depend on unrelated UE skills, consequently, a UE’s ability to support each is also independent of the other.

Observation 4: It is feasible for a UE to support both ‘configuration based’ and ‘activation based’ enhancements.

Proposal 1:  For Rel-19 FR2 UE MPR enhancement, the capabilities for ‘configuration-based’ enhancements and ‘activation-based’ enhancement are separate and independent.

Proposal 2: For the agreed Rel-19 ‘activation based’ enhancements, the emissions requirements are unchanged from legacy CA, whether the feature is supported or not.

# 3.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2410749, ‘draft CR to TS 38.101-2 for FR2 CC configuration based MPR improvements’, NTTDocomo et al, RAN4#112-bis

[3] 	R4-2413226, ‘Activation-based FR2 MPR enhancement’, Qualcomm, RAN4#112, Aug. 2024

[4] 	R4-2420527, ‘WF on power domain enhancements’, Huawei, RAN4#113, Nov. 2024