# 3GPP TSG RAN WG4 Meeting#114	R4-2500115

# Athens, GR, Feb 17 – Feb 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussion on the demodulation performance requirements for 6Rx devices

Agenda item:	7.1.1.3.2

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, several key simulation assumptions related to the feasibility evaluation and performance requirements for 6Rx devices were discussed. However, no consensus was reached on these assumptions [1]. In this paper, we present our perspectives on these unresolved issues and provide link-level simulation results for MIMO layer evaluation of the 6Rx device. Additionally, we offer field data on the number of CSI-RS ports configured in various geographic regions and statistics on SNR levels observed on different reference signals.

2. Field Tests

2.1 Observations of SNRs and Transmit Antenna Ports

In this section, we share field data for an commercial Snapdragon X80 device that is equipped with 6Rx antennas, see Snapdragon X80 5G Modem-RF System | Qualcomm. The data has been collected in a commercial network in San Diego in band n41 in an outdoor urban scenario. The network was configured with 32 CSI-RS ports. The velocity was between 30 and 60 miles per hour.

In Fig. 1, the PDSCH number of layers is shown as a function as SSB SNR. Fig. 2 provides the number of layers as a function of SSB RSRP. It is mentioned again that this commercial modem is equipped with 6Rx antennas since this helps to increase the number of layers.

The plots show that 4 layers are seen with probability between 10% and 20% between SNR = 0dB and SNR = 10dB. At SNR > 20 dB, the percentage of four layers even increases to up to 50%. This reveals that a large number of layers is not only achievable at high SNR but can also occur at medium to low SNR if the environment provides rich scattering. At high SNR > 20 dB, the likelihood that four layers are observed with 4Rx antennas is very high.

In the second plot it is seen that percentage of four MIMO layers is between 10% and 20% for RSRP between -100dBm and -80dBm. This confirms the analysis in [2] where it was also shown that in this RSRP region four layers are feasible, and more than four layers can be expected in mid-cell scenarios. Even at cell edge, four layers have been observed. Above -80dBm, the likelihood of four layers increases up to 40% as seen below.

Figure 1: PDSCH Number of Layers vs SSB SNR

Figure 2: PDSCH Number of Layers vs SSB RSRP

Observation 1: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% between SNR = 0dB and SNR = 10dB. At SNR > 20 dB, the percentage of 4L even increases to up to 50%.

Observation 2: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% for RSRP between -100dBm and -80dBm. The likelihood of 4L increases up to 40% above -80dBm.

Observation 3: Given the likelihood of 4L in a commercial device with 6Rx above -80dBm RSRP is up to 40% and up to 50% for SNR > 20 dB in an urban environment, more than 4L can be expected in mid-cell and cell-center scenarios.

As a second set of field data, below we provide a table illustrating the prevalence of 32Tx configurations (or 4 CSI resources with 8 port per resource) across various geographic regions.

Table 1: 32Tx configurations observed in various geographical regions worldwide

| Operator | Infra | SA/NSA |  | Geo | Band | nrofPorts |
| --- | --- | --- | --- | --- | --- | --- |
| Operator 1 | Infra Vendor 1 | SA | USA |  | n41 | 32 |
| Operator 1 | Infra Vendor 1 | SA | USA |  | n41 | 32 |
| Operator 1 | Infra Vendor 2 | SA | USA |  | n41 | 8 per CSI resource ¦¦ 4 Resources |
| Operator 2 | Infra Vendor 1 | SA | USA |  | n77 | 32 |
| Operator 2 | Infra Vendor 1 | NSA | USA |  | n77 | 32 |
| Operator 2 | Infra Vendor 3 | NSA | USA |  | n77 | 32 |
| Operator 3 | Infra Vendor 1 | NSA | USA |  | n77 | 32 |
| Operator 4 | Infra Vendor 1 | SA | India |  | n78 | 32 |
| Operator 4 | Infra Vendor 2 | SA | India |  | n78 | 8 per CSI resource ¦¦ 4 Resources |
| Operator 5 | Infra Vendor 3 | SA | Korea |  | n78 | 32 |
| Operator 6 | Infra Vendor 3 | SA | Korea |  | n78 | 32 |
| Operator 6 | Infra Vendor 4 | SA | Korea |  | n78 | 8 per CSI resource ¦¦ 4 Resources |
| Operator 7 | Infra Vendor 4 | SA | China |  | n41 | 8 per CSI resource ¦¦ 4 Resources |
| Operator 7 | Infra Vendor 5 | SA | China |  | n41 | 8 per CSI resource ¦¦ 4 Resources |

It should be noted that the 32Tx antenna configuration is commonly observed in most TDD markets.

Observation 4: The 32Tx antenna configuration is frequently observed in TDD deployment scenarios and is supported by several operators and major infrastructure vendors worldwide.

2.2 Prediction of Expected Field Gains

To understand the achievable gains for a 6Rx/6-layer configuration in field deployment scenarios, we conduct an experiment with test devices having 8 receive antennas. As it is shown in Fig. 3, the test device has a regular form factor of a non-foldable handheld device and has been optimized considering antenna placing and decoupling for operation in 3.5GHz band. The target bandwidth of the test device is 3.3 – 3.8 GHz. Beside comparing the performance across different layers of configurations, a secondary goal is to understand the achievable correlation values between the pairs of receive antennas.

Figure 3: Test phones with up to eight receive antennas

The measurement campaign was carried under a commercial network in several regions of a cell. Typical inter-site distances in the field test were between 200 and 500 ms. CSI-RS was configured with 8 Tx ports, hence also PDSCH transmission happened with 8 Tx ports. Since 6 layers are not supported in the commercial network, CSI-RS measurements were extracted from the log files and spectral efficiency was analyzed offline. For the offline analysis of the CSI-RS based spectral efficiencies a linear MMSE-IRC receiver has been applied. Hence, the results can be seen as conservative compared to a non-linear reduced ML receiver that commercial devices may implement.

Figure 4: Projected throughput gains by six receive antennas

Figure 5: Average number of layers across SSB RSRP.

For the field tests, the spectral efficiencies based on CSI-RS measurements have been evaluated in those three categories for a test device equipped with four receive antennas and a test device equipped with six receive antennas. Clearly, the device with 4Rx antennas is limited to four MIMO layers. Fig. 4 shows projected throughput gains from field measurements which uses a sub-optimal MMSE receiver and compared with that of an advanced receiver, such as R-ML performance.

Observation 5: Six receive antennas provide very large throughput gain for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm in the order of 35% and 45%, respectively.

Observation 6: An advanced receiver, such as R-ML receiver can offer noticeable performance improvement over an MMSE receiver for 6Rx devices.

Fig. 5 shows the achievable number of layers scheduled for both 6Rx and 4Rx devices across measured SSB RSRP values. In particular, the observed rank for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm has been increased from four to five even with MMSE receiver and can achieve six-layer transmission with a R-ML receiver.

Observation 7: The following observations can be made for achievable average number of layers:

- Six receive antennas can achieve 5-layer transmission for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm with sub-optimal MMSE processing.

- Six receive antennas can achieve 6-layer transmission with a R-ML receiver when SSB RSRP > -80 dBm.

Th above observations suggest that more than four MIMO layers can be achieved in a handheld device, even without needing the form factor of a foldable device. For a foldable device, it is expected that larger gains will be observed since it may be possible to further reduce correlations between receive antenna pairs.

3. Assumption on Tx EVM

Tx EVM assumptions for 6-layer feasibility evaluation

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Use Tx EVM = 1%

-	Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

-	Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

As part of the simulation assumptions for the feasibility study, RAN4 deliberated on the assumption regarding Tx EVM. The proposed way forward for this issue is outlined below [1].

During the online discussions, several network vendors and operators highlighted that the Tx EVM in practical BS equipment can be significantly lower than the values assumed by RAN4 in performance requirements and close to 1% to 1.25%.

Observation 8: Major BS vendors and operators have confirmed that the Tx EVM in practical BS equipment can be significantly lower than the values assumed by RAN4 in performance requirements, typically ranging between 1% and 1.25%.

Based on the feedback from major network vendors and operators, we believe that a 1% Tx EVM is a reasonable assumption for the 6-layer feasibility study.

Proposal 1: Support option 1 (Use TxEVM = 1%)

4. Assumption on BS Correlation

As part of the simulation assumptions for the feasibility study, RAN4 also discussed the assumption on the BS correlation. The proposed way forward for this issue is captured below [1].

BS Antenna Correlation

Way Forward:

- RAN4 to use BS correlation matrix with alpha = 0.1

- RAN4 shall use no correlation on BS Tx Ports

- RAN4 shall not use BS correlation matrix other than 3GPP specified ones

We note that in practical deployments, Tx Ports can be configured to achieve near-zero correlation at the BS. Additionally, major BS vendors have mentioned that BS correlation can be neglected for the feasibility study. Therefore, Alpha can be assumed to be zero in the 3GPP MIMO Tx correlation matrix.

Observation 9: Major BS vendors have indicated that BS correlation can be neglected for the feasibility study.

Proposal 2: RAN4 shall use no correlation on BS Tx Ports (3GPP MIMO correlation matrix with Alpha = 0.0)

5. Assumption on Antenna Configuration

Over the past several meetings, RAN4 has deliberated on whether to include 32Tx/6Rx antenna configurations (in addition to 8Tx/6Rx) as part of the simulation assumptions. The proposed way forward for this issue is outlined below [1].

Antenna Configuration

Way Forward:

- Limit the transmit/receive antenna configuration to 8Tx/6Rx and exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations.

- Consider both 8Tx/6Rx and 32Tx/6Rx antenna configurations

As shown in Table 1, the 32Tx antenna configuration is supported by several operators and major infrastructure vendors globally. Therefore, we believe it is crucial to include the 32Tx antenna configuration in our simulation assumptions for 6Rx devices.

Proposal 3: In addition to 8Tx/6Rx antenna configuration, RAN4 should include the 32Tx/6Rx antenna configuration in the simulation assumptions.

6. Performance Comparison Across UE Correlation Matrices

In this section, we evaluate the performance using all ECC correlation matrices provided by four companies, as listed in Table 1 of Appendix B. We note that the correlation matrices in options 2-4 exhibit higher average correlation compared to option 1. Therefore, it is important to assess the extent of performance degradation due to increased correlation across receive antennas. To this end, Figs. 3 provide the performance gain relative to 6Rx/4L Rel-18 1+1 DMRS with correlation matrices from options 1 to 4 for both 5L and 6L.

Figure 6: Throughput performance relative to 6Rx/4L Rel-18 1+1 DMRS with correlation matrices from options 1 to 4 for both 5L and 6L

Figure 6 demonstrates that the performance of both 5L and 6L remains consistent across correlation options 1 to 4. However, the 6L performance, as depicted on the right side of Figure 6, shows a slight degradation with correlation option 3, which has the highest average correlation among the four options since it is for carrier frequency 2.5 GHz while the others are all for carrier frequency 3.5 GHz. Despite this, the degradation for 6L with correlation option 3 compared to option 1 is minimal, not exceeding 8%. This indicates the robustness of the performance against UE correlation for both 5L and 6L. More importantly, with the ECC matrices from four companies, both 5L and 6L configurations provide gains over 4L at practical SNR values.

Observation 10: Both 5L and 6L exhibit similar performance trends with correlation matrices from options 1 to 4 under the given simulation assumptions. With the ECC matrices from four companies, both 5L and 6L configurations provide gains over 4L

Observation 11: For 6L, a minimal performance loss of 3% at 38dB SNR is observed when using the correlation matrix from option 2 compared to option 1.

Observation 12: For 6L, minimal performance losses are observed with correlation matrices from options 3 and 4 compared to option 1, specifically 7% and 3% losses, respectively. The SNR loss is approximately 1.5dB for option 3 and around 1dB for option 4.

7. Summary of Key Simulation Assumptions and Results Across Companies

In this section, we examine the simulation results submitted by participating companies as of the last meeting (RAN4#113). We start by analyzing the results for companies that observed gains for 6L over 4L. Table 2 provides a summary of key simulation assumptions across these companies, along with their performance results for 8Tx antenna configuration.

Table 2: Summary of key simulation assumptions and corresponding results across companies that observed performance gains for 6L over 4L for MCS Table 2 with 8Tx

| Company | T-doc | Key simulation assumptions |  |  |  | Required SNR for 6L outperformance over 4L |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UE Corr Matrix | DMRS Config | TxEVM | BS Corr Matrix |  |
| Oppo | R4-2417916 | Op 1 | [Rel-15 1+1(4L); Rel-15 2+0(6L)] | 0 | Ideal | 27dB (IL off)/30dB (IL on) |
| ZTE | R4-2419165 | Ops 1-2 | [Rel-15 1+1(4L), Rel-15 2+0(6L)] | Not available | Ideal | 28dB |
| Qualcomm | R4-2418437 | Ops 1-4 | [Rel-18 1+1(6L), Rel-15 2+0(4L)] [Rel-18 1+1(6L), Rel-18 1+1(4L)] | 1 | Ideal | 32dB |
| Nokia | R4-2418011 | Op 2 | [Rel-15 1+1(4L), Rel-15 2+0(6L)] | 0 | Ideal | 24dB (CDL-C) |
| Huawei, HiSi | R4-2419029 | Ops 2-3 | Rel-15 1+1(4L), Rel-15 2+0(6L) | 1 | Ideal | 31dB |

As discussed in the previous section, where we presented results for all correlation matrices (options 1-4), the performance impact of higher average correlation is minimal, with less than 1dB degradation for 3.5 GHz carrier frequency. Additionally, as previously mentioned, major BS vendors and operators have confirmed that the Tx EVM in practical BS equipment typically ranges between 1 and 1.25%, and BS correlation can be neglected. Assuming the same DMRS overhead for both 4L and 6L, we observe that 6L starts to outperform 4L at similar SNR levels across all companies.

Observation 13: Companies that assume a typical Tx EVM in the range of 1 to 1.25%, with no BS correlation, equal DMRS overhead and measured UE ECC matrices, observe that 6L outperforms 4L at similar SNR levels.

Table 3: Summary of key simulation assumptions across companies that observed no performance gains for 6L over 4L transmission

| Company | T-doc | Key simulation assumptions |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | UE Corr Matrix | DMRS Config | TxEVM | BS Corr Matrix |
| Apple | R4-2418595 | 3GPP Model with alpha=0.005154, beta=0.005154 3GPP Model with alpha=0, beta=0.005154 | Rel-15 1+1(4L), Rel-15 2+2(6L) | 0, 3 | Ideal |
| MediaTek | R4-2417561 | Op 2 3GPP model w/ beta=0.00718 | Rel-15 1+1(4L), Rel-15 2+2(6L) | 3 | Ideal |
| Vivo | R4-2418188 | Op 1, Ops 3-4 | Rel-15 1+1(4L), Rel-15 2+2(6L) | 3 | Ideal, 3GPP with alpha=0.1 |
| Huawei, HiSi | R4-2419029 | Ops 2-3 | Rel-15 1+1(4L), Rel-15 2+2(6L) | 3 | 3GPP with alpha=0.05 |
| Samsung | R4-2418619 | Ops 3-4 | Rel-15 1+1(4L), Rel-15 2+2(6L) | 0 | Ideal, 3GPP with alpha=0.05 |

Conversely, some companies reported no gains in their studies. We now analyze the simulation assumptions from these companies to understand the potential impact on their results. As highlighted in yellow in Table 3, it is evident that a combination of overly conservative UE correlation, higher Tx EVM, unequal DMRS overhead for 6L or higher BS correlation was assumed. This led to 6L not outperforming 4L, unlike the results shown in Table 2.

In our study, we found that with an alpha value of 0.00514, a Rel-18 1+1 DMRS configuration, and UE correlation option 1, even with a 1% TxEVM, 6L only surpasses 4L at around 40dB and 5L above 44dB. Naturally, a DMRS configuration with unequal overhead for 6L, such as 2+2 DMRS configuration for 6L, will further negatively impact its performance. Therefore, it is not surprising that these assumptions lead to poor performance for 6L.

Observation 14: A combination of overly conservative UE correlation, higher Tx EVM, unequal DMRS overhead for 6L or higher BS correlation assumptions led to no performance gain for 6L in some companies' studies.

8. Impact of Number of Transmit Antenna Ports

In this section, we present our results with a larger number of transmit antenna ports. We use 32Tx/4L Rel-18 1+1 DMRS as a baseline and compare it with 32Tx antenna ports for both 5L and 6L configurations. We assume that SRS antenna switching is available for TDD scenarios and SVD-based beamforming at the transmitter. We use an update periodicity of 40 slots (i.e., 20ms for TDD scenarios), which provides a reasonable trade-off between SRS overhead and tracking the time-selective channel in a realistic deployment. We limit our simulations to correlation matrices from options 1 and 2 to reduce the simulation burden. All other simulation assumptions remain unchanged as outlined in Appendix B.

Figure 7: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 2 and 32 transmit antenna ports

From Fig. 7, we observe that 32 Tx antenna ports offer noticeable performance gain with respect to the reference 32Tx scenario even before reaching peak throughput for both correlation options 1 and 2. Similar gains are anticipated for correlation options 3 and 4 as well. We notice that 6L configuration consistently starts to outperform that of 4L above 28dB while the 5L shows some gains over the 4L configuration already at 24dB SNR for both correlation matrices. Furthermore, we see a 5dB improvement in the 5L to 6L switchover point at around 33dB with 32Tx antenna ports when compared to baseline scenario for both correlation matrices.

Observation 15: For the same DMRS configuration, specifically Rel-18 1+1 with SRS-AS and SVD-based beamforming for TDD, the 6L configuration consistently starts to outperform the 4L configuration at around 28dB. Additionally, the 5L configuration shows some gains over the 4L configuration already at 24dB SNR for both correlation matrices.

Observation 16: For the same DMRS configuration (Rel-18 1+1 with SRS-AS and SVD-based beamforming for TDD), the switchover point from 5L to 6L occurs at approximately 33dB.

These findings indicate that the 32Tx configuration, frequently observed in the field (as shown in Table 1 in the main section), can substantially reduce the required SNR to achieve desired performance in deployment scenarios.

9. Impact of Non-ideal Antenna Efficiency

During the last meeting, it was discussed that RAN4 should incorporate antenna efficiency into the envelope correlation coefficients (ECC) for link-level simulations. In this section, we examine the impact of non-ideal antenna efficiency on throughput performance. To this end, we consider two sets of antenna efficiency profiles. In the first set, the non-ideal efficiency across 6Rx antennas ranges from no better than -3.5dB to no worse than -7.5dB, with an average antenna efficiency of approximately -5.3dB and an SNR imbalance of 3.6dB. For the second set, we use the measured antenna results provided in [2], which have an average antenna efficiency of approximately -5.2dB and an SNR imbalance of 4.3dB.

For these simulations, we use 32Tx antennas and the correlation matrix from option 1. The goal is to demonstrate that the degradation caused by non-ideal antenna efficiency can be compensated for through beamforming (BF) gain. The throughput performance relative to the baseline scenario (32Tx/4L with non-ideal efficiency) is provided in Figures 8 and 9 for sets 1 and 2, respectively.

Figure 8: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 2 in the presence of non-ideal antenna efficiency from profile 1.

From Fig. 8, we observe that even with non-ideal antenna efficiency, the 32Tx configuration provides significant performance gains for both 5L and 6L compared to 4L. The figure shows that 6L achieves a gain slightly above 38 dB over 5L. This indicates approximately a 5.5dB degradation in the SNR switchover point from 5L to 6L, which aligns with the average antenna inefficiencies for this profile.

Figure 9: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 2 in the presence of non-ideal antenna efficiency from profile 2.

From Fig. 9, we observe similar performance trends: the 32Tx configuration offers significant gains for both 5L and 6L compared to 4L. However, due to a larger antenna imbalance, profile 2 shows slightly less performance gain compared to the results in Fig. 8 with profile 1. In this case, the 5L to 6L switchover occurs at an SNR slightly above 38dB. As with profile 1, this also indicates approximately a 5.5dB degradation in the SNR switchover point from 5L to 6L.

Observation 17: Even if antenna inefficiencies are considered from both profiles, 6Rx/6L feature offers consistent gains over 6Rx/4L starting from SNR = 26 dB

Observation 18: Despite non-ideal antenna efficiency, the 32Tx configuration still offers significant performance gains for both 5L and 6L over 4L, comparable to those with ideal antenna efficiency. The 6L configuration consistently starts to outperform the 5L configuration slightly above 38dB.

Observation 19: The switchover SNR from 5L to 6L is degraded by 5.5dB for the considered antenna efficiency profiles. This degradation in the SNR switchover point is largely determined by the average antenna inefficiencies across the receive antennas.

Observation 20: A larger imbalance in antenna inefficiencies across the receive antennas results in a degradation of performance compared to an antenna efficiency profile with a similar average antenna inefficiency but smaller antenna imbalance.

Our findings demonstrate that using many transmit antennas, specifically 32 transmit antenna ports, can help recover the performance losses caused by non-ideal antenna efficiency. These results suggest that antenna efficiency may not be a limiting factor in practical deployment scenarios for 6Rx devices.

9. 6-layer support

During the last meeting, RAN4 explored the feasibility of introducing 6-layer performance requirements for 6Rx devices. The proposed way forward is outlined below.

6-layer Support Feasibility

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices

-	Option 3: Companies to further discuss align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility for handhelds using the WF in R4-2417209 and any updates related to Issue 2-1-2 above.

-	Option 4: 6-layer support is not feasible for handheld UEs

Based on the simulation results with various DMRS types and configurations, the number of transmit antenna ports, and practical UE and BS correlation assumptions, we believe it is feasible to introduce 6-layer performance requirements for 6Rx devices.

Proposal 4: RAN4 shall introduce 6-layer performance requirements for handheld and FWA devices.

10. Conclusions

In this paper, we present observations from field data and provide link-level simulation results for the 6Rx device. The following have been proposed.

Observation 1: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% between SNR = 0dB and SNR = 10dB. At SNR > 20 dB, the percentage of 4L even increases to up to 50%.

Observation 2: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% for RSRP between -100dBm and -80dBm. The likelihood of 4L increases up to 40% above -80dBm.

Observation 3: Given the likelihood of 4L in a commercial device with 6Rx above -80dBm RSRP is up to 40% and up to 50% for SNR > 20 dB in an urban environment, more than 4L can be expected in mid-cell and cell-center scenarios.

Observation 4: The 32Tx antenna configuration is frequently observed in TDD deployment scenarios and is supported by several operators and major infrastructure vendors worldwide.

Observation 5: Six receive antennas provide very large throughput gain for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm in the order of 35% and 45%, respectively.

Observation 6: An advanced receiver, such as R-ML receiver can offer noticeable performance improvement over an MMSE receiver for 6Rx devices.

Observation 7: The following observations can be made for achievable average number of layers:

- Six receive antennas can achieve 5-layer transmission for SSB RSRP > -80 dBm and -80 dBm<SSB RSRP<-100 dBm with sub-optimal MMSE processing.

- Six receive antennas can achieve 6-layer transmission with a R-ML receiver when SSB RSRP > -80 dBm.

Observation 8: Major BS vendors and operators have confirmed that the Tx EVM in practical BS equipment can be significantly lower than the values assumed by RAN4 in performance requirements, typically ranging between 1% and 1.25%.

Proposal 1: Support option 1 (Use TxEVM = 1%)

Observation 9: Major BS vendors have indicated that BS correlation can be neglected for the feasibility study.

Proposal 2: RAN4 shall use no correlation on BS Tx Ports (3GPP MIMO correlation matrix with Alpha = 0.0)

Proposal 3: In addition to 8Tx/6Rx antenna configuration, RAN4 should include the 32Tx/6Rx antenna configuration in the simulation assumptions.

Observation 10: Both 5L and 6L exhibit similar performance trends with correlation matrices from options 1 to 4 under the given simulation assumptions. With the ECC matrices from four companies, both 5L and 6L configurations provide gains over 4L

Observation 11: For 6L, a minimal performance loss of 3% at 38dB SNR is observed when using the correlation matrix from option 2 compared to option 1.

Observation 12: For 6L, minimal performance losses are observed with correlation matrices from options 3 and 4 compared to option 1, specifically 7% and 3% losses, respectively. The SNR loss is approximately 1.5dB for option 3 and around 1dB for option 4.

Observation 13: Companies that assume a typical Tx EVM in the range of 1 to 1.25%, with no BS correlation, equal DMRS overhead and measured UE ECC matrices, observe that 6L outperforms 4L at similar SNR levels.

Observation 14: A combination of overly conservative UE correlation, higher Tx EVM, unequal DMRS overhead for 6L or higher BS correlation assumptions led to no performance gain for 6L in some companies' studies.

Observation 15: For the same DMRS configuration, specifically Rel-18 1+1 with SRS-AS and SVD-based beamforming for TDD, the 6L configuration consistently starts to outperform the 4L configuration at around 28dB. Additionally, the 5L configuration shows some gains over the 4L configuration already at 24dB SNR for both correlation matrices.

Observation 16: For the same DMRS configuration (Rel-18 1+1 with SRS-AS and SVD-based beamforming for TDD), the switchover point from 5L to 6L occurs at approximately 33dB.

Observation 17: Even if antenna inefficiencies are considered from both profiles, 6Rx/6L feature offers consistent gains over 6Rx/4L starting from SNR = 26 dB

Observation 18: Despite non-ideal antenna efficiency, the 32Tx configuration still offers significant performance gains for both 5L and 6L over 4L, comparable to those with ideal antenna efficiency. The 6L configuration consistently starts to outperform the 5L configuration slightly above 38dB.

Observation 19: The switchover SNR from 5L to 6L is degraded by 5.5dB for the considered antenna efficiency profiles. This degradation in the SNR switchover point is largely determined by the average antenna inefficiencies across the receive antennas.

Observation 20: A larger imbalance in antenna inefficiencies across the receive antennas results in a degradation of performance compared to an antenna efficiency profile with a similar average antenna inefficiency but smaller antenna imbalance.

Proposal 4: RAN4 shall introduce 6-layer performance requirements for handheld and FWA devices.

Observation 21: The Rel-18 1+1 DMRS configuration provides performance comparable to the Rel-15 2+0 configuration.

Observation 22: The Rel-18 1+1 DMRS configuration is anticipated to perform better in higher Doppler scenarios compared to the Rel-15 2+0 configuration, due to the additional DMRS symbol.

Observation 23: The Rel-18 1+1 DMRS configuration demonstrates a noticeable performance gain over the Rel-15 2+0 configuration starting from 32dB SNR.

Observation 24: With the Rel-18 1+1 DMRS configuration, the 6L configuration begins to outperform the 5L configuration just above 38dB, and the 5L configuration starts to outperform the 4L configuration at around 22dB.

Observation 25: For the Rel-15 2+2 DMRS configuration, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 26: With the Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead. For instance, approximately 27% and 17% throughput losses are observed at high SNR regime for 5L and 6L, respectively.

Proposal 5: RAN4 should consider using Rel-18 eType1 DMRS for 6RX simulation assumptions.

11. References

[1] R4-2410376, “WF on requirements for 6Rx,” 3GPP TSG RAN WG Meeting #113, Orlando, USA, Nov 18 – 22, 2024.

[2] R4-2417561, “Feasibility evaluation of MIMO layer for 6Rx UE,” 3GPP TSG RAN WG Meeting #113, orlando, USA, Nov 18 – 22, 2024.

[3] TS 38.211, ‘Physical channels and modulation (Release 18)’, v18.2.0, March 2024

Appendix A: Simulation Results

1.1 Simulation Assumptions

For our simulations, we use the assumptions outlined in Appendix A. We consider a 4L configuration with 6 receive antennas and 8 transmit antenna ports as the reference scenario. Given that the 32Tx antenna configuration is commonly observed in TDD field scenarios, we also include 32 transmit antenna ports in our link-level simulations.

For the 8Tx simulations, we use fixed PMI, while for the 32Tx simulations, we employ SVD-based precoding to evaluate beamforming gain. We note that fixed PMI becomes as mismatched to the channel as random PMI after one period of the channel's coherence time. Since we update PMI at the beginning and then exclude several slots, the results should be very similar to those obtained with random PMI. The simulation assumptions are provided in Appendix B.

1.2 Link-level Simulation Results

1.2.1 Impact of Demodulation Reference Symbols

We note that to support more than four MIMO layers, it is necessary to configure six or more DMRS antenna ports. This can be accomplished using various DMRS configurations, such as the Rel-15 and Rel-18 eType1 DMRS configurations.

- Rel-15 Type1 DMRS with two frontloaded DMRS symbols: Type1 DMRS supports only four orthogonal DMRS ports with one frontloaded DMRS symbol.

- Rel-18 eType1 DMRS with one frontloaded and one additional DMRS symbols: eType1 DMRS allows to configure up to eight orthogonal DMRS ports with one frontloaded DMRS symbol [3]. The larger number of DMRS ports per symbol comes at the expense of a reduced frequency density of DMRS resource elements. Using only a single frontloaded DMRS is sensitive to higher Doppler spread. Therefore, we consider a configuration with one frontloaded and one additional DMRS symbols.

The simulations apply MCS link adaptation via ACK/NACK feedback targeting 10% first-tx BLER and fixed rank  {4,5,6} for 6Rx antennas with MCS Table 2. The number of Tx antenna ports is assumed eight and an advanced R-ML receiver is considered for these results.

1.2.2 Performance Comparison with Rel-15 2+0 and Rel-18 1+1 DMRS Configurations

Figures 6 and 7 shows the relative throughput for five and six layers with respect to a reference scenario for each SNR value with MCS Table 2. The receiver correlation matrix is option 1 from Table 1 in Appendix B and we assume zero Tx antenna correlation. The reference scenario applies four layers and 6Rx antennas with 8Tx antenna ports and configured with Rel-18 1+1 DMRS configuration. A positive relative throughput means that five and/or six layers provide a gain over four layers for 6Rx antennas with the same number of transmit antenna ports.

Figure 10: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with correlation matrix from option 1 for 5L

Figure 11: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with correlation matrix from option 1 for 6L

As shown in Figs. 10 and 11, the Rel-18 1+1 DMRS configuration already outperforms the Rel-15 2+0 configuration starting at 32dB, with the performance gap widening at higher SNR levels. With the addition of one more DMRS symbol, it is expected that the Rel-18 1+1 DMRS configuration will surpass the Rel-15 2+0 configuration in higher Doppler scenarios. Additionally, when comparing the 5L and 6L performances with the Rel-18 1+1 DMRS configurations, it is observed that the 6L configuration begins to outperform the 5L configuration at around 30dB, demonstrating the feasibility of 6L performance.

Observation 21: The Rel-18 1+1 DMRS configuration provides performance comparable to the Rel-15 2+0 configuration.

Observation 22: The Rel-18 1+1 DMRS configuration is anticipated to perform better in higher Doppler scenarios compared to the Rel-15 2+0 configuration, due to the additional DMRS symbol.

Observation 23: The Rel-18 1+1 DMRS configuration demonstrates a noticeable performance gain over the Rel-15 2+0 configuration starting from 32dB SNR.

Observation 24: With the Rel-18 1+1 DMRS configuration, the 6L configuration begins to outperform the 5L configuration just above 38dB, and the 5L configuration starts to outperform the 4L configuration at around 22dB.

1.2.3 Performance Comparison with Rel-15 2+2 DMRS configurations

Now, we compare the performance results with Rel-15 2+2 DMRS configurations. In these results, we use a 4L/Rel-15 2+2 DMRS configuration as the reference scenario. The objective is to evaluate the performance of 5L and 6L against this reference. It is important to note that this configuration significantly increases the DMRS overhead, leading to lower peak throughput compared to the 2+0 configuration. For these simulation results, we use the correlation matrix from option 1.

Figure 12: Throughput performance relative to 6Rx/4L Rel-15 2+2 DMRS

The relative performance is illustrated in Fig. 12. These results show that for the Rel-15 2+2 configuration, 6L begins to outperform 4L at around 32dB and 5L at approximately 37dB.

Observation 25: For the Rel-15 2+2 DMRS configuration, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 26: With the Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead. For instance, approximately 27% and 17% throughput losses are observed at high SNR regime for 5L and 6L, respectively.

We observe that eType1 DMRS is likely to become more common in advanced 5G use cases due to its reduced overhead and better support for higher-order MIMO. Additionally, RAN4 has already introduced requirements for eType1 DMRS during the Rel-18 timeframe [3]. Based on the simulation results, it is evident that Rel-18 eType1 DMRS offers comparable performance with just one frontloaded DMRS symbol compared to the Rel-15 2+0 case, and it also provides robustness in high Doppler scenarios due to one additional DMRS symbols. Therefore, we believe that Rel-18 eType1 DMRS should be considered for feasibility evaluation and performance requirements for 6Rx devices.

Proposal 5: RAN4 should consider using Rel-18 eType1 DMRS for 6RX simulation assumptions.

1.3 Summary of Simulation Results

Based on the simulation assumptions provided in Table 1 of Appendix B, this section presents simulation results with different DMRS overheads (both Rel-15 Type 1 and Rel-18 eType1). It is observed that the Rel-18 1+1 DMRS configuration provides performance comparable to the Rel-15 2+0 configurations. Additionally, the 6Rx/6L configuration outperforms the 6Rx/4L configuration at around 33dB SNR and the 6Rx/5L configuration just above 38dB SNR for MCS Table 2.

Although Rel-15 2+2 DMRS configurations maintain the same DMRS overhead for 4L, 5L, and 6L, the increased DMRS overhead results in a 17% throughput loss at high SNR for 6L, making it less appealing for practical use. Additionally, field data in this paper indicates that 4L occurs up to 50% of the time, and more than 4L is expected in mid-cell and cell-center scenarios. Consequently, networks may avoid using Rel-15 2+2 DMRS configurations for 6L scheduling in these situations.

Appendix B: Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} {32Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,5,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Fixed PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx |
| PDSCH DMRS | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | [Rel-18 1+1(6L), Rel-15 2+0(4L)] [Rel-18 1+1(6L), Rel-15 1+1(4L)] [Rel-18 1+1(6L), Rel-18 1+1(4L)] [Rel-15 2+2(6L), Rel-15 2+2(4L)] |
|  | Precoding | Fixed PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 |
| Antenna Efficiency (dB) |  | Profile 1: Bounded between -3.5dB to -7.5dB, with an average antenna efficiency of -5.3dB Profile 2: [-7.8, -6.7, -5.5, -3.5, -4.4, -3.7] |
| BS correlation matrix |  | 3GPP MIMO correlation matrix with Alpha = 0.0 |
| Tx EVM |  | 1% |

Table 2: Simulation Assumptions