# 3GPP TSG RAN WG4 Meeting#114	R4-2500116

# Athens, GR, Feb 17 – Feb 21, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussion on NTN testing for NGSO

Agenda item:	7.10.4

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to propagator models and test procedures [1]. In this paper, we share our perspectives on the remaining open issues and introduce new observations regarding the estimation of Doppler shift and propagation delay.

2. TE Emulated Channel Model

2.1 Derivation of time varying Doppler shift and propagation delay

As part of the post-meeting alignment process across companies after RAN#113, we have shared our data in the email thread. We observed a good match in the Doppler shift and propagation delay estimations, including the intermediate steps as agreed upon in [1]. Below, we present our estimated Doppler shift and propagation delay at T1=1sec.

Table 1: Estimated Doppler shift and propagation delay at T1=1sec

| Time T1 (sec) | Estimated Doppler shift (Hz) | Estimated propagation delay (sec) |
| --- | --- | --- |
| 1 | -5.446583509875E+02 | 2.031460967422E-03 |

Additionally, as illustrated in Fig. 1 below, we have provided results for the estimated Doppler shift and propagation delay over a 1000-second period. These results use Keplerian estimates at one second interval to obtain the Doppler shift and propagation delay, with the initial ephemeris information taken from TS 38.521-5.

Fig. 1: Estimated Doppler shift and propagation delay obtained using the Keplerian estimate over a 1000-second period.

It is observed that the estimated Doppler shift and propagation delay are well aligned across companies.

Observation 1: The Estimated Doppler shift and propagation delay based on the Keplerian estimate, as well as the intermedia steps are consistently aligned across companies.

2.2 Stopping criteria for the Newton-Raphson method

Number of iteration and accuracy criteria for stopping numerical method

        - Number of iterations

- Option 1: 4

        - Accuracy requirement E-E0>ϵ

- Option 1: ϵ=10-8

- Option 2: ϵ=10-6

During RAN#113, it was agreed that the Newton-Raphson method would be used to solve Kepler’s equation. Both the number of iterations and accuracy requirements can be considered as stopping criteria. The proposed way forward is presented below [1].

In this context, we observed during the post-meeting alignment exercise that all companies required only 4 iterations, regardless of the accuracy requirements. This indicates that robust accuracy can be achieved with just 4 iterations. Therefore, we propose using accuracy requirements as the stopping criterion for the Newton-Raphson method.

Observation 2: Robust accuracy can be achieved with 4 iterations.

Proposal 1: Do not consider the number of iterations as a stopping criterion for the Newton-Raphson method.

Proposal 2: Consider option 1 for the accuracy requirement (𝝐=10-8).

2.3 Interpolation method for the Doppler shift and Propagation delay

Numerical method

- Option 1: cubic spline interpolation

- Option 2: linear interpolation

- Option 3: calculate propagation delay and Doppler shift at any time instant

- Other options are not precluded

As specified in Step 3 of Issue 1-1-1 [1], the estimated Doppler shift and propagation delay at times T0 and T1 will be interpolated over the T1-T0 period to obtain the estimated Doppler shift and propagation delay at the agreed-upon update periodicity. The proposed way forward is outlined below.

In this context, our preference is to use linear interpolation to keep the computational complexity manageable.

Proposal 3: Support Option 2 (linear interpolation).

2.4 Accuracy requirements for the Doppler and Propagation Delay

Accuracy requirement

- Option 1:

- Max|delay-error| ~ 3e-13 s

- Max|delay-rate error| ~ 3.28e-12

- Other options not precluded

As part of our RAN4 contribution during RAN4#113, we shared our observations on the propagation delay and Doppler shift accuracy that can be achieved via cubic spline interpolation []2. As shown below, it appears that these observations were captured in the way forward as the propagation delay and Doppler shift accuracy requirements [1].

However, these accuracies were simply observations from our side for a given interpolation technique and the intention was not to propose them as requirements.

Proposal 4: Do not consider accuracy requirements for the estimation of propagation delay and Doppler shift.

2.5 Assumption of Elevation Angle Range

Assumption of Elevation angle range at time 0

Set more satellite position/velocity depending on test purpose and pre-existing test configuration

    - For demodulation test

      - Option 1: starting from [30°] for both NR and IoT NTN

      - Option 2:

        - starting from [ 30°] for NR NTN

        - starting from [10°] for IoT NTN

    - For RF test

      - starting from [10°] for both NR and IoT NTN with considering maximum positive/negative doppler

      - starting from [60°] for both NR and IoT NTN with considering the half maximum positive/negative doppler

    - For RRM test

      - Option 1: starting from [30°]

      - Option 2: starting from [90°]

The proposed way forward for this issue is outlined below.

We note that in previous releases, such as Rel-17 NR NTN, a 30-degree elevation angle was used for both demodulation and RRM test cases. Therefore, we prefer to maintain the 30-degree elevation angle unless there is a justified reason to change it.

Proposal 5: Option 1 for both RRM and demod test cases (consider 30-degree initial elevation angle for both RRM and demod test cases).

2.6 Initial Ephemeris Information

Initial ephemeris information

  - Option 1

    - Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model

  - Option 2

    - GMAT

  - Other options are not precluded

The proposed WF for the initial ephemeris information is outlined below.

We note that during the post-meeting alignment process, the initial ephemeris information was assumed as specified in TS 38.521-5 for LEO600 satellite. It appears that results across all companies are well aligned. Therefore, we believe that the initial ephemeris information can be taken from TS 38.521-5 for LEO600 and from TS 38.508-1 for LEO1200.

Proposal 6: Support Option 1 (Consider the initial ephemeris information as specified in TS 38.521-5 and TS 38.508-1).

2.7 Update granularity for propagation delay and Doppler shift calculation

Assumption of updated granularity for propagation delay and Doppler shift calculation

- WF

- Option 1 (WF in the last meeting)

    - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model per slot

    - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot

During the previous meeting, RAN4 discussed the update granularity for the propagation delay and Doppler shift calculation. The proposed way forward is outlined below.

We note that the hardware/software resources and complexity associated with an update granularity of less than 1ms, particularly for SCS > 15kHz, could be prohibitively expensive. Additionally, the observed maximum absolute delay error (3e-13s) and delay-rate error (3.28e-12) for a 1ms update granularity are sufficiently small [2]. Therefore, we propose using a 1ms update granularity.

Proposal 7: Consider 1ms update granularity for the estimation of propagation delay and Doppler shift.

2.8 Consideration of Earth rotation during signal flight time

In our Doppler shift and propagation delay estimation thus far, RAN4 has not factored in the Earth's rotation during the signal's flight time. This means that both the satellite and the UE are assumed to be static while the signal is propagating. Assuming a static satellite during signal flight time for NGSO is an artificial scenario, potentially leading to some estimation errors. Below, we present the observed propagation delay error resulting from ignoring the Earth's rotation during signal flight time. For this study, we use the same simulation assumptions as those considered for the post-meeting alignment exercise.

Fig. 2: Propagation delay error due to ignoring Earth's rotation during signal flight time

As illustrated in Fig. 2, at 262 seconds (corresponding to a 10-degree elevation), the delay estimation error is 0.1493 µs, while at 128 seconds (corresponding to a 30-degree initial elevation angle), the delay estimation error is 0.075 µs. We note that the maximum delay error could reach 0.5 µs. This propagation delay error could be significant for certain test cases, such as UL timing test cases. Therefore, we believe this phenomenon warrants further study in RAN4.

Observation 3: A significant error in propagation delay is noted when the Earth's rotation during signal flight time is not considered in the estimation.

Let us assume that tR is the signal’s receive time and considering the same mathematical notations as in [1], the corresponding propagation delay considering the earth rotation, DelayERtR, can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b, respectively.

Proposal 8: RAN4 should account for the Earth's rotation during signal flight time for more accurate estimations of propagation delay and Doppler shift.

Proposal 9: Considering the earth rotation, the propagation delay, DelayERtR, can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b, respectively.

2.9 Update granularity for Keplerian estimates

We note that updating Keplerian elements every 1.024 seconds is advantageous as it aligns with SIB periodicity. This approach allows the position and velocity state vectors to be included in SIB without the need to solve the Kepler equation again.

Observation 4: Updating Keplerian elements every 1.024 seconds (instead of 1 second) is advantageous as it aligns with SIB periodicity.

Proposal 10: Adjust the interval for Keplerian model-based propagation delay and Doppler shift estimation from 1 second to 1.024 seconds.

3. RAN4 Requirements and Tests

3.1 UE Assumption for the Propagator Model

So far, RAN4 has briefly discussed the assumptions regarding the UE for the propagation model. The proposed way forward is outlined below.

UE assumption on propagator model for satellite motion for RAN4 simulation purpose

  - Option 1: Consider Keplerian model for UE assumption on the propagator model

  - Option 2:

    - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

  - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information.

To ensure consistent results across companies during the propagation model evaluation phase, it would be beneficial to adopt a specific propagation model on the UE side. Our preference is to use the Keplerian propagation model, as it provides a good balance between implementation complexity and performance. However, this does not prevent the UE from implementing a more accurate propagation model than the Keplerian model.

Proposal 11: Support option 1 (Keplerian model for the UE assumption on the propagator model)

4. Conclusions

In this paper, we present our perspectives on the performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Observation 1: The Estimated Doppler shift and propagation delay based on the Keplerian estimate, as well as the intermedia steps are consistently aligned across companies.

Observation 2: Robust accuracy can be achieved with 4 iterations.

Proposal 1: Do not consider the number of iterations as a stopping criterion for the Newton-Raphson method.

Proposal 2: Consider option 1 for the accuracy requirement (𝝐=10-8).

Proposal 3: Support Option 2 (linear interpolation).

Proposal 4: Do not consider accuracy requirements for the estimation of propagation delay and Doppler shift.

Proposal 5: Option 1 for both RRM and demod test cases (consider 30-degree initial elevation angle for both RRM and demod test cases).

Proposal 6: Support Option 1 (Consider the initial ephemeris information as specified in TS 38.521-5 and TS 38.508-1).

Proposal 7: Consider 1ms update granularity for the estimation of propagation delay and Doppler shift.

Observation 3: A significant error in propagation delay is noted when the Earth's rotation during signal flight time is not considered in the estimation.

Proposal 8: RAN4 should account for the Earth's rotation during signal flight time for more accurate estimations of propagation delay and Doppler shift.

Proposal 9: Considering the earth rotation, the propagation delay, DelayERtR, can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEF and vtRECEF are the satellite position and velocity vectors at time tR, ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a,b> is the dot product between a and b,

Observation 4: Updating Keplerian elements every 1.024 seconds (instead of 1 second) is advantageous as it aligns with SIB periodicity.

Proposal 10: Adjust the interval for Keplerian model-based propagation delay and Doppler shift estimation from 1 second to 1.024 seconds.

Proposal 11: Support option 1 (Keplerian model for the UE assumption on the propagator model)

5. References

[1] R4-24197866, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG Meeting #113, Orlando, USA, Nov 18 – 22, 2024.

[2] R4-2418438, “Discussion on NTN testing for NGSO,” 3GPP TSG RAN WG Meeting #113, Orlando, USA, Nov 18 – 22, 2024.