3GPP TSG-RAN WG4 Meeting #114	                                 R4-2500117

Athens, Greece, February 17 – February 21, 2025

Source: 	Murata Manufacturing Co Ltd.

Title: 	Further discussion on HPUE MSD rules

Agenda item:	7.1.1.1.1

Document for:	Discussion

Introduction

In the last RAN4#113 meeting, the general MSD rules for HPUE was discussed and the WF [1] was approved. In this WF, it is agreed that RAN4 specifies or provides equations/LUT/simplified guidelines identifying the HPUE MSD either as normative or informative requirements and the simplification rules provided by proponent companies were analyzed. However, the detailed rules and methods need further discussion and consensus between interested companies.

In this contribution	we evaluate the HPUE MSD calculation method further in detail based on the existing PC3 MSD. From the analysis results, we make some proposals to make the discussion move forward.

Discussion

One UL HPUE MSD calculation (Harmonic, Harmonic mixing, Crossband isolation)

It is recommended that the existing specified 1Tx PC3 MSD should be used as a reference when deriving HPUE MSD [2]-[7].

In general, MSD is calculated using the MRC equation with primary receiver and secondary receiver noise composed of thermal and interferer noise. But it should be noted that the primary receiver noise and the secondary receiver noise are not necessarily same and there might be interferer power imbalance between them. This is because the RF front end architecture is not always symmetrical for both primary and secondary receiver path. So, we need to consider this interferer power imbalance for the MSD calculation of HPUE.

This is well explained in several contributions in last meeting [2]-[7] and summarized in WF [1]. In our understanding, the calculation equation and procedure provided by three companies is same except some minor assumptions which are interferer power imbalance, RevIM (Rtx) and whether 2Tx PC2 and 1Tx PC2 MSD should be separately specified or only 2Tx PC2 MSD should be specified or not.

In our view, specifying two sets of requirements of PC2 is reasonable since 1Tx PC2 and 2Tx PC2 have different RF architecture, and this is aligned with the current PC2 MSD table format. Also, MSD is minimum requirement for all types of UE, so 9dB interferer power imbalance and 0.5dB RevIM (Rtx) assumptions might be enough to cover most cases as in [5]. By using 9dB interferer power imbalance, the PC1.5 graph in [1] can be well aligned.

Proposal 1: RAN4 to specify two sets of requirements of PC2 (1Tx PC2 and 2Tx PC2).

Proposal 2: RAN4 to use 9dB interferer imbalance and 0.5dB RevIM (Rtx) to derive the HPUE MSD from the existing PC3 MSD.

Figure 1 shows the delta MSD relative to the default PC3 MSD using 9dB interferer power imbalance and 0.5dB RevIM (Rtx) for 2Tx. This is well aligned with the current PC2 MSD specifications [5].

Figure 1. Delta MSD between PC2 (1Tx and 2Tx), PC1.5 and PC3

Two UL HPUE MSD calculation (IMD)

Unlike one UL HPUE MSD calculation, two UL HPUE MSD calculation due to IMD from two aggressors is not simple and needs more analysis. This is because there are many power class combinations for one single band, different number of Tx, different IMD order and coefficient and different linearity between PC3 PA and PC2 PA.

Table 1 summarizes five categories of power class and number of Tx – PC3 2Tx (baseline), PC2 2Tx, PC2 3Tx, PC1.5 2Tx and PC1.5 3Tx. The third and the fourth column means power combination to achieve 2 UL CA power class and PA power capability for each band respectively. The fifth column denotes the case number in WF [1]. It should be noted that some cases (5, 6, 7, 8, 10) were not specified until Rel-18 and we do not yet have specified MSD for those cases.

We will investigate two UL HPUE MSD rules for each category based on PC3 MSD (case #1) with different IMD orders.

Table 1 Two UL MSD categories of PC3, PC2 and PC1.5 with power class combinations

|  | Combination | Band A | Band B |  | Case # | Rel |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 2Tx | Power | 20 | 20 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 1 | R15 |
| PC2 2Tx | Power | 23 | 23 |  |  |  |
|  | PA | PC3 PA | PC3 PA |  | 2 | R17 |
|  |  | PC3 PA | PC2 PA |  | 3 | R17 |
|  |  | PC2 PA | PC2 PA |  | 5 | R19 |
| PC2 3Tx | Power | 23 | 23 |  |  |  |
|  |  |  | 20 | 20 |  |  |
|  | PA | PC3 PA | PC2 PA | PC2 PA | 4 | R18 |
|  |  | PC2 PA | PC2 PA | PC2 PA | 6 | R19 |
| PC1.5 2Tx | Power | 26 | 26 |  |  |  |
|  | PA | PC2 PA | PC2 PA |  | 7 | R19 |
| PC1.5 3Tx | Power | 23 | 27.8 |  |  |  |
|  |  |  | 24.8 | 24.8 |  |  |
|  | PA | PC3 PA | PC2 PA | PC2 PA | 9 | R18 |
|  | Power | 26 | 26 |  |  |  |
|  |  |  | 23 | 23 |  |  |
|  | PA | PC2 PA | PC3 PA | PC3 PA | 8 | R19 |
|  |  | PC2 PA | PC2 PA | PC2 PA | 10 | R19 |

PC2 2Tx/3Tx MSD calculation (case # 2~6)

The procedure to calculate PC2 MSD due to IMD is same as one UL case as shown in [3]. Therefore, 9dB interferer imbalance can be assumed, too. And the only remaining and more important parameters are how much interferer is increased compared to PC3. It is shown that the interferer increase is highly dependent on PA capability [3].

There are three PA combinations in Table 1 for PC2 2Tx which are PC3 PA + PC3 PA (case 2), PC3 PA + PC2 PA (case 3) and PC2 PA + PC2 PA (case 5). It is well known that PC2 PA IPn is 3dB higher than PC3 PA IPn, so the interferer increase for case 5 is the smallest. However, it is not reasonable to specify three PC2 2Tx MSD requirements in spec. Moreover, the IMDn from other components except PC2 PA can be dominant or comparable for some cases such as triple beat scenario. Considering MSD is the minimum requirement and to simplify the MSD rules as much as possible, the worst-case interferer increase (PC3 PA + PC3 PA, case 2) should be referred to calculation. In conclusion, the interferer increase for IMDn should be 3*n dB.

Proposal 3: RAN4 to use 3*n dB interferer increase to derive the PC2 2Tx MSD due to IMDn.

During the Rel-18 discussion, it was agreed that the requirements specified based on 1Tx-1Tx UL configuration are applicable to 1Tx-2Tx UL configuration (case 4). There is no consideration of MRC imbalance change between 2Tx and 3Tx, higher headroom for PC2 TDD PA versus a PC3 TDD PA, or different coupling between PAs in a 2Tx and 3Tx architecture [7]. Therefore, there is no discrepancy between PC2 2Tx and PC2 3Tx in the spec.

Following the same principle, PC2 2Tx MSD calculation can be applicable to PC2 3Tx MSD calculation even though all PAs used in case 6 have 3dB higher IPn which is the new combination under the scope of Rel-19.

Proposal 4: RAN4 to re-use 3*n dB interferer increase to derive the PC2 3Tx MSD due to IMDn. In other words, PC2 2Tx MSD calculation is applicable to PC2 3Tx MSD calculation.

Figure 2 shows delta MSD between PC2 2Tx/3Tx and PC3. It is verified that this is well aligned with the current existing PC2 MSD requirements except for a few cases [7].

Figure 2. Delta MSD between PC2 2Tx/3Tx and PC3 for IMD orders

PC1.5 2Tx/3Tx MSD calculation (case # 7~10)

For PC1.5, there are two power combinations: 26dBm + 26dBm/2*23dBm (case 7, 8, 10, Rel-19) and 23dBm + 2*24.8dBm (case 9, Rel-18). So, we will propose separate calculations in this contribution. It is FFS whether only one unified PC1.5 MSD requirements from Rel-19 are used or not.

PC1.5 2Tx/3Tx MSD calculation of 26dBm + 26dBm/2*23dBm

Firstly, we will present the PC1.5 2Tx MSD calculation of 26dBm + 26dBm power combination (case 7).

Two PC2 PAs are used which means that the IMDn increase is 3+3*n dBm from PC3 PAs. For most cases, PA IMD contribution is dominant so using 3+3*n dBm IMDn increase is one good candidate option. However, this combination has not been studied until now, so it is better to consider various options in this phase.

Figure 3 shows IMDn increase from PC3 to PC1.5 depending on the difference between IMD of PC2 PA and other components. If PC2 PA IMDn is much larger, e.g. by 20dB, the IMDn increase is close to 3+3*n dBm. If other components IMDn is dominant, the IMDn increase is converged to 6*n dBm. So, the IMDn increase is between 3+3*n dBm and 6*n dBm depending on the IMDn budget.

Figure 3. IMDn increase from PC3 to PC1.5 of 26dBm + 26dBm power combination

To see the impact of IMDn increase on delta MSD between PC1.5 and PC3, four choices were selected as below.

- PC2 PA IMDn – other IMDn = 20dB

- PC2 PA IMDn – other IMDn = 10dB

- PC2 PA IMDn – other IMDn = 0dB

- PC2 PA IMDn – other IMDn = -10dB

The delta MSD results for different IMDn budget are shown in Figure 4 for each IMD order. It can be shown that the delta MSD is reduced as PC2 PA IMDn is dominant and delta MSD is maximum when other IMDn is dominant, but it is less likely. Therefore, we propose to use somewhere in the grey region in Figure 3, i.e. 0dB < PC2 PA IMDn – other IMDn < 10dB and encourages interested companies to bring further inputs next meeting.

Proposal 5: RAN4 to consider using somewhere in the 0dB < PC2 PA IMDn – other IMDn < 10dB of IMDn increase for PC1.5 2Tx MSD calculation.

- IMD2                                   (b) IMD3

(c) IMD4                                   (d) IMD5

Figure 4. Delta MSD for PC1.5 2Tx with different IMDn budget

Similarly to PC2 2Tx and 3Tx MSD, PC1.5 2Tx MSD calculation (case 7) is applicable to PC1.5 3Tx MSD calculation (case 8, 10).

Proposal 6: RAN4 to re-use same interferer increase to derive the PC1.5 3Tx MSD (26dBm + 2*23dBm) due to IMDn as PC1.5 2Tx (26dBm + 26dBm). In other words, PC1.5 2Tx MSD (26dBm + 26dBm) calculation is applicable to PC1.5 3Tx MSD (26dBm + 2*23dBm) calculation.

PC1.5 3Tx MSD calculation of 23dBm + 2*24.8dBm

The remaining power combination is PC1.5 3Tx with 23dBm+2*24.8dBm which was specified in Rel-18 with some band combinations (case 9). Unlike the other cases where the power of two bands is same, the power of two bands is different. Therefore, IMDn increase should be examined for every IMD order coefficients (m, n) at PC3 PA at Band A as in Table 2 since PC3 PA at Band A experiences the greatest IMDn increase. As a result, we can see that the IMDn increase is not constant for a given IMD order but depends on the specified IMD order coefficients (m, n).

Table 2 IMDn increase at PC3 PA for 23dBm + 2*24.8dBm

| IMD order | Band A | Band B | Delta at PC3 PA_(Band A) vs PC3 2Tx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | m | n | Pin_A | Pin_B | IPn | IMDn |
| 2 | 1 | 1 | 3 | 7.8 | 0 | 10.8 |
| 3 | 1 | 2 | 3 | 7.8 | 0 | 18.6 |
|  | 2 | 1 | 3 | 7.8 | 0 | 13.8 |
| 4 | 1 | 3 | 3 | 7.8 | 0 | 26.4 |
|  | 2 | 2 | 3 | 7.8 | 0 | 21.6 |
|  | 3 | 1 | 3 | 7.8 | 0 | 16.8 |
| 5 | 1 | 4 | 3 | 7.8 | 0 | 34.2 |
|  | 2 | 3 | 3 | 7.8 | 0 | 29.4 |
|  | 3 | 2 | 3 | 7.8 | 0 | 24.6 |
|  | 4 | 1 | 3 | 7.8 | 0 | 19.8 |

Figure 5 shows the calculated delta MSD for PC1.5 3Tx with 23dBm + 2*24.8dBm and existing PC1.5 delta MSD from specification in Rel-18. Note that there are two, three and four delta MSD calculations for IMD3, IMD4 and IMD5 respectively depending on IMD order coefficients (m,n). This is because the different IMDn increases were used for each coefficient as in Table 2. Note that the existing spec is also grouped based on IMD order coefficient.

The calculated delta MSD results are better aligned with the existing requirements compared to the calculation in [7] which used only one IMDn increase for each IMD order.

There is one exception point in IMD5 which should be re-visited later.

(a) IMD2                                   (b) IMD3

(c) IMD4                                   (d) IMD5

Figure 5. Delta MSD for PC1.5 3Tx (23dBm + 2*24.8dBm) with existing specifications

Proposal 7: RAN4 to use interferer increase in Table 2 to derive the PC1.5 3Tx MSD (23dBm + 2*24.8dBm) due to IMDn.

Conclusion

In this contribution, we analyzed and provided our view on the HPUE MSD derivation rules based on the existing PC3 MSD. With this analysis result, we make the following proposals.

- One UL HPUE MSD calculation

Proposal 1: RAN4 to specify two sets of requirements of PC2 (1Tx PC2 and 2Tx PC2).

Proposal 2: RAN4 to use 9dB interferer imbalance and 0.5dB RevIM (Rtx) to derive the HPUE MSD from the existing PC3 MSD.

- Two UL HPUE MSD calculation

  - PC2 2Tx/3Tx MSD

Proposal 3: RAN4 to use 3*n dB interferer increase to derive the PC2 2Tx MSD due to IMDn.

Proposal 4: RAN4 to re-use 3*n dB interferer increase to derive the PC2 3Tx MSD due to IMDn. In other words, PC2 2Tx MSD calculation is applicable to PC2 3Tx MSD calculation.

  - PC1.5 2Tx/3Tx MSD

Proposal 5: RAN4 to consider using somewhere in the 0dB < PC2 PA IMDn – other IMDn < 10dB of IMDn increase for PC1.5 2Tx MSD calculation.

Proposal 6: RAN4 to re-use same interferer increase to derive the PC1.5 3Tx MSD (26dBm + 2*23dBm) due to IMDn as PC1.5 2Tx (26dBm + 26dBm). In other words, PC1.5 2Tx MSD (26dBm + 26dBm) calculation is applicable to PC1.5 3Tx MSD (26dBm + 2*23dBm) calculation.

Proposal 7: RAN4 to use interferer increase in Table 2 to derive the PC1.5 3Tx MSD (23dBm + 2*24.8dBm) due to IMDn.

References

[1]	R4-24203380, “WF on MSD rule”, Skyworks, Murata, Nokia, Qualcomm, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[2] R4-2419082, “MSD for HPUE”, Qualcomm France, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[3] R4-2417784, “Further discussion on HPUE MSD rules”, Murata Manufacturing Co. Ltd, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[4] R4-2419699, “1UL HPUE NRCA MSD Simplifications for Rel-19”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[5] R4-2419712, “1UL HPUE NRCA MSD Observations”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[6] R4-2417794, “Simplification of HPUE 2UL IMD MSDs”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[7] R4-2417793, “Observations on HPUE 2UL IMD MSDs”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024